3GPP TSG RAN4 Meeting #81	R4-1609969

Reno, USA, 14 – 18 November, 2016

Title: 	TP for TR36.714-00-02: xDL/2UL CA analysis results

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This collects MSD analysis from different companies.

# 2	Start of TP

#### <Unchanged sections omitted>

#### 6.24.1.5	MSD

When uplink CA configurations CA_3A-5A is paired with downlink CA configuration CA_3A-5A-7A there is interference components from 2 uplink operation which would interfere the downlink of the Band 7.

When uplink CA configurations CA_3A-7A is paired with downlink CA configuration CA_3A-5A-7A there is interference components from 2 uplink operation which would interfere the downlink of the Band 5.

When uplink CA configurations CA_5A-7A is paired with downlink CA configuration CA_3A-5A-7A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

The merged MSD level was provided from interested companies as below Table 6.24.1.5-1.

Table 6.24.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B5+B7 | B3 | IMD2 | fB3 +fB5 | 26.35 | [33.6] | TBD | [30] |
|  | B5 |  |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB5 | N/A | [17.8] | TBD | [18] |
|  | B5 |  |  |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 -fB7 | 15.63 | [21.7] | TBD | [19] |
|  | B7 |  |  |  |  |  |  |

#### <Unchanged sections omitted>

#### 6.26.1.5	MSD

When uplink CA configurations CA_1A-3A is paired with downlink CA configuration CA_1A-3A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_1A-42A is paired with downlink CA configuration CA_1A-3A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 3.

When uplink CA configurations CA_3A-42A is paired with downlink CA configuration CA_1A-3A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 1.

The merged MSD level was provided from interested companies as below Table 6.26.1.5-1.

Table 6.26.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B42 | B1 | IMD4 | 3*fB3 -fB1 | 8.47 | [14.1] | TBD | [11] |
|  | B3 |  |  |  |  |  |  |

#### <Unchanged sections omitted>

#### 6.29.1.5	MSD

When uplink CA configurations CA_3A-19A is paired with downlink CA configuration CA_3A-19A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_3A-21A is paired with downlink CA configuration CA_3A-19A-21A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 19.

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_3A-19A-21A there is interference components from 2 uplink operation which would interfere the downlink of the Band 3.

The merged MSD level was provided from interested companies as below Table 6.29.1.5-1.

Table 6.29.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B19+B21 | B19 | IMD5 | 4*fB19 – fB21 | 3.75 | [4.4] | TBD | [4] |
|  | B21 |  |  |  |  |  |  |

#### <Unchanged sections omitted>

#### 6.32.1.5	MSD

When uplink CA configurations CA_19A-21A is paired with downlink CA configuration CA_19A-21A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 42.

When uplink CA configurations CA_19A-42A is paired with downlink CA configuration CA_19A-21A-42A there is no interference components from 2 uplink operation which would interfere the downlink of the Band 21.

When uplink CA configurations CA_21A-42A is paired with downlink CA configuration CA_19A-21A-42A there is interference components from 2 uplink operation which would interfere the downlink of the Band 19.

The merged MSD level was provided from interested companies as below Table 6.32.1.5-1.

Table 6.32.1.5-1: MSD values provided by companies

| DL CA | UL CA | IMD |  | LGE | Huawei | Qualcomm | Avg. MSD [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B19+B21+B42 | B19 | IMD4 | 3*fB21 - fB19 | 8.71 | [14.2] | TBD | [11] |
|  | B21 |  |  |  |  |  |  |
|  | B21 | IMD4 | 3*fB21 - fB42 | 12.85 | [12.4] | TBD | [13] |
|  | B42 |  |  |  |  |  |  |