TSG-RAN Working Group 4 (Radio) meeting #81	 R4-1610238

Reno, USA, 14 - 18 Nov, 2016

Source:	Airbus DS SLC

Title:	Band 68 compliance with ECC Decision (16)02: Simulation assumptions for normal environmental conditions

Agenda item:	8.32.2

Document for:	Approval

# Introduction

At RAN4 meeting #80bis, simulation assumptions to be used to derive A-MPR requirement for band 68 to meet European requirements specified in Decision (16)02 [1] were discussed [2].

It was noted that for the lower part of the band 68 (698-703MHz), two requirements for DTV protection are applicable:

- -42dBm/8MHz in normal environmental conditions

- -30dBm/8MHz in extreme environmental conditions

It was then proposed to agree on simulation assumptions to derive A-MPR requirement for both types of environmental conditions, to be able to assess which of the two above requirements is the most stringent and shall be used to defined requirements in TS36.101.

# Discussion

## Simulation assumptions for band 68 work item

In the framework of the band 68 work item, simulations to derive A-MPR requirements were performed and results can be found in TR36.893 [3].

For such simulations, the performances of the following blocks shall be taken into account and thus need to be modelled:

- The transmit chain (I/Q modulator and power amplifier)

- The uplink filter

### Performances of the transmit chain

The performances of the transmit chain are well aligned in contributions presenting simulations results (e.g. [4], [5], [6]):

- Modulator IQ – image = 28dB

- Modulator carrier leakage = 28dBc

- Modulator C_IM3 = 60dBc

- UTRA ACLR 1= 33dB

Although several performance requirements can be used to assess the power amplifier linearity, all companies have identified that UTRA ACLR is the most stringent one.

Thus, the PA model is chosen so that the signal at the output of the PA – taking into account the I/Q impairments of the transmitter – has an UTRA ACLR value of 33dB.

The 33dB value was chosen to model the worst case conditions since this value is equal to the minimum requirement defined in TS36.101.

### Performances of the uplink filter

The performances of the uplink filter are defined by the rejection values that are applicable in the band 686-694MHz. As for the transmitter impairments, these values were selected to represent the worst case scenario.

Agreed values can be found in [3] and are copied below:

| Worst case relative filter rejection |  |
| --- | --- |
| Frequency (MHz) | Rejection (dB) |
| 693.5 | 5.7 |
| 692.5 | 10.9 |
| 691.5 | 18.4 |
| 690.5 | 26.2 |
| 689.5 | 30.6 |
| 688.5 | 31.7 |
| 687.5 | 32.2 |
| 686.5 | 32.2 |

## Extreme environmental conditions

Since worst case conditions were assumed when defining the simulation assumptions for band 68,

Proposal 1: it is proposed to re-use the parameters agreed in the scope of the work item   LTE_FDD_700_ARAB  (Band 68) that are described in TR36.893 [3] to derive the A-MPR values corresponding to the requirement of the ECC Decision (16)02 to be met in extreme environmental conditions (-30dBm/8MHz)

## Normal environmental conditions

As mentioned above, the agreed values for simulations that were used for the LTE_FDD_700_ARAB  work item were selected to be representative of the worst case scenario.

“Worst case scenario” means the combination of two worst case conditions:

- Part-to-part dispersion: in production, for a given test environment, the performances of the components are spread over a certain distribution. “Worst case” assumes the lowest performances are taken into account.

- Variations due to environmental conditions: the performances of the components changes with the environmental conditions (e.g. the temperature). “Worst case” assumes that the most stringent conditions – those having the larger negative impact on the performances – have been taken into account. This is referred as extreme environmental conditions.

To derive the parameters applicable to normal environmental conditions,

- the part-to-part dispersion shall still be taken into account.

- the degradation due to the environmental conditions shall be revised

### Power amplifier datasheets

The values of the table below are extracted for datasheet of power amplifiers of two manufacturers.

| Manufacturer |  | #1 |  | #2 |  |
| --- | --- | --- | --- | --- | --- |
|  | Spec. | Typ. | Max. | Typ. | Max. |
| E-UTRA ACLR | 30 | 40 | 35 | 37 | 33 |
| UTRA ACLR 1 | 33 | 41 | 37 | 39 | 36 |

Table 1: PA typical and guaranteed specifications

For both manufacturers, the values are defined for the test conditions: temperature 25°C, LTE 10MHz, 12 active RBs, no MPR, Fc=787MHz.

The above table gives an indication on how the performance margin is split:

- The manufacturer #1 specifies 5~6dB for part-to-part dispersion (= typ. – max) and 4~5dB for environmental degradations (= max. – spec.)

- The manufacturer #2 specifies 3~4dB for part-to-part dispersion (= typ. – max) and 3dB for environmental degradations (= max. – spec.)

### LTE UE measurements

Five UEs have been measured in normal temperature conditions (+25°C) in order to check the ACLR performances. Measurements results can be found in annex. The lowest ACLR UTRA 1 values are summarized in the Table 2 below for each unit.

| UE | #1 | #2 | #3 | #4 | #5 |
| --- | --- | --- | --- | --- | --- |
| Lowest ACLR UTRA 1 | 43.2dB | 42.7dB | 40.8dB | 41.6dB | 43.9dB |

Table 2: ACLR UTRA 1 measurements @ 25°C

### Simulation assumptions for Tx chain performances in normal conditions

From Table 1, it can be observed that the manufacturers guarantee at least 3 to 4dB margin over the specification @ 25°C for the UTRA ACLR1.

From Table 2, it can be observed that all measured UEs have a significant margin compared to the guaranteed values of the data sheets.

Given the above elements,

Proposal 2: it is proposed to use the following signal characteristics for normal conditions:

- Modulator IQ – image = 28dB

- Modulator carrier leakage = 28dBc

- Modulator C_IM3 = 60dBc

- UTRA ACLR 1= 36.5dB

### Filter performances

As mentioned above the agreed values in [3] correspond to worst case scenario.

Similarly to what has been done for the PA performances, the filter performances shall be assessed for normal environmental conditions.

The performance degradations of the filter are mainly due to the frequency drift of the filter transfer function.

Figure 1: Filter frequency response

Since the DTV protection requirement is below the operating band, the worst case would be for high temperature since the response in frequency is “shifted” towards lower frequency as described on Figure 1.

694MHz

698MHz

Band 68

DTV

f

90°C

20°C

The drift f depends on the filter technology. For TC-SAW filters – as agreed in [3] – the typical frequency variation is ~22ppm/°C.

Hence, the frequency drift between normal (25°C) and extreme conditions (90°C) can be assessed at ~1MHz (=22.10-6 * 700 * (90-25)).

Proposal 3: For normal conditions, it is proposed to apply a +1MHz shift to the frequency response of the agreed filter defined in [3].

| Frequency (MHz) | Average TC-SAW filter in [3] | Proposal for normal environmental conditions |
| --- | --- | --- |
| 693.5 | 5.7 | 10.9 |
| 692.5 | 10.9 | 18.4 |
| 691.5 | 18.4 | 26.2 |
| 690.5 | 26.2 | 30.6 |
| 689.5 | 30.6 | 31.7 |
| 688.5 | 31.7 | 32.2 |
| 687.5 | 32.2 | 32.2 |
| 686.5 | 32.2 | 32.2 |

## 3	Conclusion

In this contribution, performances in extreme and normal environmental conditions have been discussed. The goal is to agree on simulation assumptions to derive the A-MPR requirements for band 68 to meet the European requirements defined in ECC Decision (16)02 [1].

The following is proposed:

Proposal 1: it is proposed to re-use the parameters agreed in the scope of the work item   LTE_FDD_700_ARAB  (Band 68) that are described in TR36.893 [3] to derive the A-MPR values corresponding to the requirement of the ECC Decision (16)02 to be met in extreme environmental conditions (-30dBm/8MHz)

Proposal 2: it is proposed to use the following signal characteristics for normal conditions:

- Modulator IQ – image = 28dB

- Modulator carrier leakage = 28dBc

- Modulator C_IM3 = 60dBc

- UTRA ACLR 1= 36.5dB

Proposal 3: For normal conditions, it is proposed to apply a +1MHz shift to the frequency response of the agreed filter defined in [3].

The proposed simulation assumptions are summarized in the Table 3 below.

| Tx chain performances |  |  | Worst case relative filter rejection (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Extreme | Normal | Frequency (MHz) | Extreme | Normal |
| Modulator IQ – image | 28dB | 28dB | 693.5 | 5.7 | 10.9 |
| Modulator carrier leakage | 28dBc | 28dBc | 692.5 | 10.9 | 18.4 |
| Modulator C_IM3 | 60dBc | 60dBc | 691.5 | 18.4 | 26.2 |
| UTRA ACLR 1 | 33dB | 36.5dB | 690.5 | 26.2 | 30.6 |
|  |  |  | 689.5 | 30.6 | 31.7 |
|  |  |  | 688.5 | 31.7 | 32.2 |
|  |  |  | 687.5 | 32.2 | 32.2 |
|  |  |  | 686.5 | 32.2 | 32.2 |

Table 3: Assumptions for normal and extreme conditions for A-MPR simulations

## 4	References

[1]	Decision (16)02: Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems - http://www.erodocdb.dk/Docs/doc98/official/pdf/ECCDEC1602.PDF

[2]	R4-168556 , Band 68 compliance with ECC Decision 6(02): Impacts on TS36.101, Airbus DS SLC

[3]	TR36.893, Evolved Universal Terrestrial Radio Access (E-UTRA); 700MHz band for Arab Region

[4]	R4-162656 B68 Filter Consolidation with B28A, TP for 36.893: TCSAW filter data and A-MPR for 700 MHz protection of DTV, Skyworks Solutions, inc.

[5]	R4-157644 A-MPR for LTE 700M, Huawei, Hisilicon

[6]	R4-158252 Discussion on A-MPR for B68 protection of broadcast services, Motorola Solutions

## Annexes

| UE #1 12RBs low |  |
| --- | --- |
| UE #1 12RBs high |  |
| UE #1 50RBs |  |

| UE #2 12RBs low |  |
| --- | --- |
| UE #2 12RBs high |  |
| UE #2 50RBs |  |

| UE #3 12RBs low |  |
| --- | --- |
| UE #3 12RBs high |  |
| UE #3 50RBs |  |

| UE #4 12RBs low |  |
| --- | --- |
| UE #4 12RBs high |  |
| UE #4 50RBs |  |

| UE #5 12RBs low |  |
| --- | --- |
| UE #5 12RBs high |  |
| UE #5 50RBs |  |