3GPP TSG-WG RAN4 Meeting #81 			R4-1610328

Reno, US, Date 14th – 1th November

Title:	LS on Definition of eNB and UE Rx – Tx time differences

Release:	Rel-13

Work Item:	TEI13

Source:	3GPP RAN WG4

To:	3GPP RAN WG1

Contact Person:

#### Name:	Ren Da

E-mail Address:	Ren.Da@nokia.com

1. Overall Description:

TS 36.214 provides the following definitions for UE Rx – Tx time differences as follows:

5.1.15	UE Rx – Tx time difference

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink radio frame #i from the serving cell, defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink radio frame #i. The reference point for the UE Rx – Tx time difference measurement shall be the UE antenna connector. |
| --- | --- |

It is RAN4’s understanding that when UE receives downlink radio frame#i from the serving cell, the UE should be capable of determining UE Rx – Tx time difference with the timing of the uplink radio frame#i, but there is no need to perform the uplink transmission of UL radio frame #i, since the timing of the uplink radio frame#i is independent of whether there is a real uplink transmission of UL radio frame #i. The definition “TUE-TX  is the UE transmit timing of uplink radio frame #i”, however, may results in a misunderstanding that the UE is required to transmit uplink radio frame #i in order to obtain the UE Rx – Tx time difference. Thus, RAN4 suggests making the following modification on the definition of UE Rx – Tx time difference:

5.1.15	UE Rx – Tx time difference

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink radio frame #i from the serving cell, defined by the first detected path in time. TUE-TX is the timing of uplink radio frame #i of the UE. The reference point for the UE Rx – Tx time difference measurement shall be the UE antenna connector. |
| --- | --- |

For the similar reason as shown above, RAN4 also suggests making the following modification on the definition of eNB Rx – Tx time difference:

5.2.5	eNB Rx – Tx time difference

| Definition | The eNB Rx – Tx time difference is defined as T eNB-RX – TeNB-TX Where: T eNB-RX is the eNB received timing of uplink radio frame #i, defined by the first detected path in time. The reference point for TeNB-RX shall be the Rx antenna connector. T eNB-TX is the timing of downlink radio frame #i of the eNB. The reference point for TeNB-TX shall be the Tx antenna connector. |
| --- | --- |

2. Actions:

RAN4 respectfully request RAN1 to consider above suggestions.

3. Date of Next TSG-RAN4 Meetings:

TSG RAN4#82	13-17 Feb, 2017	Athens, Greece

TSG RAN4#82bis	3-7 Apr, 2017			US