3GPP TSG-RAN WG4 Meeting #81 	R4-1610837

Reno, NV, US, 14 – 18 November, 2016

Source:	SoftBank Corp.

Title:	TP for TR36.745: Evaluation results and conclusion (section 6 and 7)

Agenda item:	9.2.2

Document for:	Approval

1.  Introduction

The SI of globalization of Band 11 and Band 21 was approved in RAN#72[1]. This contribution is to reflect the content of an agreed contribution on perquisites on UE test setting and operation conditions [2] in the last meeting.  In addition, A-MPR and RB restriction schemes are evaluated based also on [2] using simulation and measurement results presented in the last two meetings [3-6].

Since the method to evaluate was the same as in [2], the following text is to summarize the results and the detailed data could be found in TP attached. This TP also includes some cosmetic things like putting symbols/abbreviations.

Note: in this updated version from R4-1609043, the summary of Skyworks’ contribution [7] is added in sec. 6.2. as indicated below (and with marker in TP). The original content of R4-1609043 is confirmed OK with Qualcomm.

“Based on the measurement of actual PA in max (22 or 23dBm) and reduced power, the document [X] reported some observations and suggestions to set relaxed criteria in operation above 15dBm output power. While the document echoes the unlimited use of the second 5/10MHz as above, it also suggests the possibility of extended RB usage in the first 5/10MHz with improvement of Tx impairments such as agreed for UL 256QAM.”

2.  Prerequisites

Agreed conditions in [2] are captured in TP of section 6.1 and 6.2. Essential agreements are repeated here:

- It is assumed that the UE should be tested at 15dBm and the possible methods, i.e. A-MPR or RB restriction, should guarantee the protection requirement at 15dBm transmission power in the first place.

- It is assumed that UE can take any power as long as the protection requirements are met. This means that if an RB allocation (i.e. a combination of RBSTART and RB length: LCRB) is permitted at 15dBm, this allocation can be used in any power up to 23dBm.

These are captured in section 6.1 of the TP attached.

3.  Evaluation with simulation and measurement results

Since the initial simulation results [3, 4] can be used, the evaluation results for the first 10MHz (UL: 1427.9 – 1437.9MHz) in [2] can be used as it is. The outcome is:

- Up to 8RB can be used without any restriction,

- eMTC and in-band NB-IoT can be used without any restriction.

In the first 5MHz (1427.9 – 1432.9MHz), the same limitations as 1) and 2) above can also be applied according to [5]. For the second 10MHz (1437.9 – 1447.9MHz) and 5MHz (1432.9MHz – 1437.9MHz), it can be concluded that no restriction is needed and it is apparent for eMTC and in-band NB-IoT to be deployed without any restriction.

A measurement date of PA is also presented [6] and the results were aligned with simulations. Thus, it is proposed to approve:

[Proposal-1] Resource restriction, such as RB restriction or A-MPR, is only needed for the first (= lowest) 5 and 10 MHz in Band 11.

[Proposal-2] In the lowest 5 and 10MHz of Band 11, up to 8RB can be used without any restriction.

[Proposal-3] eMTC and in-band NB-IoT can be used without any restriction.

The observations were described in section 6.2 of the TP.

4.  Actual restriction schemes

In case of A-MPR, the result of the first 10MHz in Band 11 was presented in [2] and proposed here as it is. As in [Proposal-1] of this paper, the first 5MHz case should be evaluated and a possible A-MPR is captured in the table below:

Introducing A-MPR will, however, cause a backward compatibility issue with “legacy” Band 11 UEs which do not understand newly defined A-MPR. TP includes description to draw an attention to this aspect.

Concerning RB restriction, the same discussion as in [2] is done for the first 5MHz and results are same as in [Proposal-2]. In addition, possibility to support other RB allocation as suggested in [2, 6] is also discussed with a caution that it is up to implementation of eNodeB scheduler.

[Proposal-4] Actual restriction schemes in the section 6.3 of TP are agreed as a result of this SI.

5.  Conclusion

This contribution proposes TP to capture essential portion of this study, prerequisites agreed in the last meeting, simulation/measurement results and findings from these, and actual methods such as A-MPR and RB restriction.

The conclusion section (section 7) is also added to summarize the findings of the SI.

TP is attached as Annex.

6.  Reference

[1]      RP-161220   New Study Item proposal: Feasibility study on global application of LTE Band 11 and of LTE Band 21 UEs, SoftBank Corp.

[2]      R4-167338	Consideration on RB restriction/A-MPR schemes, SoftBank Corp.

[3]      R4-166173	Initial simulation results for the feasibility study on global application of LTE Band 11 and of LTE Band 21 UEs, Nokia

[4]      R4-166324	Discussion on Band 11 UE emission to protect EESS, Huawei, Hisilicon

[5]      R4-168453	EESS protection study, Nokia, Alcatel-Lucent Shanghai Bell

[6]      R4-167412	B11 measurement results for EESS protection, Huawei, Hisilicon

[7]      R4-169494   Band 11 PA measurements on EESS protection, Skyworks,

Annex: TP for TR36.745 (v0.2.0)

[Unchanged portions skipped]

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	 “Final Acts WRC-15”, WORLD RADIOCOMMUNICATION CONFERENCE (GENEVA, 2015) .

[3]	R4-165235: “Operating scenarios to enhance the value of the 1.5GHz band”.

[4]	R4-166173: “Initial simulation results for the feasibility study on global application of LTE Band 11 and of LTE Band 21 UEs”.

[5]	R4-166324: “Discussion on Band 11 UE emission to protect EESS”.

[6]	R4-168453: “EESS protection study”.

[7]	R4-167412: “B11 measurement results for EESS protection”.

[8]	R4-168411: “For maximum use of the 1.5GHz band”.

[9]	R4-168506: “How to protect EESS by 1.5GHz UE”

[10]	R4-169494: ” Band 11 PA measurements on EESS protection”

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks.

RBstart 	Indicates the lowest RB index of transmitted resource blocks.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A-MPR		Additional Maximum Power Reduction

EESS		Earth Exploration-Satellite Service

RB		Resource Block

[Unchanged portions skipped]

6.1	Prerequisites for the evaluation

6.1.1 	Conditions for UE testing for compliance

[2] can be understood that a UE is tested at Tx = 15dBm to comply with the protection requirement mentioned above. Thus, it is assumed that the UE should be tested at 15dBm and the possible methods, e.g. A-MPR or RB restriction, should guarantee the protection requirement at 15dBm transmission power in the first place.

6.1.2	UE operation other than Tx power = 15dBm

[2] does not mention about requirements/conditions for a UE to transmit other than 15dBm. Since co-existence studies have already been done in ITU-R taking into account collective effect of many UEs toward a victim satellite, it is understood that variation of UE Tx power, up to 23dBm or lower than 15dBm, is also taken into account. In this context, it is assumed that UE can take any power as long as the protection requirements are met. This means that if an RB allocation (i.e. a combination of RBSTART and RB length: LCRB) is permitted at 15dBm, this allocation can be used in any power up to 23dBm.

There are two possible schemes to limit RB allocation to satisfy the requirement: using A-MPR and RB restriction. The following sections describe the details of these methods including pros and cons.

6.2	Simulation results

Two simulation results are presented for the first 10MHz case, i.e. (1) in section 5.1 [4, 5]. Note that simulations in this section were done in 23dBm in maximum power and evaluate necessary back off relative to 23dBm with MPR when needed.

Figure 6.2-1: Simulation result for the first 10MHz case [4]

Figure 6.2-2: Simulation result for the first 10MHz case [5]

Firstly, region where backoff is less than or equal to than 8dB (7dB in case of 1dB MPR to be applied, LCRB>12 in 10MHz) is defined as “permitted region”. On the other hand, region where further backoff is needed at 15dBm is called as “prohibited region”. As mentioned above, in the permitted region, a UE can use the RB allocation at any Tx power in actual operation.

Two results show similar tendencies in terms of >8dB (or 7dB) backoff region, i.e.

- up to 8RBs, no restriction is required,

- the border of the regions is almost linearly extended from (RBSTART, LCRB) = (0, 9) toward (20, 30), via (4,12),

This is depicted in Figure 6.2-3 where the white-colored region is “permitted” in any power while yellow-colored region means “prohibited” due to 15dBm results.

Figure 6.2-3: Prohibited and permitted regions for the first 10MHz case

From these observations, it can be concluded that in this 10MHz operation:

- up to 8RBs can be used without restriction.

This fact leads to the following outcomes:

- no PUCCH overprovisioning is required and,

- eMTC (6RB) or in-band NB-IoT (1RB) could also be accommodated without restriction.

Simulations for scenarios (2) – (4) were presented in [6]. Concerning the first 5MHz, the simulation result is shown in Figure X which is similar to first 10MHz, i.e., up to 8RB can be used without any restriction and this leads to the same outcomes in 2) and 3) above. (Note that in 5MHz and QPSK, 1 dB MPR is to be applied for LCRB>8).

Figure 6.2-4: Simulation result for the first 5MHz case

Concerning second 10MHz and 5MHz, as shown in Figure 6.2-5 and 6.2-6 below, there is no need to apply deep backoff such as 7 - 8dB. Thus, it can be concluded that these frequency ranges can be used without any restriction.

Figure 6.2-5: Simulation result for the second 10MHz case

Figure 6.2-6: Simulation result for the second 5MHz case

In addition, measurement results of PA were presented in [7]:

Table 6.2-1: Measurement result for Band 11 scenarios [7]

| The first 5 MHz of B11 (case 3 in 5.1) |  | The first 10 MHz of B11 (case 1 in 5.1) |  | The second 5MHz and the second 10MHz in B11 (case 2 and case 4 in 5.1) |
| --- | --- | --- | --- | --- |
| RBstart | LCRB | RBstart | LCRB | No restriction for RB allocation |
| 0 | 8 | 0 | 8 |  |
| 12 or 13 | 12 | 25 | 25 |  |

This measurement result is agreed well with simulation results described above.

Based on the measurement of actual PA in max (22 or 23dBm) and reduced power, the document [10] reported some observations and suggestions to set relaxed criteria in operation above 15dBm output power. While the document echoes the unlimited use of the second 5/10MHz as above, it also suggests the possibility of extended RB usage in the first 5/10MHz with improvement of Tx impairments such as agreed for UL 256QAM.

6.3	Conditions required for EESS protection

Since the first 10MHz and the first 5MHz cases are only subject to resource restriction, this section is focused on the two cases.

6.3.1	A-MPR scheme

A-MPR is an ordinary method in LTE for the compliance to a protection requirement. This requests a UE to implement the relation between RB allocation (RBstart and LCRB) and required additional (i.e. post MPR) backoff in a certain band. When an eNodeB broadcasts a relevant signal (NS_XX), a UE shall limit its Tx power if necessary according to RB allocation. Normally, A-MPR is defined when a new band is introduced: since it is requested to implement in UEs and this is mainly used for regulatory compliance, it is highly desirable for all the UEs for the band to support the A-MPR. However, it is permitted to introduce A-MPR in the middle but for the case, it is necessary to understand the drawback: this does not work for “legacy” UEs which were released before the A-MPR is defined.

In the course of discussion, a new A-MPR schemes was discussed in [8, 9] but with the clarification of prerequisites noted in section 6.1, it was concluded to use conventional A-MPR scheme.

For the first 10MHz case, as discussed above, permitted region is free from A-MPR and the prohibited region is for heavy backoff to satisfy the protection requirement at 15dBm. An example of possible A-MPR is derived from Figure 6.3.1-1 and shown in Table 6.3.1-1 below.

Figure 6.3.1-1: A-MPR for the first 10MHz case

Table 6.3.1-1: A-MPR for the first 10MHz case

| Parameters | Region A |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 4 |  | 5 – 11 |  | 12 – 20 |  |
| LCRB [RBs] | 9-15 | ≥16 | 12-16 | ≥ 18 | 18-24 | ≥ 25 |
| A-MPR [dB] | ≤ 13 | ≤ 15 | ≤ 9 | ≤ 13 | ≤ 9 | ≤ 13 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR=0. |  |  |  |  |  |  |

As per the convention of A-MPR/NS table, RB allocations not indicated in the table can be used without A-MPR. Since clarification in this point is really needed for this SI, a note is added to the table.

For the first 5MHz, an example of A-MPR table based on Figure 6.2-4 is:

Table 6.3.1-2: A-MPR for the first 5MHz case

| Parameters | Region A |  | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 6 |
| LCRB [RBs] | 9-16 | ≥18 | 12 - 16 | ≥18 | ≥18 |
| A-MPR [dB] | ≤ 10 | ≤ 11 | ≤ 9 | ≤ 10 | ≤ 9 |
| Note: RB allocations other than indicated in this table, UE is permitted to transmit power with A-MPR=0. |  |  |  |  |  |  |  |

6.3.2	RB restriction scheme

RB restriction can be understood, in terms of RB allocation, as a special case of A-MPR scheme where A-MPR is equal to 0. Thus, RB allocation is limited to where additional power backoff is not required in A-MPR scheme and the actual RB allocation conditions shall be implemented in eNodeB scheduler, not in UE. Because of this difference, a major advantage over A-MPR scheme is that legacy UEs can also be operated under an eNodeB supporting the RB restriction but flexibility of RB allocation should be compromised.

Because no power reduction is applied, the operation in RB restriction is limited to the “permitted” region in Figure 6.2-3 and in principle this region can be used without any restriction as long as a scheduler in an eNodeB can support. However, the degree of freedom in RB allocation is a trade off with the complexity of implementation or testability.

In 10MHz case, up to LCRB = 8 can be used for any RBStart first of all. This is indicated in light green in Figure 6.3.2-1. In addition, practically speaking, considering 5-6 RBs on both ends are occupied by PUCCH, it is likely to introduce a combination like (RBstart≧ 8, LCRB≦16) or (RBstart≧12, LCRB≦20) in addition to LCRB≦8 for any RBstart. Note that Table 6.2-1 suggests (RBstart≧25, LCRB≦25).  An example of such extension is shown in Figure 6.3.2-1 with dark green.

Figure 6.3.2-1: RB restriction for the first 10MHz case

In case of the first 5MHz, in addition to up to 8RB without constraints, (RBstart≧4, LCRB≦12) might be likely as extended allocation taking 2 or 3 PUCCH region into consideration.

7	Conclusion

In this TR, possible schemes of Band 11 and Band 21 UE for EESS protection were discussed. It was found that:

1)	Only the lowest 5 or 10MHz in Band 11 is subject to limitation of RB allocation and the rest of Band 11 and Band 21 can be used without any restriction,

2)	In the lowest 5 or 10MHz of Band 11, up to 8RB can be used without limitation,

3)	Because of 2), Cat. M1(eMTC) or Cat NB1(NB-IoT) in in-band can also be used without restriction, and

4)	Possible A-MPR and RB restriction schemes for the lowest 5 or 10MHz are also studied.

On the other hand, two methods, A-MPR or RB restriction, have pros and cons of their own and the choice will strongly depend on the availability of “legacy” Band 11 and Band 21 UEs at the time of adopting the bands under EESS protection. Therefore it is recommended for potential adopters to consider this aspect carefully.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"