3GPP TSG-RAN WG4 RAN4 #80bis                                     R4-1610845

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda item:			9.3

Source:			Samsung, China Telecom

Title:	TP for TR 36.752: Description of simulation methodology

Document for:			Discussion

# 1	Introduction

The text proposal provides the clarification on the description of simulation methodolgoy.

# Text Proposal

<<< Start of text proposal for TR >>>

## 6.3	Coexistence simulation methodology

The following methodology for coexistence analysis will be used:

Step-1: RAN4 get agreement on RF performance assumptions for legacy CDMA system.

- The RF performance assumptions for legacy CDMA system should be determined firstly, e.g., ACLR/ACS performance for both CDMA UE and BS.

Step-2: Based upon operators’ input on the NB-IoT deployment scenarios, the carrier separation between NB-IoT and CDMA systems are determined.

- The carrier separation should be determined based on practical operators’ deployment needs.

Step-3: Conduct coexistence simulation based on agreed simulation assumptions, under certain agreed RF performance assumptions for legacy CDMA system and certain carrier separation options between NB-IoT and CDMA systems.

- After obtaining coexistence simulation results, RAN4 should compare the simulation results with Rel-13 NB-IoT RF requirement to determine it could be reused or not.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements could be reused to guarantee the coexistence between NB-IoT and legacy system, go to step 4.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements is not enough to guarantee the coexistence between NB-IoT and legacy system, go to step 5.

Step-4: Conclusion can be reached that Rel-13 NB-IoT RF requirements could be reused to make sure a proper coexistence with CDMA system.

Step-5: Identify the necessity of introducing new NB-IoT RF requirement to make sure a proper coexistence with CDMA system.

Similar to the methodology utilized in Rel-13 coexistence study for NB-IoT [4], the evaluation metrics used in this coexistence study is

- For NB-IoT, SINR loss should be less or equal to 1dB

- For CDMA , Capacity loss should be less or equal to 5%

Similar to the methodology utilized in Rel-13 coexistence study for NB-IoT [4], the following calculation for ACLR/ACS modeling is applied in the coexistence study:

•	Definition and assumption

•	CDMA ACLR is defined for NB-IoT system bandwidth of 180 kHz.

•	CDMA ACS is defined for its own bandwidth (while the adjacent channel measurement bandwidth (as aggressor) is 180kHz), and the value is derived based on single tone desensitization test.

•	NB-IoT ACLR and ACS are defined for its own bandwidth size

•	Attenuations are flat in the adjacent channels

•	ACLR and ACS are the inputs to the simulations

•	ACLR_e is the effective ACLR in the interference calculation

•	ACS_e is the effective ACS in the interference calculation

•	Interference from CDMA2000 to NB-IoT

•	ACLR_e = ACLRCDMA – 10*log10(1/(# of IoT users))

•	ACS_e = ACSNB-IoT [pessimistic assumption]

•	Interference from NB-IoT to CDMA2000

•	ACLR_e = ACLRNB-IoT – 10*log10(1.23MHz/180kHz) [pessimistic assumption]

•	ACS_e = ACSCDMA

<<< End of text proposal for TR >>>