3GPP TSG RAN4 Meeting #82	R4-1701497

Gothenburg, Sweden, 22 – 26 August, 2016

Title: 	MSD for 2UL/xDL CA

Source: 	Huawei, HiSilicon

Agenda item:	7.7.2

Document for:	Other (Approval)

# 1	Introduction

This contribution discusses MSD requirements for 1+3+28, 1+28+42, and 3+21+28 3DL 2 UL CA.

# 2	Discussion

1+3+28:

This CA combination has one IMD4 and IMD5 product that fall on top of own B1 RX and B3 RX channel, respectively.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B28 | B1 | IMD5 | ¦3*fB28 -2*fB1¦ | 1975 | 5 | 25 | 1818.5 | 5 | TBD | TBD |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | ¦2*fB28 -2*fB3¦ | 1780 | 5 | 25 | 2139 | 5 | TBD | TBD |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |

Table 1 Test points

We use a diplexer based reference architecture for the analysis. B1+B3 are combined using a quadplexer in high branch.

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- |
| Antenna switch | 56 | 53 |
| Diplexer | 55 | 53 |
| Duplexer | 55 | 53 |
| LNA | -6 | -10 |

Table 2 General linearity parameters

B1:

B28 duplexer TX attenuation at B1 RX							               32

B3 quadplexer TX attenuation at B1 RX							         35

B28 PA forward mixing IMD4 at PA output with 60dB isolation	       -47

B1 FE loss (switch+trace+filter)										          5

B3:

B28 duplexer TX attenuation at B3 RX							               30

B1 quadplexer TX attenuation at B3 RX							         46

B28 PA forward mixing IMD4 at PA output with 60dB isolation	        -46

B3 FE loss (switch+trace+filter)										          5

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation	 											 		        15

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD.

B1 MSD:

B3 MSD:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B28 | B1 | IMD5 | ¦3*fB28 -2*fB1¦ | 1975 | 5 | 25 | 1818.5 | 5 | 2.5 | 5.34 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | ¦2*fB28 -2*fB3¦ | 1780 | 5 | 25 | 2139 | 5 | 2 | 9.60 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |

1+28+42:

This CA combination has two IMD3 products falling on top of own B1 and B42 RX channel

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B28+B42 | B1 | IMD3 | ¦2*fB28 +*fB1¦ | 1955 | 5 | 25 | 3425 | 5 | TBD | TBD |
|  | B28 |  |  | 735 | 5 | 25 |  |  |  |  |
|  | B28 | IMD3 | ¦2*fB28 -fB42¦ | 710.5 | 5 | 25 | 2139 | 5 | TBD | TBD |
|  | B42 |  |  | 3560 | 5 | 25 |  |  |  |  |

Table 1 Test points

We use a triplexer based reference architecture for the analysis

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP3 (dBm) |
| --- | --- |
| Antenna switch | 68 |
| Triplexer | 86 |
| Duplexer | 74 |
| LNA | -6 |

Table 2 General linearity parameters

B42 MSD:

B28 duplexer TX attenuation at B42 RX							        30

B1 duplexer TX attenuation at B42 RX									  26

B28 PA forward mixing IMD3 at PA output with 60dB isolation	       -25

B28 PA RFIC P2P fw mixing IMD3 at PA output						 -30

B42 FE loss (switch+trace+filter)										    5.5

B1 MSD:

B28 duplexer TX attenuation at B1 RX							              32

B42 duplexer TX attenuation at B1 RX									  45

B28 PA forward mixing IMD3 at PA output with 60dB isolation	       -35.5

B28 PA RFIC P2P fw mixing IMD3 at PA output						 -41

B1 FE loss (switch+trace+filter)										          5

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation	 LB-HB												  30

Diplexer isolation MB-HB												  18

Diplexer isolation LB-MB											        20

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD. Please note that the CF is 1dB.

B42 MSD:

B1 MSD:

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B28+B42 | B1 | IMD3 | ¦2*fB28 +*fB1¦ | 1955 | 5 | 25 | 3425 | 5 | 1.6 | 14.91 |
|  | B28 |  |  | 735 | 5 | 25 |  |  |  |  |
|  | B28 | IMD3 | ¦2*fB28 -fB42¦ | 710.5 | 5 | 25 | 2139 | 5 | 1.6 | 10.86 |
|  | B42 |  |  | 3560 | 5 | 25 |  |  |  |  |

3+21+28:

This CA combination has only IMD5 product that falls on top of own B28 RX channel

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B21+B28 | B3 | IMD5 | ¦3*fB21 -2*fB3¦ | 1782 | 5 | 25 | 789 | 5 | TBD | TBD |
|  | B21 |  |  | 1451 | 5 | 25 |  |  |  |  |

Table 1 Test points

We use a triplexer based reference architecture for the analysis

We analyze the amount of IMD using the following component linearity assumptions:

| Component | IP5 (dBm) |
| --- | --- |
| Antenna switch | 53 |
| Triplexer | 53 |
| Duplexer | 53 |
| LNA | -10 |

Table 2 General linearity parameters

B21 duplexer TX attenuation at B28 RX							        44

B3 duplexer TX attenuation at B28 RX									  37

B21 PA forward mixing IMD5 at PA output with 60dB isolation	       -45

B21 PA RFIC P2P fw mixing IMD5 at PA output						 -52

B28 FE loss (switch+trace+filter)										    5.5

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation	 LB-HB												  30

Diplexer isolation MB-HB												  18

Diplexer isolation LB-MB											        20

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

Based on these parameters we get the following MSD.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B21+B28 | B3 | IMD5 | ¦3*fB21 -2*fB3¦ | 1782 | 5 | 25 | 789 | 5 | 2.5 | 0 |
|  | B21 |  |  | 1451 | 5 | 25 |  |  |  |  |

# 3	Conclusion

MSD for B1+B3+B28 3DL 2UL CA MSD is provided.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B28 | B1 | IMD5 | ¦3*fB28 -2*fB1¦ | 1975 | 5 | 25 | 1818.5 | 5 | 2.5 | 5.34 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |
|  | B3 | IMD4 | ¦2*fB28 -2*fB3¦ | 1780 | 5 | 25 | 2139 | 5 | 2 | 9.60 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |

MSD for B1+B28+B42 3DL 2UL CA MSD is provided.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B28+B42 | B1 | IMD3 | ¦2*fB28 +*fB1¦ | 1955 | 5 | 25 | 3425 | 5 | 1.6 | 14.91 |
|  | B28 |  |  | 735 | 5 | 25 |  |  |  |  |
|  | B28 | IMD3 | ¦2*fB28 -fB42¦ | 710.5 | 5 | 25 | 2139 | 5 | 1.6 | 10.86 |
|  | B42 |  |  | 3560 | 5 | 25 |  |  |  |  |

MSD for B3+B21+B28 3DL 2UL CA MSD is provided.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B21+B28 | B3 | IMD5 | ¦3*fB21 -2*fB3¦ | 1782 | 5 | 25 | 789 | 5 | 2.5 | 0 |
|  | B21 |  |  | 1451 | 5 | 25 |  |  |  |  |

# 4	References