3GPP TSG-RAN WG4 Meeting #82    									           R4-1702372

Athens, Greece, 13-17 February 2017

Source: 		Nokia, Alcatel-Lucent Shanghai Bell, ZTE, NTT DOCOMO, Ericsson, Qualcomm

Title: 	          TP for 38.803: Conclusion of NR in-band requirements

Agenda Item:	10.4.1.2

Document for:	Approval

# Introduction

NR in-band requirements have been discussed for several meetings.

Text proposal is provided to capture the study outcome into the TR [1].

# References

- TR 38.803, v1.1.1

# Text Proposal for TR 38.803 v1.1.1

## <Start of TP>

### 6.1.4 	NR in-band requirements

Based on the discussions and agreed WFs [R4-1610921, R4-1702093], it can be concluded that for NR in-band requirements,

- NR will develop DL and UL in-band emission, and in-band selectivity requirements with different numerologies on the same NR carrier

- In the first phase, two different numerologies within one carrier are assumed to define in-band requirements

- Sub-block 1 with 15kHz subcarrier spacing

- Sub-block 2 with 60kHz subcarrier spacing

- Start evaluations of in-band performance and needed requirements for mixed numerology case by evaluating 20 MHz NR carrier with roughly two 10 MHz sub-blocks with different numerologies (15 kHz and 60 kHz SCS) for <6 GHz frequencies at the first phase

- When RAN4 defines 5G NR requirements, RAN4 should ensure sufficiently good spectral efficiency in both single and mixed numerology cases

- Flexibility in approach to allocating non used PRBs (guard) and filtering/windowing in order to maximize spectral efficiency for each specific link situation and different reference measurement channels e.g. with different MCSs should be enabled

- For NR mixed numerology case RAN4 should specify new type of minimum requirements to define. These might include some of the following

  - how close to each other in frequency BS Tx can transmit two sub-blocks with different numerologies while still meeting the other relevant requirements like the EVM

    - The possibility of several requirements with different  sub-block spacing, different reference measurement channels (e.g. different MCS) and EVM may need to be considered

    - Requirements should consider both the interference arising from the other numerology and impact to EVM due to filtering of a numerology

  - how close to each other in frequency two sub-blocks with different numerologies could be while BS Rx and UE Rx still need to meet the relevant receiver requirements like the selectivity

    - The possibility of several requirements with different sub-block spacing and selectivity may need to be considered

- It is FFS how many requirements are needed for ensuring flexibility to optimize for different link situations and whether all requirements should be mandatory

  - Companies should express analysis & views on which requirements are needed and whether they should be mandatory

- NR UL in-band emission and EVM requirements at UE Tx

- Develop UE Tx in-band emission and EVM requirements for the baseline CP-OFDM baseline waveform

assuming suitable spectral confinement methods

- Perform link simulations with two numerologies next to each other in frequency domain to study how stringent UL in-band emission requirements would benefit frequency domain multiplexing of different numerologies within the same spectrum block

- If the current LTE based UE Tx in-band emission requirement definition is reused in NR, update the UE Tx in-band emission requirement definition so that both the same and different numerologies are verified as victim and aggressor UEs by checking all the numerologies in the test equipment receiver

  - Before agreeing the exact definition of NR UE Tx in-band emission requirement, study if it is feasible to define NR UE Tx in-band emission requirements in a new numerology independent way

- Define both average UE Tx EVM requirements measured over all the allocated PRBs and a few edge PRBs

  - Study further to decide if 1 PRB EVM measurement could be assumed as narrow bandwidth UE Tx EVM measurement

- TRX impairments like Image and carrier leakages and PA ACLR impact must be carefully budgeted when interference between different numerologies are studied

  - TRX impairments are to be considered at any UL power levels where ACLR is only to be considered for the upper part of the UE power levels

  - Need for Improved TRX impairments compared to LTE and achievable sub6GHz and mmW UE image and carrier leakage values should be further studied

- NR DL selectivity requirements at UE Rx

- Define DL in-band selectivity requirements at UE Rx with different numerologies in adjacent wanted and interfering sub-blocks

- Definition in mixed numerology case should follow the same format as uplink,

- Investigate the necessity of guard band for the targeted modulation

- Receiver complexity should be considered when defining the requirements

- NR DL EVM and in-band emission requirements at BS Tx

- EVM is required regardless on single or mixed numerology

  - Define two numerology EVM requirements with two sub-blocks with different numerologies

  - Further study on the issue that EVMs are unevenly distributed among the allocated resource blocks

  - High order modulation up to 256QAM can be studied in the evaluation

- In the two numerology case, EVM will be measured for both of the sub-blocks using the numerology used in a given sub-block edge PRBs

  - EVM performance of the edge subcarrier shall be covered

- Investigate the necessity of guard band for the two numerology EVM requirements

  - Evaluate also the feasibility of zero guard band case

  - BS scheduler may decide the used guard band

  - FFS if granularity is based on PRB or SCS

- For single numerology case define both average BS Tx EVM requirements over all the PRBs and over 1 PRB for the edge PRBs

- For mixed numerology case define both average BS Tx EVM requirements over all the PRBs of a given numerology and over 1 PRB for the edge PRBs

- Consider reducing the number of test cases and testing in first phase NR specification development by defining only EVM based requirements for BS Tx in-band requirements with the mixed numerology case

- Define the receiver assumptions for validating the requirements as Tx EVM measurement depends on the implementation of the receiver used for the measurement

- Tx and Rx complexity should be taken into account

- NR UL selectivity requirements at BS RX

- The uplink in-band selectivity requirement definition in single and mixed numerology case should target for sub-block edge PRB(s)

  - Verify the overall performance in addition

- Spatial aspects to be considered

- RAN4 will use the following assumptions and priorities for developing UL in-band selectivity requirements at BS Rx:

  - First requirements using the baseline CP-OFDM signal both for wanted and inferring signals

  - Requirements both with the same and different numerologies for wanted and interfering signals

  - First requirements using time-aligned wanted and interfering signals. Later requirements for asynchronous cases with non-time-aligned signals

  - Investigate suitable guard band between the wanted and interfering signals in case of mixed numerologies and/or non-time aligned signals

- Define the minimum requirement for NR BS in-channel selectivity requirements with mixed numerologies as follows:

For NR, the throughput shall be ≥ TBD % of the maximum throughput of the reference measurement channel as specified in Annex TBD with parameters specified in Table TBD for NR TBD type BS.

| NR channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of wanted signal | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |
| [10] | TBD | TBD | TBD | [10 MHz] NR CP-OFDMA signal, TBD RBs* | [10 MHz] NR CP-OFDMA signal, TBD RBs* |
| Note*: Wanted and interfering signal are placed around Fc with TBD RBs between the signals |  |  |  |  |  |

Note: One NR channel bandwidth is listed in the table as an example to illustrate the meaning of different columns

Additionally in [R4-1700053] for NR UL in-band emissions and EVM requirements at UE TX, it was agreed that in order to enable better UL coverage for CP-OFDM waveform RAN4 should study a possibility to define two sets of in-band emissions and EVM requirements for NR UE Tx; more and less stringent requirements.

With the above agreed methodology and assumptions, the detailed requirements for both DL and UL can be further investigated.

It was agreed in [R4-168776] that

- RAN 4 should define the Tx unit emission requirements and Rx unit selectivity requirements in the presence of predefined interfering signals.

- RAN 4 should define signal quality performance targets for Tx and Rx unit testing in predefined channels, while noting the possible balancing of EVM between Tx and Rx units.

- Independent Tx and Rx unit requirements and corresponding test setups will be defined in RAN4

<End of TP>