3GPP TSG-RAN WG4 Meeting #82BIS 	R4-1702959

Spokane, U.S.A., 3 – 7 April, 2017

Agenda Item:	7.19.6

Source: 	Intel Corporation

Title: 	Simulation results and discussion on CQI test under AWGN conditions

Document for:	Discussion

- Introduction

In this contribution, we provide the simulation results for CQI test in AWGN channels based on the agreed simulation assumptions in [1].

- Simulation Results

The simulation assumptions are shown in Table 1 below.

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |  |  |
| SNR (Note 2) |  | dB | [0] | [1] | [6] | [7] |
|  |  | dB[mW/15kHz] | [-98] | [-97] | [-92] | [-91] |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel RC.4 FDD according to Table A.4-1 with one/two sided dynamic OCNG Pattern OP.2 FDD as described in Annex A.5.1.2. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Table 1 Simulation assumptions for AWGN channel

The reporting CQI index distribution for FDD is illustrated in Table 2 below.

| SNR (dB) | Median CQI | Probability of median CQI – 1 | Probability of Median CQI | Probability of median CQI + 1 |
| --- | --- | --- | --- | --- |
| -2 | 3 | 0 | 0.8 | 0.2 |
| -1 | 4 | 0.06 | 0.94 | 0 |
| 0 | 4 | 0 | 0.97 | 0.03 |
| 1 | 5 | 0.19 | 0.81 | 0 |
| 2 | 5 | 0 | 1 | 0 |
| 3 | 5 | 0 | 0.60 | 0.40 |
| 4 | 6 | 0 | 0.97 | 0.03 |
| 5 | 7 | 0.06 | 0.94 | 0 |
| 6 | 7 | 0 | 0.97 | 0.03 |
| 7 | 8 | 0.11 | 0.89 | 0 |
| 8 | 8 | 0 | 0.98 | 0.02 |
| 9 | 9 | 0.18 | 0.82 | 0 |
| 10 | 9 | 0 | 0.99 | 0.01 |

Table 2 Reporting CQI index distribution for FDD

The BLER for median CQI and median CQI±1 of SNR = 0, 1, 6 and 7dB are provided in Table 3.

| SNR (dB) | Median CQI | BLER of Median CQI – 1 | BLER of Median CQI | BLER of Median CQI + 1 |
| --- | --- | --- | --- | --- |
| 0 | 4 | 0 | 0 | 0.6312 |
| 1 | 5 | 0 | 0 | 1 |
| 6 | 7 | 0 | 0 | 0.075 |
| 7 | 8 | 0 | 0 | 1 |

Table 3 BLER for median CQI and median CQI±1 (FDD)

- Conclusion

In this contribution, we provide the simulation results for CQI test in AWGN channels based on the agreed simulation assumptions in [1].

- References

[1] R4-1609699, “Simulation assumption for CQI tests for Cat.1 UE with single receiver chain”, Qualcomm, Nov. 2016.