3GPP TSG-RAN WG4 Meeting #82bis                                                                R4-1703312

# Spokane, US, 3 - 7 April, 2017

Title:                        UE enhancement for 5G NR in 3.5GHz

Source: 	CMCC

Agenda Item:	10.4.2.2

Document for:	Approval

1    Introduction

In last RAN plenary, NR bands and band combinations(s) in Rel-15 NR scope were approved together with NR WID [1]. Frequency range 3.3-4.2GHz has 15 supporting operators, which is one of the most promising NR bands for 5G deployments. In China, around 3.5GHz will be used for 5G field trial and commercial deployments. In order to meet the deployment requirements and ITU-R target requirements of IMT2020, some enhancement for UE from RF perspective should be considered in Rel-15 NR WI.

In this contribution, we discuss the required UE enhancement for 5G NR in 3.5GHz frequency range.

| Frequency range /LTE band | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) |
| --- | --- | --- | --- | --- | --- |
| 3.3-4.2 GHz | REL-15 | TBD | TBD | NTT DOCOMO, KDDI, SBM, CMCC, China Unicom, China Telecom, KT, SK Telecom, LG Uplus, Etisalat, Orange, Telecom Italia, British Telecom, Deutsche Telekom | new |
| 4.4-4.99 GHz | REL-15 | TBD | TBD |  | new |
| 24.25-29.5 GHz | REL-15 | TBD | TBD | NTT DOCOMO, KDDI, SBM, CMCC, KT, SK Telecom, LG Uplus, Etisalat, Orange, Verizon, T-mobile, Telecom Italia, British Telecom, Deutsche Telekom | new |
| 31.8-33.4GHz | REL-15 | TBD | TBD | Orange, Telecom Italia, British Telecom | new |
| 37-40 GHz | REL-15 | TBD | TBD | AT&T, Verizon, T-mobile | new |
| 1.427-1.518G | REL-15 | TBD | TBD | Etisalat | new |
| Band 3 | REL-15 | TBD | TBD | CMCC, China Telecom | new |
| Band 7 | REL-15 | TBD | TBD | CHTTL, British Telecom | new |
| Band 8 | REL-15 | TBD | TBD | CMCC | new |
| Band 20 | REL-15 | TBD | TBD | Orange | new |
| Band 28 | REL-15 | TBD | TBD | Orange, Swisscom, Telecom Italia, Telefonica, Vodafone | new |
| Band 41 | REL-15 | TBD | TBD | Sprint, China Telecom, C-Spire, China Unicom | new |
| Band 66 | REL-15 | TBD | TBD | T-mobile, DISH | new |
| Band 1 | REL-15 | TBD | TBD | China Unicom, China Telecom | new |

2    Discussion

In this section, we consider the UE enhancement from both downlink and uplink perspective.

2.1 Downlink enhancement: 4Rx

Multiple antennas for transmission and reception are used for improving both the user and cell throughput. Starting from LTE Rel-10, up to 8 layers is supported. RAN4 started the related specification work for 4Rx in Rel-13.

The most important motivation of introducing UE with 4 receive antenna ports (AP) is to achieve higher data rates. The peak data rate of ITU-R IMT-2020 for enhanced Mobile Broadband is expected to reach 10 Gbps. However under certain conditions and scenarios IMT-2020 would support up to 20 Gbps peak data rate [2].

In Table 1, we calculate the data rates based on existing LTE features. According to the table, at least 2 layers/256QAM with 1GHz bandwidth or 4 layers/64QAM with 800MHz is required to reach 10Gbps. Hence, 4 layer MIMO is one of the key technologies in order to meet the target peak data rate of IMT-2020 for eMBB.  Except for achieving peak data rate, UE with 4Rx can also achieve additional diversity gain and enhance the cell edge throughput, which is beneficial to improve user experienced data rate.

Moreover, band 42 4Rx UE for LTE was specified in Rel-13. There are already commercial UEs supported in this band. Hence, we think it is feasible for UE to support 4Rx in 3.5GHz frequency range in technical. Therefore, we propose to specify UE with 4Rx for 3.5GHz in Rel-15 NR work item.

Proposal 1: it is proposed to specify UE with 4Rx for 3.5GHz in Rel-15 NR WI.

Table 1: Data rates with 4RX

| Bandwidth | 20MHz | 100MHz | 400MHz | 800MHz | 1GHz |
| --- | --- | --- | --- | --- | --- |
| 2-layers / 64QAM | 150Mbps | 750Mbps | 3Gbps | 6Gbps | 7.5Mbps |
| 2-layers / 256QAM | 200Mbps | 1Gbps | 4Gbps | 8Gbps | 10Gbps |
| 4-layers / 64QAM | 300Mbps | 1.5Gbps | 6Gbps | 12Gbps | 15Gbps |
| 4-layers / 256QAM | 400Mbps | 2Gbps | 8Gbps | 16Gbps | 20Gbps |

2.2 Uplink enhancement: 2Tx and +26dBm maximum output power

From operator perspective, it is desired to reuse the existing base station locations for 5G deployments due to the high cost and difficulty of finding new locations. For example, CMCC’s LTE network is mainly deployed in band 39 (1.9GHz) and band 41 (2.6GHz). Due to the higher frequency of 3.5GHz, the cell coverage may be reduced compared to 2.6GHz or 1.9GHz.

In order to understand cell coverage, we preliminary calculate the coverage difference between 2.6GHz and 3.5GHz as shown in Table 2 and 3. Note that all the values in Table 2 and 3 are the delta values by comparison between 3.5GHz and 2.6GHz. 64T64R for 3.5GHz base station and 8T8R for 2.6GHz base station is assumed. TDD configuration 2 (1:3) is assumed.

According to downlink budget (Table 2), it is observed that coverage of 3.5 GHz is similar as 2.6GHz for traffic channel, and even 4dB better than 2.6GHz for control channel.  So it is possible to reuse the existing LTE base station locations from downlink perspective in order to save the cost and accelerate 5G deployments.

According to uplink budget (Table 3), it is observed that with UE maximum output power of +26dBm and 2Tx, the uplink coverage of 3.5GHz is similar as 2.6GHz for traffic channel, but still have 3dB gap for PRACH. Therefore, in order to improve the uplink coverage for 5G deployments, +26dBm UE maximum output power and 2Tx are the key technologies and should be considered in Rel-15 NR WI.

Proposal 2: it is proposed to specify UE with +26dBm maximum output power for 3.5GHz in Rel-15 NR WI.

Proposal 3: it is proposed to specify UE with 2Tx for 3.5GHz in Rel-15 NR WI.

Table 1: Downlink budget different between 3.5GHz and 2.6GHz

| Frequency | PDCCH | PDSCH |
| --- | --- | --- |
|  | Difference between 3.5GHz and 2.6GHz (3.5GHz – 2.6GHz) dB | Difference between 3.5GHz and 2.6GHz (3.5GHz – 2.6GHz) dB |
| Propagation loss | -4.5 | -4.5 |
| Penetration loss | -2 | -2 |
| BS transmit beamforming gain | 7.5 | 2.5 |
| UE 4Rx | 3 | 3 |
| Total Difference | 4dB | -1dB |

Table 2: Uplink budget difference between 3.5GHz and 2.6GHz

| Frequency | #### PRACH | #### PUSCH |
| --- | --- | --- |
|  | Difference between 3.5GHz and 2.6GHz (3.5GHz – 2.6GHz) dB | Difference between 3.5GHz and 2.6GHz (3.5GHz – 2.6GHz) dB |
| Propagation loss | -4.5 | -4.5 |
| #### Penetration loss | -2 | -2 |
| BS receiving gain | 1.5 | 1.5 |
| +26dBm maximum output power | 3 | 3 |
| #### UE 2Tx | 0 | 2 |
| #### Total Difference | #### -2 | #### 0 |

3   Conclusions

In this contribution, we discuss the required UE enhancement for 5G NR in 3.5GHz frequency range in order to meet the 5G deployment requirements. The proposals are provided as follows:

Proposal 1: it is proposed to specify UE with 4Rx for 3.5GHz in Rel-15 NR WI.

Proposal 2: it is proposed to specify UE with +26dBm maximum output power for 3.5GHz in Rel-15 NR WI.

Proposal 3: it is proposed to specify UE with 2Tx for 3.5GHz in Rel-15 NR WI.

4   References

RP-170847, Work Item on New Radio (NR) Access Technology IMT Vision – Framework and overall objectives of the future development of IMT for 2020 and beyond, ITU-R.