3GPP TSG-RAN WG4 Meeting #82bis 					R4-1703576

Spokane, USA, 3rd – 7th April, 2017

Source:	Airbus DS SLC

Title:	A-MPR Simulation results for Band 68 UEs in Europe and Middle-East

Agenda item:	9.3.1

Document for:	Approval

# Introduction

At the last RAN4 meeting #82, the performance of the Tx filter of UEs supporting Band 68 was discussed [1].

Although an “average” filter was agreed during the Band 68 work item, it was questioned whether this agreed filter is realistic. Alternative filters were proposed, based on data received from filter manufacturers.

In this contribution, simulation results for “Option A” (i.e. 30MHz TC SAW filter from Vendor A) and “Option B” (i.e. 15MHz SAW filter from Vendor B) are presented for normal (DTT protection = -42dBm/8MHz) and extreme (DTT protection = -30dBm/8MHz) environmental conditions are presented.

The objective of this work item (LTE700_Europe_PPDR) is to Band 68 UEs (originally designed to operate in Middle-East) usable in Europe, where a new “NS” value shall be broadcasted by the network so that the UEs are aware that the OOBE to protect DTT band is more stringent.

It is thus important that the filter model that is used in this work item is also compatible with the existing requirements that are already defined in 36.101.

Thus, “Option A” and “Option B” are also evaluated in extreme environmental conditions with a DTT protection requirement of -25dBm/8MHz.

# Simulation results

## Filter characteristics

The table below is copied from [1]. Simulation results in the next section are provided for the filter agreed in the TR, Option A and Option B.

|  | TR Band 68 |  | Option A |  |  |  | Option B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TC SAW [2] 30MHz passband |  | Vendor A TCSAW 30MHz passband |  | Vendor B SAW 30MHz passband |  | Vendor B SAW 15MHz passband |  | Vendor B TCSAW 15MHz passband |  |
| MHz | Extreme | Normal | Extreme | Normal | Extreme | Normal | Extreme | Normal | Extreme | Normal |
| 693.5 | 5.7 | 10.9 | 1.2 | 5.1 | 0.0 | 2.0 | 0.0 | 10.0 | 18.0 | 20.0 |
| 692.5 | 10.9 | 18.4 | 2.4 | 12.9 | 2.0 | 8.0 | 4.0 | 16.0 | 18.0 | 20.0 |
| 691.5 | 18.4 | 26.2 | 3.3 | 27.9 | 5.0 | 15.0 | 10.0 | 20.0 | 22.0 | 24.0 |
| 690.5 | 26.2 | 30.6 | 10.0 | 33.9 | 12.0 | 21.0 | 16.0 | 26.0 | 24.0 | 24.0 |
| 689.5 | 30.6 | 31.7 | 26.0 | 33.9 | 20.0 | 28.0 | 20.0 | 30.0 | 24.0 | 24.0 |
| 688.5 | 31.7 | 32.2 | 31.0 | 37.9 | 26.0 | 28.0 | 26.0 | 30.0 | 24.0 | 24.0 |
| 687.5 | 32.2 | 32.2 | 32.0 | 47.9 | 28.0 | 28.0 | 30.0 | 30.0 | 24.0 | 24.0 |
| 686.5 | 32.2 | 32.2 | 36.0 | 48.9 | 28.0 | 28.0 | 30.0 | 30.0 | 24.0 | 24.0 |

## Simulation results

The figures below depict the needed power reduction (MPR + A-MPR) to meet the requirement for protection of the DTT band (measured in 8MHz bandwidth).

### Europe

Simulations are performed in extreme and normal environmental conditions with a requirement for protection of DTT of -30dBm/8MHz (extreme conditions) or -42dBm/8MHz (normal conditions)

| Extreme conditions | Normal conditions |
| --- | --- |
| Average filter in Band 68 TR |  |

| Extreme conditions | Normal conditions |
| --- | --- |
| Option A |  |

| Extreme conditions | Normal conditions |
| --- | --- |
| Option B |  |

### Middle-East

Simulations are performed assuming extreme environmental conditions and a requirement for protection of the DTT of -25dBm/8MHz.

Extreme conditions

Option A

Extreme conditions

Option B

# Conclusion

Simulations results for “Option A” and “Option B” defined in [1] were presented for requirements applicable in Europe and Middle-East.

It is noted that none of these options strictly fulfil the existing performance requirements defined in 36.101 for Band 68 (no A-MPR for LTE 5MHz).

Thus, for consistency with the past work on Ban 68, it would be preferable to keep the average filter defined in TR36.893 [3].

However, “Option B” might be considered as a compromise since the amount of required A-MPR based on this assumption is relatively moderate for Europe and that the technical feasibility has been verified.

Moreover, adopting this option to derive the requirements does not preclude other implementation.

# References

[1]	R4-1701021, Band 68 A-MPR simulation results, Nokia

[2]		R4-1610832, Way forward on A-MPR simulation assumptions for Band 68, Airbus DS SLC, Nokia, RAN4#81

[3]	3GPP TR 36.893 V13.1.0 (2016-06), 700MHz band for Arab Region