3GPP TSG RAN WG4 Meeting #82bis              R4-1704012

Spokane, US, 3 - 7 April, 2017

China Communications Standards Association

# Guangzhou, 30 March, 2017

Ref: 2016-09-01

Tel: +86 10 62302645

Fax: +86 10 62301849

E-mail: zhaosz@ccsa.org.cn

# OH, KyoungseokSECRETARY

# 3GPP RAN4

E-mail:kyoungseok.oh@etsi.org

cc:  ingbert.sigovich@etsi.org  (SIGOVICH, Ingbert, SECRETARY, 3GPP RAN5)

wangruixin@caict.ac.cn (CATR)

Subject: LS on CCSA progress on LTE OTA TRP and TRS test requirements for 2CC CA capable UE

In October of 2016, CCSA TC9 WG1 sent a LS to 3GPP RAN4 to inform the progress of CCSA LTE OTA TRP and TRS requirements for non-CA configuration.  The requirements for B38, B39, B40 and B41 were introduced.

In CCSA TC9 WG1 #37 meeting, LTE OTA TRP and TRS requirements for 2CC CA capable UE were approved and will be sent to TC9 plenary for final approval in May(TBD). CCSA would like to kindly inform 3GPP RAN4 regarding the progress. The key principles are concluded as follows.

- LTE OTA requirements for 2CC CA capable UE  have been developed based on non-CA requirements with some back-off. CCSA agreed on 1.5dB back-off for both TRP and TRS, compared to non-CA requirements.

- To increase the test efficiency, the OTA test for each component carrier can be exempted if CA test is passed.

TRP and TRS of CCSA LTE OTA requirements for 2CC CA capable UE are summarized in brief for information. These requirements apply to B38 and B40 as well as the band combinations of B1+B3 and B39+B41.

- CCSA LTE OTA TRP requirements for 2CC CA capable UE

| TIRP / dBm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FS |  | BHHR |  | HR |  |
| Average | Minimum | Average | Minimum | Average | Minimum |
| 15 | 14 | 11 | 10 | 12.5 | 11.5 |

- CCSA LTE OTA TRS requirements for 2CC CA capable UE

| TIRS / dBm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FS |  | BHHR |  | HR |  |
| Average | Minimum | Average | Minimum | Average | Minimum |
| -86.5 | -85.5 | -83.5 | -82.5 | -85 | -84 |

# Date of Next CCSA TC9 plenary Meetings:

CCSA TC9 plenary Meeting #24	May, 2017(TBD)		TBD, China

Reference

[1]R4-167215,” LS on TD-LTE OTA TRP TRS test requirements”

[2]CCSA YD/T 1486.6-xxxx, draft for TC9 plenary approval, ”Measurement method for radiated RF power and receiver performance of wireless device Part 6: LTE wireless device”, March 30th, 2017