3GPP TSG-WG RAN4 Meeting #83 	R4-1705449

# Hangzhou, China, Date 15rd – 19th May, 2017

Source: 	Nokia

Title: 					Necessary A-MPR to meet band 71 spectrum emission mask

Agenda Item: 			9.8.2

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting SEM was agreed for band 71[1]. In this contribution we present A-MPR simulation results against this SEM. Results were already presented ealier in [2]

# 2	Discussion

Following SEM was agreed in previous RAN4 meeting, see Table1

Table 1: Additional requirements

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

A-MPR results against this SEM are presented in Table 2.

Table 2: A-MPR results

| Modulation | 15 MHz | 20 MHz |
| --- | --- | --- |
| QPSK | 0 | 0 |
| 16-QAM | 0 | 0 |
| 64-QAM | 0 | 0 |

As can be seen from Table 2 no A-MPR is necessary to meet the agreed SEM for band 71.

Proposal: No A-MPR is specified for band 71

# 3 	Summary

In this contribution we have presented A-MPR results for band 71 against agreed SEM and made following proposal.

Proposal: No A-MPR is specified for band 71

# 4	REFERENCES

[1] R4-1703984, TP for TR ab.cde: NS value and spectrum emission mask, Qualcomm, RAN4#82bis

[2] R4-1703891, Necessary A-MPR to meet US 600 MHz band spectrum emission mask, Nokia, RAN4#82bis