3GPP TSG-RAN WG4 #83	R4- 1705476

May 15th – 19th, 2017

Hangzhou, China

Agenda item:	6.1.3

Source: 	Qualcomm Incorporated, Huawei, Ericsson, Nokia

Title: 	TP for TR 36.789: updated on section 6.2

Document for:	Approval

# Introduction

In this contribution, we provide an update to the multi-node test TR 36.789 [1]. In particular, we focus on the test procedure for the outage test which is captured in section 6.2. The proposed test procedure follows the same principle adopted for the throughput test, i.e. consists of the following two steps: creating a baseline and performing the coexistence test to be compared against the baseline. The main difference with respect to the throughput test is the traffic type adopted and the pass/fail criteria, which for now is FFS.

# TP for TR 36.789 v0.0.3

We propose to modify section 6.2 in TR 36.789 as follows.

<<<<<<< Start of Changes >>>>>>>>>

## 6.2	Outage tests

<Editor’s note: Include contents related to channel access priority tests >

### 6.2.1	Test purpose

The purpose of the multi-node outage test is to verify whether a 5GHz device operating under latency-sensitive traffic (e.g. VOIP traffic) can achieve a certain level of performance when other 5GHz systems are present in the spectrum. For scenario 1b [and 2a], the purpose of this test is to evaluate the performance of IEEE 802.11 AP when a LAA eNB transmit in DL. For scenario 3b [and 4a], the purpose of the test is to help 3GPP to validate LAA and enhancement of system performance, while assessing the impact of IEEE 802.11 to LAA. There will be no pass/fail criteria for Wi-Fi devices complying with existing IEEE 802.11n and 802.11ac standards by the close of this SI. The performance metrics adopted in this test are described in section 6.2.3.

### 6.2.2	Test procedure

For Scenario 1b [and 2a], the procedure is made of the following steps:

- Baseline: Wi-Fi to Wi-Fi. In this step Node A and Node B are Wi-Fi APs, Node C and Node D are Wi-Fi STAs.

- LAA and Wi-Fi coexistence. In this step Node B is replaced with LAA eNodeB and Node D is replaced with LAA UEs

The specific configuration for the two steps in case of scenario 1b [and 2a] are listed in Table 6.2.2-1.

Table 6.2.2-1 Configuration for the throughput test in case of Scenario 1.

|  | Node B | Node A | Criteria |
| --- | --- | --- | --- |
| Step 1: | IEEE 802.11 AP | IEEE 802.11 AP | Impact of IEEE 802.11 transmitter on IEEE 802.11 device: Node A IEEE 802.11 system achieves the performance metrics described in section 6.2.3 in presence of Node B IEEE 802.11 system. |
| Step 2: | LAA eNB | IEEE 802.11 AP | Impact of LAA transmitter on IEEE 802.11 device: Node A IEEE 802.11 system achieves certain performance metrics described in section 6.2.3 in presence of Node B LAA system. |

The different signal levels to be adopted in Step 1 and 2 are specified in Section 5.1.1. The victim and aggressor traffic are specified based on what specified in Table 5.2.1-1 for scenarios 1b [and 2a].

Step-1: Determination of baseline

The step-1 is detailed below:

- Define a set of Wi-Fi APs and associated Wi-Fi STAs (terminal devices). APs and STAs need to be considered, composing a total number of [TBD1] AP-STA sets. The APs and STAs to be used are described in section 5.2.4

- Fix Link A-C with one specific AP and one specific STA model taken from the sets defined in a). The wanted signal level for the Link A-C is as specified in section 5.2.5. This is the Reference Link.

- For Link B-D, pick one AP from the set of vendors identified in a), and associate a STA from the set identified in a). The signal level for the Link B-C is specified section 5.2.5. This is the Aggressor Link. The specific STA-AP sets to be considered in this step are described in section 5.2.4. A total number of [TBD2] different configurations is obtained at this stage.

- The Interferer signal links are set as specified in section 5.1.1.

- Push the traffic type specified in Table 5.2.1-1 traffic for the reference link.

- For Link B-D, push the traffic type specified in Table 5.2.1-1.

- Record the performance metrics specified in section 6.2.3 achieved by the Link A-C averaged over 30 seconds.

- Repeat [TBD3] times to achieve enough statistical confidence. There will be total of [TBD2] x [TBD3] test points.

- Collect the data points from g) for each of the [TBD2] x [TBD3] test points.

Step-2: Coexistence case

- Step 1 is repeated by replacing the nodes in Link B-D with LAA eNodeB (Node B) and LAA UE (Node D). The wanted signal level for the Link B-D is specified in in section 5.2.5.

- The Interferer signal links are set as specified above.

- Performance metrics specified in section 6.2.4 of the Link A-C is recorded.

- The test is passed if the Pass/Fail criteria specified in section 6.2.3 is met.

### 6.2.3	Test metric and pass/fail criteria

### The test metric and pass fail criteria are FFS.

### 6.2.4	Test results

<<<<<<< End of Changes >>>>>>>>>

# Proposals

We propose to approve the TP described in section 2:

Proposal: to approve to the TP to TR 36.789 described in in section 2.

# Reference

- 3GPP TR 36.789 v0.0.3, “	Multi-node tests for LAA”, Release 13.