3GPP TSG-RAN WG4 Meeting #84	R4-1707153

Berlin, Germany, 21 - 25 August, 2017

Agenda Item:	9.5.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposal on mmWave NR BS Receiver In-band Blocking

Document for:	Approval

# 1.	Introduction

mmWave NR BS receiver in-band blocking was further discussed in RAN4#83, and a way forward was agreed [1].

This contribution provides a proposal to specify the mmWave NR BS receiver in-band blocking requirement in the RAN4 specifications per the agreed way forward.

# 2.	Discussion

Per the agreed way forward [1], the blocking probability of 1% is considered. To investigate the received blocking signal power for mmWave NR BS from an adjacent NR system, UL simulation runs have been performed using the same simulation assumptions and parameters as those RAN4 used for NR coexistence simulation for WP5D reply (as recorded in TR 38.803 [2]), and the simulation results are provided in Figures 1 to 5 below (extracted from [3-5]). Simulation runs have also been performed to study un-coordinated site deployment (100% grid shift) between victim and interfering systems, and the results are shown in Figure 6 below. It can be seen from Figures 1 to 6 that the 1% blocking probability correspond to -70dBm OTA interfering signal power level, which is decided by the urban macro scenario (as expected as the UE transmit power will be the highest to compensate for the highest path loss).

Figure 1: CDF of OTA received blocking signal power of victim BS (UMa 0% grid shift)

Figure 2: CDF of OTA received blocking signal power of victim BS (InH 30GHz)

Figure 3: CDF of OTA received blocking signal power of victim BS (InH 45GHz)

Figure 4: CDF of OTA received blocking signal power of victim BS (UMi 30GHz)

Figure 5: CDF of OTA received blocking signal power of victim BS (UMi 45GHz)

Figure 6: CDF of OTA received blocking signal power of victim BS (UMa 100% grid shift)

Moreover, it was agreed in the way forward [1] to investigate how to deal with analog and digital beamforming. Simulation runs have been performed in the UMa 0% grid shift case (the most demanding case for the received blocking signal power as shown in Figures 1 to 6) on the received blocking signal power including the antenna element gain (representing the case with fully digital beamforming where each antenna port is connected to a separate antenna element in the antenna array), as well as including both antenna array and element gain (representing the case with fully analog beamforming where one antenna port is connected to the whole antenna array). The simulation results are shown in Figures 7 and 8 below.

Figure 7: CDF of received blocking signal power with antenna element gain of victim BS (UMa 0% grid shift)

Figure 8: CDF of received blocking signal power with antenna element and array gain of victim BS (UMa 0% grid shift)

Comparing the results in Figures 7 and 8 with those in Figure 1, it can be seen at 1% blocking probability, the received blocking signal power with antenna element gain and with antenna element and array gain are around 4dB and 7dB, respectively, higher than the OTA received blocking signal power, which are much lower than the expected element and array gain at the antenna boresight (5dB and 29dB, respectively). Therefore, it can be deduced that the random directions of the blocking sources (i.e. UE transmit power) referencing to the BS antenna boresight have already been included in the OTA simulation results. Therefore, it is proposed to specify the mmWave NR BS receiver in-band blocking requirement with an -70dBm OTA interfering power, irrespective of the (analog or digital) beamforming implementation of the BS.

On the other hand, it has been proposed in [6, 7] to consider also the wanted signal level profile when deciding the in-band blocking signal power level. The uplink SINR profile of the victim UE in the UMa 0% grid shift case is shown in Figure 9 below.

Figure 9: CDF of UL SINR of victim UE (UMa 0% grid shift)

It can be seen from Figure 9 that the uplink SINR of the victim UE are similar with and without the interfering system due to the sufficient uplink ACIR. Moreover, it has been explained in TR36.804 [7] that the wanted signal mean power is specified as 6 dB above the reference sensitivity but does not mean that 6 dB degradation is allowed, it is simply a selected test parameter in order to make the interference impact measurable. Therefore, while it is logical to specify the wanted and in-band blocking signal power levels together, it is not necessary to link the in-band blocking signal power level together with the wanted signal level profile.

For the wanted signal power level, a 6dB desensitization (compared to the BS reference sensitivity) is allowed for both UTRA and E-UTRA BS in-band blocking requirements. It is proposed to adopt the 6dB desensitization (compared to the BS OTA sensitivity) also for mmWave NR BS receiver in-band blocking requirement. The SNR for the BS OTA sensitivity can be obtained at 95% relative throughput from link level simulations.

Moreover, the interfering signal should be defined as the same type as the wanted signal, with carrier frequency offset of two times the channel bandwidth of the wanted signal, as the receiver performance with an adjacent interfering signal is verified by the ACS requirement.

Besides, it is proposed to consider any extension on the lower and upper boundaries that would be agreed for the NR BS spectrum emission mask also for the in-band blocking requirement, as the front-end filter performance should be similar for the transmitter and receiver sides in mmWave.

# 3.	Conclusion and proposals

This contribution has provided a proposal to specify the mmWave NR BS receiver in-band blocking requirement in the RAN4 specifications per the agreed way forward.

Proposals:

1)	To specify the mmWave NR BS receiver in-band blocking requirement with an -70dBm OTA interfering power, and the wanted signal level calculated as the BS OTA sensitivity plus 6dB.

2)	The SNR for the BS OTA sensitivity can be obtained at 95% relative throughput from link level simulations.

3)	The interfering signal should be defined as the same type as the wanted signal, with carrier frequency offset of two times the channel bandwidth of the wanted signal.

4)	Any extension on the lower and upper boundaries that would be agreed for the NR BS spectrum emission mask should also be considered for the in-band blocking requirement.

# References

[1]	R4-1706313, “Way forward on NR BS blocking for mmWave operation”, Ericsson, ZTE.

[2]	3GPP TR 38.803 v14.0.0, “Study on New Radio Access Technology; RF and co-existence aspects”.

[3]	R4-1703826, “Proposal on mmWave NR BS Receiver In-band Blocking (urban macro scenario)”, Nokia, Alcatel-Lucent Shanghai Bell.

[4]	R4-1703827, “Proposal on mmWave NR BS Receiver In-band Blocking (indoor hotspot scenario)”, Nokia, Alcatel-Lucent Shanghai Bell.

[5]	R4-1703828, “Proposal on mmWave NR BS Receiver In-band Blocking (dense urban scenario)”, Nokia, Alcatel-Lucent Shanghai Bell.

[6]	R4-1706812, “Blocking simulation results for NR BS by considering joint probability for receiver blocking”, Ericsson.

[7]	R4-1706822, “Some initial results for blocking to wanted signal interference ratio”, Huawei.