3GPP TSG-RAN WG4 Meeting #84	R4-1707197

# Berlin, Germany, 21 – 25 August, 2017

Source:	SoftBank Corp.

Title:	TP for TR36.715-05-01: the support of CA_1A-3A-8A-11A-28A BCS0

Agenda item:	8.5.2

Document for:	Approval

1.  Introduction

1A-3A-8A-11A-28A BCS0 was approved in 5DL/1UL basket WID [1]. This paper is to evaluate IM/harmonics and relaxation required, including the relevant TP to complete this combination.

This paper needs approval of TPs on constituent 4DL CAs also proposed in this meeting [2-5].

[Note to the rapporteur: To create a big CR, table 7 of the main text can be used to implement REFSENS exception tables : relevant table no of 36.101 is indicated in the brackets.]

2. Basic Configurations

As written in the WID, the following configurations are assumed for B1+B3+B8+B11+B28.

Table 1: 5DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3-8-11-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |

Table 2: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-8A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

3.  Co-existence

On BS side, there is no need to analyse harmonic/IMD in REL-15. On UE side, there are two harmonic issues within this 5DL CA, B8 Tx to B3 Rx (no HTF) and B28 Tx toward B11/B1 Rx (HTF assumed). The UE issues are discussed in the following sections.

4.  ΔTIB and ΔRIB

In terms of ΔTIB and ΔRIB, we can reuse the results of constituent 4DL CAs. In B8+B28, uplink is limited to B8 so we should add notes to ΔTIB and ΔRIB tables.

Table 3: ΔTIB of constituent 4DL/1UL

| ΔTIB | 1+3+8+11 | 1+3+8+28X | 1+3+11+28 | 1+8+11+28Y | 3+8+11+28Z | Proposed Value |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 | 0.3 |  | 0.3 |
| B3 | 0.8 | 0.3 | 0.8 |  | 0.8 | 0.8 |
| B8 | 0.3 | 0.6 |  | 0.6 | 0.6 | 0.6 |
| B11 | 0.9 |  | 0.9 | 0.4 | 0.9 | 0.9 |
| B28 |  | 0.5 | 0.6 | 0.6 | 0.6 | 0.6 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3 or Band 8 NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11. NOTE Z: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3, Band 8 or Band 11. |  |  |  |  |  |  |

Table 4: ΔRIB of constituent 4DL/1UL

| ΔRIB | 1+3+8+11 | 1+3+8+28X | 1+3+11+28 | 1+8+11+28Y | 3+8+11+28Z | Proposed Value |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 | 0 |  | 0 |
| B3 | 0.3 | 0 | 0.3 |  | 0.3 | 0.3 |
| B8 | 0 | 0.2 |  | 0.2 | 0.2 | 0.2 |
| B11 | 0.5 |  | 0.5 | 0 | 0.5 | 0.5 |
| B28 |  | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3 or Band 8 NOTE Y: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11. NOTE Z: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3, Band 8 or Band 11. |  |  |  |  |  |  |

Then the following values are proposed.

Table 5: Proposed ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A-28AX | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3, Band 8 or Band 11 |  |  |

Table 6: Proposed ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c Inter-band with one active UL serving cell [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A-28AX | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3, Band 8 or Band 11 |  |  |

5.  REFSENS

As noted above, two harmonic relations are included in the 4DL CA. Band 3 REFSENS should be “N/A” when UL is Band 8. The proposal is shown in Table 7.

Table 7 (7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A-28A4 | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | N/A | N/A |  |  |  |
|  | 11 |  |  | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A | N/A | N/A |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

On the other hand, since B28 uplink is not applicable due to B8+B28 limitation, the harmonic case of B28 Tx to B11/B1 Rx does not happen in this 5DL CA. The fact is captured only in NOTEs for ΔTIB and ΔRIB tables, not in REFSENS exception.

6.  Summary

This paper investigates all the necessary elements to complete CA_1A-3A-8A-11A-28A. The relevant TP is shown in Annex.

7.  Reference

[1]      RP-170935    New WI: LTE Advanced inter-band CA Rel-15 for 5DL/1UL, Nokia (Rapporteur).

[2]      R4-1707192  TP for TR36.715-04-01: the support of CA_1A-3A-8A-28A BCS0

[3]      R4-1707193  TP for TR36.715-04-01: the support of CA_1A-3A-11A-28A BCS0

[4]      R4-1707194  TP for TR36.715-04-01: the support of CA_1A-8A-11A-28A BCS0

[5]      R4-1707195  TP for TR36.715-04-01: the support of CA_3A-8A-11A-28A BCS0

Annex: TP for TR36.715-05-01 (v0.0.0)

[Unchanged portions skipped]

5.Z	CA_1A-3A-8A-11A-28A_BCS0

5.Z.1	Channel bandwidths per operating band for CA

Table 5.Z.1-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-3A-8A-11A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 11 |  |  | Yes | Yes |  |  |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

5.Z.2	UE co-existence studies

On UE side, there are two harmonic issues within this 5DL CA, B8 Tx to B3 Rx (no HTF) and B28 Tx toward B11/B1 Rx (HTF assumed). The UE issues are discussed in the following sections.

5.Z.3	∆TIB and ∆RIB values

For the UE which supports inter-band carrier aggregation configurations with uplink assigned to one E-UTRA band the ΔTIB,c and ΔRIB,c values are defined for applicable bands in Table 5.Z.3-1 and 5.Z.3-2 respectively. In B8+B28, uplink is limited to B8 so notes should be added to ΔTIB and ΔRIB tables.

Table 5.Z.3-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A-28AX | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3, Band 8 or Band 11 |  |  |

Table 5.Z.3-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-8A-11A-28AX | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| NOTE X: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3, Band 8 or Band 11 |  |  |

5.Z.4	REFSENS requirements

As noted above, two harmonic relations are included in the 5DL CA. Band 3 REFSENS should be “N/A” when UL is Band 8. The proposal is shown in the table below.

Table 5.Z.4-1 (36.101: 7.3.1A-0a): Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-8A-11A-28A4 | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 3 |  |  | N/A | N/A | N/A | N/A |  |
|  | 8 |  |  | N/A | N/A |  |  |  |
|  | 11 |  |  | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A | N/A | N/A |  |
| NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

On the other hand, since B28 uplink is not applicable due to B8+B28 limitation, the harmonic case of B28 Tx to B11/B1 Rx does not happen in this 5DL CA.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"