3GPP TSG-WG RAN4 Meeting #84 	R4-1707592

# Berlin, Germany, Date 21st  – 25th Aug, 2017

Source: 	Nokia

Title: 					TP to TR 36.715-03-01: CA_3DL_2A-4A-28A_1UL_BCS0

Agenda Item: 			8.3.2

Document for:	Approval

# Introduction

This TP introduces 3DL/1UL CA configuration CA_2A-4A-28A in to the TR 36.715-03-01

**************************** Start of changes ********************************************

## 6.X	CA_2A-4A-28A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-4-28 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-4A-28A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-3 summarizes frequency ranges where harmonics occur due to Band 2, Band 4 and Band 28 CA with 1 UL.

Table 6.X.3-3: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |
| 4 | 1710 | 1755 | 3420 | 3510 | 5130 | 5265 |
| 28 | 703 | 748 | 1406 | 1496 | 2109 | 2244 |

It can be seen from Table 6.X.3-3 for CA_2A-4A-28A that there is 3rd harmonic interterference from band 28 uplink towards band 4 downlink.

### 6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 2, Band 4 and Band 28 for 1UL/3DL CA_2A-4A-28A, the TIB,c and RIB,c  values are shown in table 6.X.4-1 and  table 6.X.4-2, respectively. This configuration is very similar to CA_2A-4A-12A and therefore the applicaple relaxations for that CA configuration are re-used.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-28A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 28 | 0.8 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRB,c [dB] |
| --- | --- | --- |
| CA_2A-4A-28A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 28 | 0.5 |

### 6.X.5	REFSENS requirements

Reference sensitivity exceptions for CA_2A-4A-28A are defined due to third harmonic interference from the uplink in Band 28 to the downlink in Band 4.  The impact of the 3rd harmonic has already been studied and specified for CA_2A-4A-12A. The reference sensitivity for Band 4 is adjusted by exception when there is Tx-to-Rx harmonic overlap to account for the interference in Tables 7.3.1A-0a and 7.3.1A-0b in TS 36.101.  The same values are to be applied to CA_2A-4A-28A.

Table 6.X.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-4A-28A 1,2 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 28 |  |  | -98 | -95 | -93.2 | -90.5 |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 2: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

Table 6.X.5-2: Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-4A-28A | 28 |  |  | 8 | 16 | 25 |  | FDD |

**************************** End of changes ********************************************