3GPP TSG-RAN WG4 Meeting #84						R4-1707998

Berlin, Deutsche, 21 - 25 August, 2017

Source:		NTT DOCOMO, INC., SGS Wireless

Title:	MIMO OTA measurement results for band 1 and 19

Agenda Item:	7.2.1.2

Document for:	Discussion

Introduction

This contribution provides measurement result of LTE band 1 and 19 for defining the final requirement of MIMO OTA. There are 15 samples results align with the agreed framework in the last ad-hoc meeting [1]. These results can be a base of TRMS requirement.

Test Conditions

The measurement is done at SGS Taiwan which is CATL. In the last meeting, we have completed the harmonization with CATR which is reference lab of this work. Test condition aligns with [2] and same chamber is used in [3]. Information of measurement equipment is shown in Table 1. 15-samples are measured in this contribution.

Table 1.  Measurement equipment

| Equipment | Manufacturer |
| --- | --- |
| Anechoic chamber | MVG Star MIMO |
| Channel emulator | Keysight Technology Anite F32-16C |
| BS emulator | Anritsu MT8820C |
| Network Analyzer | Agilent E5071C |

Test results

Measurement results

6-results of the devices have been provided past meeting [4]. However, these data was gotten before harmonization with CATR. So, new 15 results are provided in this contribution.

Table 2 is summary of measurement results of TRMS for band 1 and 19.

Table 2.  Summary for band 1data for 70% of TRMS .

| DUT | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -98.13 | -97.21 | -96.14 | -97.24 | 0 | 20 | 5H 6L |
| Sample_2 | -99.71 | -99.70 | -97.50 | -99.09 | 0 | 21 | 8H 5L |
| Sample_3 | -101.74 | -102.15 | -99.01 | -101.17 | 0 | 12 | 2H 2L |
| Sample_4 | -101.08 | -100.27 | -97.91 | -99.95 | 0 | 13 | 3H 2L |
| Sample_5 | -98.83 | -99.86 | -96.59 | -98.63 | 0 | 12 | 3H 1L |
| Sample_6 | -97.83 | -100.72 | -97.81 | -99.01 | 0 | 14 | 3H 3L |
| Sample_7 | -104.08 | -104.42 | -100.80 | -103.38 | 0 | 24 | 10H 6L |
| Sample_8 | -101.30 | -101.44 | -98.81 | -100.67 | 0 | 23 | 9H 6L |
| Sample_9 | -99.73 | -100.30 | -96.35 | -99.11 | 0 | 14 | 3H 3L |
| Sample_10 | -102.82 | -103.13 | -98.94 | -102.00 | 0 | 13 | 3H2L |
| Sample_11 | -101.86 | -103.74 | -98.89 | -101.93 | 0 | 21 | 8H 5L |
| Sample_12 | -96.05 | -96.19 | -91.23 | -95.01 | 0 | 21 | 8H 5L |
| Sample_13 | -99.42 | -99.01 | -95.94 | -98.38 | 0 | 17 | 6H 3L |
| Sample_14 | -99.35 | -99.64 | -98.79 | -99.27 | 0 | 13 | 3H 2L |
| Sample_15 | -101.30 | -100.39 | -99.50 | -100.46 | 0 | 32 | 13H 12L |

Table 3.  Summary for band 1data for 95% of TRMS .

| DUT | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -96.15 | -95.26 | -94.21 | -95.28 | 0 | 20 | 5H 6L |
| Sample_2 | -97.48 | -97.87 | -95.71 | -97.11 | 0 | 21 | 8H 5L |
| Sample_3 | -99.81 | -100.30 | -97.24 | -99.31 | 0 | 12 | 2H 2L |
| Sample_4 | -99.22 | -98.46 | -96.17 | -98.13 | 0 | 13 | 3H 2L |
| Sample_5 | -96.97 | -97.98 | -94.86 | -96.79 | 0 | 12 | 3H 1L |
| Sample_6 | -95.68 | -98.89 | -95.91 | -97.09 | 0 | 14 | 3H 3L |
| Sample_7 | -102.78 | -103.15 | -99.50 | -102.09 | 0 | 24 | 10H 6L |
| Sample_8 | -99.77 | -99.93 | -97.35 | -99.17 | 0 | 23 | 9H 6L |
| Sample_9 | -97.81 | -98.45 | -94.27 | -97.19 | 0 | 14 | 3H 3L |
| Sample_10 | -100.96 | -101.34 | -97.20 | -100.19 | 0 | 13 | 3H2L |
| Sample_11 | -100.32 | -102.19 | -97.53 | -100.41 | 0 | 21 | 8H 5L |
| Sample_12 | -93.75 | -93.80 | -88.94 | -92.67 | 0 | 21 | 8H 5L |
| Sample_13 | -97.50 | -96.88 | -94.20 | -96.41 | 0 | 17 | 6H 3L |
| Sample_14 | -97.38 | -97.59 | -96.84 | -97.28 | 0 | 13 | 3H 2L |
| Sample_15 | -99.45 | -98.58 | -97.73 | -98.65 | 0 | 32 | 13H 12L |

Table 4.  Summary for band 19 data for 70% of TRMS .

| DUT | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -96.71 | -97.51 | -94.10 | -96.33 | 0 | 20 | 5H 6L |
| Sample_2 | -97.57 | -95.31 | -93.76 | -95.83 | 0 | 21 | 8H 5L |
| Sample_3 | -98.65 | -98.37 | -97.06 | -98.08 | 0 | 12 | 2H 2L |
| Sample_4 | -99.11 | -99.87 | -96.90 | -98.80 | 0 | 13 | 3H 2L |
| Sample_5 | -98.10 | -97.33 | -95.29 | -97.06 | 0 | 12 | 3H 1L |
| Sample_6 | -97.35 | -96.92 | -94.20 | -96.36 | 0 | 14 | 3H 3L |
| Sample_7 | -98.31 | -95.19 | -96.75 | -96.94 | 0 | 24 | 10H 6L |
| Sample_8 | -98.06 | -97.85 | -94.83 | -97.14 | 0 | 23 | 9H 6L |
| Sample_9 | -98.88 | -96.79 | -96.05 | -97.41 | 0 | 14 | 3H 3L |
| Sample_10 | -96.73 | -96.91 | -95.78 | -96.50 | 0 | 13 | 3H2L |
| Sample_11 | -96.96 | -94.66 | -93.69 | -95.33 | 0 | 21 | 8H 5L |
| Sample_12 | -99.64 | -99.92 | -95.49 | -98.76 | 0 | 21 | 8H 5L |
| Sample_13 | -97.69 | -93.60 | -93.09 | -95.32 | 0 | 17 | 6H 3L |
| Sample_14 | -97.49 | -96.52 | -95.46 | -96.57 | 0 | 13 | 3H 2L |
| Sample_15 | -97.47 | -96.16 | -94.32 | -96.17 | 0 | 32 | 13H 12L |

Table 5.  Summary for band 19 data for 95% of TRMS .

| DUT | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -94.79 | -95.50 | -92.26 | -94.39 | 0 | 20 | 5H 6L |
| Sample_2 | -95.39 | -92.72 | -91.64 | -93.55 | 0 | 21 | 8H 5L |
| Sample_3 | -96.18 | -96.46 | -95.14 | -95.96 | 0 | 12 | 2H 2L |
| Sample_4 | -97.00 | -97.88 | -95.11 | -96.81 | 0 | 13 | 3H 2L |
| Sample_5 | -96.17 | -95.38 | -93.44 | -95.14 | 0 | 12 | 3H 1L |
| Sample_6 | -95.14 | -94.99 | -91.75 | -94.21 | 0 | 14 | 3H 3L |
| Sample_7 | -96.26 | -93.19 | -94.70 | -94.90 | 0 | 24 | 10H 6L |
| Sample_8 | -96.20 | -95.92 | -93.04 | -95.27 | 0 | 23 | 9H 6L |
| Sample_9 | -96.80 | -94.68 | -94.07 | -95.35 | 0 | 14 | 3H 3L |
| Sample_10 | -94.71 | -95.01 | -93.93 | -94.57 | 0 | 13 | 3H2L |
| Sample_11 | -95.04 | -92.57 | -91.76 | -93.35 | 0 | 21 | 8H 5L |
| Sample_12 | -97.67 | -97.93 | -93.70 | -96.81 | 0 | 21 | 8H 5L |
| Sample_13 | -95.71 | -91.50 | -91.19 | -93.32 | 0 | 17 | 6H 3L |
| Sample_14 | -95.19 | -94.16 | -93.18 | -94.25 | 0 | 13 | 3H 2L |
| Sample_15 | -95.59 | -94.32 | -92.48 | -94.31 | 0 | 32 | 13H 12L |

We have observed that throughput reached below 60% at least one azimuth for all orientations, bands, and devices. Figure 1- 6 show the example of results which is for sample 1.

Figure 1  Throughput results for band 1 (Ψ=0; Θ=-45; Φ=0)

Figure 2   Throughput results for band 1 (Ψ=90; Θ=-45; Φ=0)

Figure 3  Throughput results for band 1 (Ψ=0; Θ=90; Φ=0)

Figure 4  Throughput results for band 19 (Ψ=0; Θ=-45; Φ=0)

Figure 5   Throughput results for band 19 (Ψ=90; Θ=-45; Φ=0)

Figure 6  Throughput results for band 19 (Ψ=0; Θ=90; Φ=0)

Conclusion

In this contribution, evaluation of 15-samples for band 1 and 19 are provided. Based on the results, MIMO OTA requirement for band 1 and 19 can be established.

References

[1]    R4-1706413, Proposals on TRMS framework, CATR, 3GPP RAN4#MIMO OTA ad-hoc

[2]  TR 37.977, V14.3.0, “Verification of radiated multi-antenna reception performance of UEs,” 3GPP RAN #75, March 2017

[3]    R4-1706401, Measurement results for lab alignment test activity – Re-tested at SGS, NTT DOCOMO INC., SGS Wireless, MVG Industries, RAN4#MIMO OTA ad-hoc

[4]    R4-1704529, MIMO OTA measurement results for band 1 and 19, NTT DOCOMO, SGS Wireless, RAN4#83