3GPP TSG-RAN WG4 Meeting #84 	R4-1708902

Berlin, Germany, 21st – 25th August 2017

Agenda item:	9.4.1

Source: 	NTT DOCOMO, INC.

Title: 	Evening AH minutes for NR UE RF

Document for:	Approval

Agenda

The contribution summarizes minutes of evening AH meeting where the following topics are handled.

- MPR simulation assumptions for sub-6 GHz

- LTE - NR band combinations with large Rx MSD

- Power class and power control for mm Wave

MPR simulation assumptions for sub-6 GHz

- MPR simulation assumptions for sub-6 GHz

- PA calibration point(slide 4)

  - Proposal 1: PA calibration point: No MPR allowed for 100 RB QPSK DFT-s-OFDM (15KHz SCS) signal

  - Proposal 2: The PA was calibrated to the conventional LTE reference i.e. 1 dB MPR for 100 RB QPSK DFT-s-OFDM (15KHz SCS) signal

KDDI: Support Proposal 1.

DCM: Support P1 if the impact of the P1 on power consumption is a reasonable level.

SBM: We would like to check if there are any penalties or not.

Qualcomm: About penalty for power consumption is for 4G and/or 5G?

SBM: We are not sure what the 5G like to be…at least could be equivalent to 4G.

Conclusion: No consensus

Session chair note : Action point would be check how much benefit we can obtain from Proposal 1. SCS is improved in NR. Hence, if the protection requirements for victim system like PHS from NR Band 1 is the same, required power back off for NR would be larger than that for LTE if the PA calibration point is the same.

- Tx Modulator impairments (slide 5)

  - Proposal 1:

      - IQ suppression 30 dBc

      - Carrier leakage 28 dBc

      - CIM3 = 60 dBc up to 3.8 GHz (Band 43). Above 3.8 GHz FFS.

      - Note: How the IQ suppression and carrier leakage are specifed as minimum requirement is FFS. Above numbers are only used as simulations assumptions for MPR studies

  - Proposal 2: LO and IQ image assumed to be -25 dBc.  CIM3 was not found to be relevant for general MPR requirements

Qualcomm: 28 dBc and 28 dBc for IQ suppression and carrier leakage, respectively would be acceptable

Nokia: it sounds nice but we need to discuss it further.

Conclusion: No consensus, alternative, however, from Qualcomm was provided.

- ACLR MBW (slide 6)

Session chair: Another alternative would be ACLR is a ratio of power of wanted signal within transmission bandwidth for a SCS for a band/frequency range and power falling into victim system from the wanted signal within an available maximum transmission bandwidth among the SCSs in the band.

Note that the original text above was “Aggressor is transmission bandwidth with a SCS and victim channel bandwidth with available maximum transmission bandwidth for the SCS.” But it was modified.

Conclusion: No consensus, alternative, however, from session chair was provided.

- Spectrum utilization (slide 7)

Qualcomm: Equation 2 in the slide has a math error. We’d like to allow to put the worst case positon of RB start 0 for DFT-s-OFDM RB allocation for MPR evaluation.

Conclusion: Qualcomm’s comment would be taken into account as the next step.

- Sub-block alignment (slide 8)

Nokia: we need to correct some values.

Conclusion: It seems what we try to capture in a table is OK, some errors must be corrected and be double-checked.

- MPR Table format (slide 9)

Session chair/Qualcomm: there may be cases not covered by this table.

Qualcomm: If we look at existing table today, A-MPR is based on the length of contiguous RBs and its position. Hence, we need to careful about not making the table complex too much.

Nokia: we can discuss the table format further after seeing simulation results in the next meetings.

Conclusion: It seems ok to use the proposed format meanwhile. Further fine tune would be applied after seeing the simulation results.

LTE - NR band combinations with large Rx MSD

- WF on LTE - NR band combinations with large Rx MSD MPR simulation assumptions for sub-6 GHz

Apple: we do not agree with the 2nd point. The reason why RAN1 selected this approach is to avoid supporting 2Tx for particular CA band combination whose MSD is significantly large.

Oppo: we share the similar view with Apple. 2nd point is impose on too much constraint for UE implementation.

Ericsson: RAN4 cannot decide if we mandate UE support 2Tx for all NSA combination or not.

Dish: it would be good to distinguish small and large clearly.

Intel: The two bullets are contradicting each other. We do not have to mandate to support TDM also we do not have to mandate UE to support 2Tx.

Ericsson: Indeed NW needs to know if UE can use TDM feature or not. We specify the dual UL. If the UE has significant MSD, then, this TDM feature would be used for that band.

Nokia: we have requirements for 2Tx and if we identify significant MSD, we apply TDM to the UE.

Apple: the 1st bullet says dual UL is default. Large or small is not sure. If we share single UL, transmit power is different from two bands, so spec is very complex. We have two options but we do not specify based on small or large.

Intel: why does UE have to decide if the UE uses TDM or not?

SBM: if the single Tx is used always, throughput will be lost. This is a matter of balance. Only with TDM, operators need to give up throughput, but with only 2tx, they may lose coverage for DC.

Intel: we understand the benefit from scheduling point. With this MSD, it is hard to say which can provide more benefit.

Conclusion: No consensus.

Session chair note: More clarification on the benefit and how we utilize the signalling capability in real network.

Power class and power control

- WF on power class and power control

Clarification on terms for Power class (Slide 2)

- Minimum EIRP definition

  - Huawei: difficult to understand the text. PPower class is included in 36.101 and Power class is connected with maximum output power. We can decouple power class and PPower class not to impact on Ran1 discussion. We should define one requirement.

Power class for smartphone (Slide 3)

Sony: is it possible to decouple 43dBm from the proposed table?

Intel: we agreed that we specify EIRP as % tile but this table has peak EIRP last meeting.

Qualcomm: there is a regulatory requirement. At least is Table for Maximum allowed TRP agreeable?

No objection for TRP.

EIRP format (Slide 5)

LGE: we may reduce the number of parameters in the table to make the future discussion easier.

Conditions for providing EIRP (Slide 6)

<Beam pointing loss>

MTK: beam pointing loss is static number

Sony: if it comes from imperfection, it is a design margin.  In short, beam pointing loss should be left as test uncertainty.

<UV or theta/phi mapping coordinates>

Sony: from simulation point of view, it does not matter.

Huawei: we provided that CDF from tow coordinates, they were different.

Sony: Even we use theta/phi mapping coordinate, as far as weigh is considered, it can provide the same condition as that for UV coordinate.

Session chair: Just describing theta/phi mapping coordinate produces misunderstanding. Need to consider the unified term for coordinate for measurement.

Conclusion: No consensus.