3GPP TSG-RAN WG4 Meeting #84bis 	R4-1710186

# Dubrovnik, Croatia, 9th – 13th Oct 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1082 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce burst model for eLAA PUSCH demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eLAA-Perf |  |  |  |  |  |  | Date: |  | 2017-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 group agreed to have burst model for eLAA PUSCH demodulation, this model is not captured in the current specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the burst model for eLAA PUSCH demodulation and make some clarifcation on the terminology |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no burst model for eLAA PUSCH demodulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 A.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

### 8.2.8	Performance requirements of PUSCH with Frame structure type 3

#### 8.2.8.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

A test for a specific number of receive antenna is only applicable if the BS supports it.

#### 8.2.8.2	Minimum Requirement

The minimum requirement is in TS 36.104 [2] subclause 8.2.8.

#### 8.2.8.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under multipath fading propagation conditions with uplink resource allocation type 3 for a given SNR.

#### 8.2.8.4	Method of test

##### 8.2.8.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.8.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.8.4.2-1.

Table 8.2.8.4.2-1: AWGN power level at the BS input

| Channel bandwidth [MHz] | AWGN power level |
| --- | --- |
| 20 | -80.4dBm / 18MHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.8.4.2-2.

Table 8.2.8.4.2-2: Test parameters for testing PUSCH

| Parameter | Value (NOTE 1) |
| --- | --- |
| Maximum number of valid HARQ transmissions (NOTE 2) | 4 |
| RV sequence | 0, 2, 0, 2 |
| PUSCH starting position (NOTE 3) | ‘01’ |
| PUSCH ending symbol | ‘0’ |
| NOTE 1: PUSCH scheduling pattern is changed as per the following bitmap that repeats every 20ms, PUSCH scheduling subframe bitmap = {0000 0a11 1100 000b cccc}. In the subframe indicated by “a” or “1”, PUSCH is scheduled and transmitted, in the subframe indicated by “b” or “c”, PUSCH is scheduled but not transmitted, and the subframe indicated by “0” is reserved for downlink transmission. NOTE 2: The PUSCH which is scheduled and transmitted is counted as valid HARQ transmission and the PUSCH which is scheduled and not transmitted is not counted as valid HARQ transmission NOTE 3: In the subframe indicated by “a” or “b”, PUSCH is scheduled and the associated DCI indicates “PUSCH starting position” as ‘01’, and in the subframe indicated by “1” or “c”, PUSCH is scheduled and the assocated DCI indicates “PUSCH starting positon” as “00”. |  |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in Table 8.2.8.5-1 is achieved at the BS input.

5)	For each of the reference channels in Table 8.2.8.5-1 applicable for the base station, measure the throughput, according to annex E as per the burst transmission model defined in Annex TBD.

#### 8.2.8.5	Test Requirement

The throughput measured according to subclause 8.2.8.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.8.5-1.

Table 8.2.8.5-1: Test requirements for eLAA PUSCH, 20MHz Channel Bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Fraction of norminal maximum throughput (NOTE 1) | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | EPA 5Hz Low | A20-1 | 70% | [0.4] |
|  |  |  |  | A20-2 | 70% | [12.7] |
|  | 4 | Normal | EPA 5Hz Low | A20-1 | 70% | [-2.6] |
|  |  |  |  | A20-2 | 70% | [8.9] |
| NOTE1: Fraction of nominal maximum throughput is calculated based on the actual transmitted PUSCH norminal |  |  |  |  |  |  |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

----------------------------------------------------------unchanged section is skiped----------------------------------------------------

# A.20	Fixed Reference Channels for PUSCH with Frame structure type 3

# Table A.20-1: FRC parameters for performance requirements

| Reference channel | A18-1 | A18-2 |
| --- | --- | --- |
| Allocated resource blocks | 50 | 50 |
| DFT-OFDM Symbols per subframe | 11 | 11 |
| Modulation | QPSK | 16QAM |
| Code rate | 1/3 | 3/4 |
| MCS index | 5 | 19 |
| Payload size (bits) | 4392 | 19848 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 24 |
| Number of code blocks - C | 1 | 4 |
| Coded block size including 12bits trellis termination (bits) | 13260 | 14988 |
| Total number of bits per the sub-frame with the PUSCH starting positions indicated as “01” in the associated DCI | 13200 | 26400 |
| Total RE per the sub-frame with the PUSCH starting positons indicated as “01” in the associated DCI | 6600 | 6600 |
| Total number of bits per the sub-frame with the PUSCH starting positions indicated as “00” in the associated DCI | 14400 | 28800 |
| Total RE per the sub-frame with the PUSCH starting positons indicated as “00” in the associated DCI | 7200 | 7200 |

--------------------------------------------------------------End of Change ---------------------------------------------------------------