3GPP TSG-RAN WG4 Meeting #84bis                                                         R4-1710239

# Dubrovnik, Croatia, 9-13 October 2017

Agenda Item:	9.7.12.1

Source:	Intel Corporation

Title:	SS block RSRP link level simulation result

Document for:	Discussion

# Introduction

In this contribution, we updated our simulation results for NR-SSS based RSRP.

# Simulation results

The detailed simulation parameters can be found in [1]. The simulation results for RSRP based on NR-SSS are given with the CDF of measurement error below for the following simulation cases firstly.

- Subcarrier spacing: 15k/30K/120K/240K

- Carrier frequency: 4G Hz/30G Hz

- Number of SS blocks per SS burst set : K=1

- UE receive antennas: 4(no beamforming)

- SNR: Cell 1(1dB), cell 2(6dB)

- Cell1 ID: (1,101)  cell 2 ID (0, 240)

- Scaling of delays = 100ns for TDL A/B/C

Simulation results are in following figures.

Fig.1 EPA for scs=15K and scs=30K

Fig.2 ETU30 for scs=15K and scs=30K

Fig.3 ETU70 for scs=15K and scs=30K

Fig.4 TDL-A for scs=120K and scs=240K

Fig.5 TDL-B for scs=120K and scs=240K

Fig.6 TDL-B for scs=120K and scs=240K

Table 1 and Table 2 are simulation results for 15K Hz and 30K Hz respectively.

Table 3 and Table 4 are simulation results for 120K Hz and 240K Hz respectively.

Table 1:  simulation results for 15K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| EPA5 | 1 sample | -0.3 | 2.6 | 2.6 |
|  | 3 sample | 0.1 | 1.55 | 1.55 |
|  | 5 sample | 0.2 | 1.25 | 1.25 |
| ETU30 | 1 sample | -1.42 | 1.75 | 1.75 |
|  | 3 sample | -0.82 | 1 | 1 |
|  | 5 sample | -0.6 | 0.75 | 0.75 |
| ETU70 | 1 sample | -1.5 | 1.8 | 1.8 |
|  | 3 sample | -0.75 | 1.1 | 1.1 |
|  | 5 sample | -0.5 | 0.7 | 0.7 |

Table 2:  simulation results for 30K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| EPA5 | 1 sample | -0.5 | 2.6 | 2.6 |
|  | 3 sample | 0.05 | 1.58 | 1.58 |
|  | 5 sample | 0.1 | 1.4 | 1.4 |
| ETU30 | 1 sample | -2.3 | 1.4 | 2.3 |
|  | 3 sample | -1.5 | 0.77 | 1.5 |
|  | 5 sample | -1.28 | 0.5 | 1.28 |
| ETU70 | 1 sample | -2.3 | 1.25 | 2.3 |
|  | 3 sample | -1.55 | 0.55 | 1.55 |
|  | 5 sample | -1.3 | 0.4 | 1.3 |

Table 3:  simulation results for 120K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| TDL-A | 1 sample | -0.7 | 2.35 | 2.35 |
|  | 3 sample | -0.2 | 1.35 | 1.35 |
|  | 5 sample | -0.02 | 1.1 | 1.1 |
| TDL-B | 1 sample | -0.6 | 2.4 | 2.4 |
|  | 3 sample | -0.2 | 1.3 | 1.3 |
|  | 5 sample | -0.08 | 1.15 | 1.15 |
| TDL-C | 1 sample | -0.8 | 2.4 | 2.4 |
|  | 3 sample | -0.3 | 1.4 | 1.4 |
|  | 5 sample | -0.08 | 1.2 | 1.2 |

Table 4:  simulation results for 240K Hz

|  |  | 5% (dB) | 95% (dB) | Max (dB) |
| --- | --- | --- | --- | --- |
| TDL-A | 1 sample | -1 | 1.87 | 1.87 |
|  | 3 sample | -0.35 | 1.08 | 1.08 |
|  | 5 sample | -0.22 | 0.87 | 0.87 |
| TDL-B | 1 sample | -0.84 | 2.05 | 2.05 |
|  | 3 sample | -0.4 | 1.1 | 1.1 |
|  | 5 sample | -0.19 | 1.02 | 1.02 |
| TDL-C | 1 sample | -0.8 | 1.9 | 1.9 |
|  | 3 sample | -0.35 | 1.18 | 1.18 |
|  | 5 sample | -0.18 | 1 | 1 |

Observation 1: the RSRP accuracy gain from N=3 to N=1 is larger than the gain from N=3 to N=5.

# Conclusion

In this contribution some NR link level simulation results for NR SS RSRP was provided. The following observation can be drawn:

Observation 1: the RSRP accuracy gain from N=3 to N=1 is larger than the gain from N=3 to N=5.

# References

[1] R4-1706904, “N Link-level simulation assumptions for SS block measurements”, Ericsson