3GPP TSG-RAN4 Meeting #84bis	R4-1710622

# Dubrovnik, HR, 9th – 13th October 2017

Source: 	Qualcomm Incorporated

Title: 	Transient period and its location for sTTI

Agenda item:	8.23.2

Document for:	Approval

# Introduction

In the past few meetings, transient period and its location has been discussed extensively. This paper captures all of the agreements in previous meetings with proposals to the locations of the transient period.

# Discussion

In this section, the transient periods and their locations are discussed for various cases.

## ON-OFF transient period

The agreements noted in [1] and in chairman notes in RAN4-84 meeting [2] for Tx BW >1.4 MHz the ON-OFF transient period is 10 usec.

Figure 1: ON/OFF mask for single sTTI for BW > 1.4 MHz

If 1.4 MHz BW is not required to be supported for sTTI, then the transient period for ON-OFF mask can be reduced from 20 usec to 10 usec. But if 1.4 MHz needs to be supported then transient period for ON-OFF needs to be 20 usec.

Agreement 1 (for ON-OFF mask):  For sTTI

- For Tx BW>1.4 MHz, t1=10 usec

- For Tx BW=1.4 MHz, t1=20 usec

## For consecutive TTIs

It has also been agreed in the previous meetings [2] that the transient period between consecutive symbols for sTTI will be 10+10 usec. These two symbols could belong to the same TTI or across TTIs and could contain SRS or not. The location of this transient period is defined in this section.

Agreement 2:  Transient periods for consecutive symbols for sTTI will be 10+10 usec.

There are multiple cases for consecutive TTIs as listed below:

| Case | Current symbol/ TTI | Next symbol/TTI |
| --- | --- | --- |
| 1 | 2os/3os | 2os/3os |
| 2 | Intra sTTI 2os/3os/7os hopping |  |
| 3 | 2os/3os | 1msec/7os |
| 4 | 1msec/7os | 2os/3os |
| 5 | 1msec/7os | 7os/1msec |

Table 1: List of all possible cases for transient periods between consecutive symbols

Proposal 1: Table 2, below, gives logic to decide the location for the transient periods for each of the above cases:

| Case | If condition | Then? |
| --- | --- | --- |
| 1, 2 | If the symbols across the transient are [R, R] | Then share the transient period symmetrically |
|  | Else if the symbol on the right of the transient is ‘R’ | Then irrespective of the symbol on the left, place the entire transient period in the right ‘R’ |
|  | Else if the symbol left of the transient is ‘R’ | Then irrespective of the symbol on the right, place the entire transient period in left ‘R’ |
|  | Else | Share transient period symmetrically between the two symbols/TTIs |
| 3 | For all possible data patterns | Place the entire transient period in right/next TTI |
| 4 | If the last symbol of current TTI is SRS and if the first symbol on the right sTTI is ‘R’ | Place the entire transient period in ‘R’ |
|  | Else if the last symbol of current TTI is SRS and if the first symbol on the right sTTI is ‘D’ | Share the transient period symmetrically between the two symbols/TTIs |
|  | Else | Place the entire transient period in current TTI |
| 5 | For all data patterns | Treat this case similar to that of legacy 1msec+1msec TTI |

Table 2: Proposals for the transient period location for various scenarios

Based on this logic, the transient period locations will be as shown below.

### Between two sTTIs of 2os or 3os without SRS

### Between two 2os sTTIs with SRS

### Between 2os and 1msec TTI

### Between two 7os TTIs when the last symbol is not SRS

Transition period between two consecutive 7os TTIs when the last symbol of 7OS or 1msec TTI is not SRS is shown below. Similarly, the transient period between 7os TTI and 1msec TTI will be similar as well.

### Between two 7os TTIs when the last symbol is SRS

Transient period should be placed outside of SRS as shown below.

### Between 7os TTI and 1msec TTI when the last symbol is SRS

# Conclusion

In this contribution, the agreements from the previous meetings [1,2]:

Agreement 1 (for ON-OFF mask):  For both 2os and 7os TTIs

- For Tx BW>1.4 MHz, transient time=10 usec

- For Tx BW=1.4 MHz, transient time =20 usec

Agreement 2:  Transient periods for consecutive symbols for sTTI will be 10+10 usec.

Also, this paper lists the locations of the transient periods for various scenarios have been proposed in Table 2.

# References

[1] R4-1706098, Transient time for sTTI, 3GPP TSG-RAN WG4 Meeting #83

[2] LTE RF Chairman notes for 3GPP TSG-RAN WG4 Meeting #84