3GPP TSG-RAN4 Meeting #84bis	R4-1710627

# Dubrovnik, HR, 9th – 13th October 2017

Source: 	Qualcomm Incorporated

Title: 	MSD Test configurations for xDL/2UL

Agenda item:	8.7.2

Document for:	Approval

# Introduction

The test configurations for new xDL/2UL have been proposed in [1]. This paper presents the MSD for these band combinations.

# Discussion

Table 1 shows the test configurations for these new band combinations.

Table 1 Proposed 3DL/2UL inter-band CA REFSENS exceptions and test configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_1A-41A-42A | CA_1A-42A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | TDD-FDD | IMD4 |
|  |  | 42 | 3410 | 10 | 50 | 3410 | 10 |  |  |  |
|  |  | 41 | 2515 | 10 | 50 | 2515 | 10 | TBD |  |  |
| CA_3A-7A-32A | CA_3A-7A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | FDD | IMD4 |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |  |
|  |  | 32 | - | - | - | 1470 | 5 | TBD |  |  |
| CA_3A-20A-32A | CA_3A-20A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | IMD4 |
|  |  | 20 | 842 | 5 | 25 | 801 | 5 |  |  |  |
|  |  | 32 | - | - | - | 806 | 5 | TBD |  |  |
| CA_3A-28A-41A | CA_3A-41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | TDD-FDD | IMD2 |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 |  |  |  |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | TBD |  |  |
|  | CA_3A-41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | TDD-FDD | IMD3 |
|  |  | 41 | 2660 | 5 | 25 | 2660 | 5 |  |  |  |
|  |  | 28 | 725 | 5 | 25 | 780 | 5 | TBD |  |  |
| CA_3A-41A-42A | CA_3A-41A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | TDD-FDD | IMD3 |
|  |  | 41 | 2660 | 10 | 50 | 2660 | 10 |  |  |  |
|  |  | 42 | 3545 | 10 | 50 | 3545 | 10 | TBD |  |  |

# Conclusion

Table 2 shows the MSD proposed for the new CA combinations.

Proposal:

Table 2: MSD proposal for xDL2UL CA combinations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA DL Configuration | EUTRA CA UL Configuration | EUTRA band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_1A-41A-42A | CA_1A-42A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | TDD-FDD | IMD4 |
|  |  | 42 | 3410 | 10 | 50 | 3410 | 10 |  |  |  |
|  |  | 41 | 2515 | 10 | 50 | 2515 | 10 | 10.37 |  |  |
| CA_3A-7A-32A | CA_3A-7A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | FDD | IMD4 |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 |  |  |  |
|  |  | 32 | - | - | - | 1470 | 5 | 8.63 |  |  |
| CA_3A-28A-41A | CA_3A-41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | TDD-FDD | IMD2 |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 |  |  |  |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | 23.24 |  |  |
|  | CA_3A-41A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | TDD-FDD | IMD3 |
|  |  | 41 | 2660 | 5 | 25 | 2660 | 5 |  |  |  |
|  |  | 28 | 725 | 5 | 25 | 780 | 5 | 14.93 |  |  |
| CA_3A-41A-42A | CA_3A-41A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | TDD-FDD | IMD3 |
|  |  | 41 | 2660 | 10 | 50 | 2660 | 10 |  |  |  |
|  |  | 42 | 3545 | 10 | 50 | 3545 | 10 | 26.40 |  |  |

# References

[1] R4-1708087, MSD Test configurations for xDL/2UL CA band combinations with self-interference issues, 3GPP TSG-RAN WG4 #84 meeting, Berlin, Germany, 21st – 25th August, 2017