3GPP TSG-RAN WG4 Meeting #84bis						R4-1711066

Dubrovnik , Croatia, 9 - 13 Oct, 2017

Source:		NTT DOCOMO, INC., SGS Wireless

Title:	MIMO OTA measurement results for low band

Agenda Item:	5.2.

Document for:	Discussion

Introduction

This contribution provides measurement result of LTE band 5 for defining the FDD low band requirement of MIMO OTA. The 5 samples are tested. Based on the work sheet, the requirements are proposed.

Test Conditions

This contribution provides measurement result of LTE band 5 for defining the FDD low band requirement of MIMO OTA. The 5 samples results are tested, which are measured at band 19 by [3].

Table 1.  Measurement equipment

| Equipment | Manufacturer |
| --- | --- |
| Anechoic chamber | MVG Star MIMO |
| Channel emulator | Keysight Technology Anite F32-16C |
| BS emulator | Anritsu MT8820C |
| Network Analyzer | Agilent E5071C |

Test results

Measurement results

Band 5 is not enough to reach target number of DUT (it is 15) to create CDF. The results are similar data as band 19 presented in [3]. Because of testing time we provide 5 data even though it does not still meet the target threshold.

Table 2 and 3 are summary of measurement results of TRMS for band 5.

Table 2.  Summary for band 5 data for 70% of TRMS .

| DUT (dummy) | RE EPRE [dBm//15kHz]@70% TP |  |  | TRMS (average 70%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -96.62 | -96.97 | -94.20 | -96.09 | 0 | 20 | 5H 6L |
| Sample_2 | -97.64 | -95.28 | -94.27 | -95.97 | 0 | 21 | 8H 5L |
| Sample_9 | -98.44 | -96.22 | -96.19 | -97.08 | 0 | 14 | 3H 3L |
| Sample_12 | -99.68 | -100.10 | -95.79 | -98.91 | 0 | 21 | 8H 5L |
| Sample_15 | -97.71 | -96.81 | -94.75 | -96.38 | 0 | 32 | 13H 12L |
| Sample_1 | -96.62 | -96.97 | -94.20 | -96.09 | 0 | 20 | 5H 6L |

Table 3.  Summary for band 5 data for 95% of TRMS .

| DUT (dummy) | RE EPRE [dBm//15kHz]@95% TP |  |  | TRMS (average 95%) | Total # Position Fail TP target | Number of Supporting bands |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FS DMP Ori1 | FS DML Ori2 | FS DMSU Ori3 |  |  | ALL Bands | LTE Bands |
| Sample_1 | -94.73 | -94.98 | -92.36 | -94.18 | 0 | 20 | 5H 6L |
| Sample_2 | -95.49 | -93.14 | -92.27 | -93.86 | 0 | 21 | 8H 5L |
| Sample_9 | -96.39 | -94.18 | -94.21 | -95.06 | 0 | 14 | 3H 3L |
| Sample_12 | -97.70 | -98.09 | -93.95 | -96.94 | 0 | 21 | 8H 5L |
| Sample_15 | -95.83 | -94.34 | -92.94 | -94.53 | 0 | 32 | 13H 12L |

Analysis of results

With using the worksheet [4], CDF value is calculated. Table 4 shows the summary of the CDF value. Band 19 data is also listed for comparison. The difference between band 5 and 19 are between 0.9dB to 1.4dB.

| DUT (dummy) | Band 5 |  |  | Band 19 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 80% value of CDF | 85% value of CDF | 90% value of CDF | 80% value of CDF | 85% value of CDF | 90% value of CDF |
| 70% limit | -94.8 | -94.5 | -94.1 | -96.1 | -95.9 | -95.5 |
| 95% limit | -92.7 | -92.6 | -92.5 | -94.1 | -93.6 | -93.4 |

Conclusion

In this contribution, evaluation of 5-samples for band 5 is provided. These results are helpful to define  MIMO OTA requirement for low band.

References

[1]    TR 37.977, V14.3.0, “Verification of radiated multi-antenna reception performance of UEs,” 3GPP RAN #75, March 2017

[2]    R4-1706401, Measurement results for lab alignment test activity – Re-tested at SGS, NTT DOCOMO INC., SGS Wireless, MVG Industries, RAN4#MIMO OTA ad-hoc

[3]    R4-1704529, MIMO OTA measurement results for band 1 and 19, NTT DOCOMO, SGS Wireless, RAN4#83

[4]     R4-1709061, TRMS joint band passing rate worksheet, Intel,