3GPP TSG-RAN WG4 RAN#84bis	R4-1711388

Dubrovnik, Croatia, October 9th-13st, 2017

Agenda item:	 9.7.11.1

Source: 	Qualcomm Incorporated

Title: 	PBCH Performance: link level simulation results

Document for:	Discussion

# 1. 	Introduction

Following the revised NR link level simulation assumptions [1] for PBCH reading performance and PBCH-DMRS time index reading performance, in this paper we present corresponding simulation results for sub6 and mmW scenarios and channel models.

# 2. 	Assumptions

PBCH detection performance simulations have been performed according to the assumptions in [1] for these particular cases:

- 1 TX antenna with cross-polarization and 2 RX antennas with ideal receiver beamforming (Rx beam is pointing towards the Rx signal and no beam sweep is simulated)

- PBCH payload incl. 16-bit CRC: 56 bits

- Carrier Frequency 30 GHz: channel models AWGN, CDL-A, CDL-B, for CDL-A/B: UE velocity of 3 km/h

- Carrier Frequency 4 GHz: channel models AWGN, EPA5, ETU30

- Single transmitted SS block per SS burst set (K=1)

- One-shot detection: no combining of different PBCHs

- Power offset of PBCH w.r.t. PSS/SSS: 0 dB

Results are reported for Case 1: PBCH reading, as well as for Case 2: PBCH-DMRS time index reading (without decoding PBCH data). For case 1, the basic SI reading time is defined as the number of SS-blocks required for 99% to successfully decode the PBCH. For Case 2, DMRS time index reading time is defined as the number of SS-blocks required for 99% to successfully reading DMRS time index.

# 3.		Case 1: PBCH SI Reading Performance

# 3.1 Results for 30 GHz

It can be observed that max. 7 independent decoding attempts are required to guarantee 99% detection probability at - 8 dB SNR.

Table 2. Basic SI reading time at 30 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | CDL-A |  | CDL-B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 4 | 4 | 3 | 4 |
| -8 | 3 | 3 | 7 | 6 | 7 | 7 |
| -10 | 11 | 11 | 15 | 12 | 20 | 23 |

3.2 Results for 4 GHz

It can be observed that max. 5 independent decoding attempts are required to guarantee 99% detection probability at - 8 dB SNR.

Table 3. Basic SI reading time at 4 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | EPA5 |  | ETU30 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 2 | 1 | 2 | 2 | 1 | 1 |
| -8 | 3 | 3 | 5 | 4 | 1 | 1 |
| -10 | 14 | 11 | 8 | 6 | 2 | 2 |

# 4.	Case 2: DMRS Time Index Detection Performance

# 4.1 Results for 30 GHz

It can be observed that max. XX independent detection attempts are required to guarantee 99% detection probability at - 8 dB SNR.

Table 2. DMRS time index reading time at 30 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | CDL-A |  | CDL-B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 120 KHz | 240 KHz | 120 KHz | 240 KHz | 120 KHz | 240 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 2 | 1 | 1 | 1 |
| -8 | 1 | 1 | 2 | 2 | 2 | 1 |
| -10 | 1 | 1 | 2 | 2 | 3 | 2 |

4.2 Results for 4 GHz

It can be observed that a single detection attempt is required to guarantee 99% detection probability at - 8 dB SNR.

Table 3. DMRS time index reading time at 4 GHz, in number of SS burst set periods

| SNR [dB] | AWGN |  | EPA5 |  | ETU30 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 KHz | 30 KHz | 15 KHz | 30 KHz | 15 KHz | 30 KHz |
|  | 99% | 99% | 99% | 99% | 99% | 99% |
| -6 | 1 | 1 | 1 | 1 | 1 | 1 |
| -8 | 1 | 1 | 1 | 1 | 1 | 1 |
| -10 | 1 | 1 | 1 | 1 | 1 | 1 |

# 5. 	Conclusions

In this paper, we provided simulation results for PBCH reading and PBCH-DMRS time index reading in NR bands, for a number of parameter combinations according to the simulation assumptions agreed in [1]. The results should be taken into account when RAN4 discusses the requirements.

# 6. 	References

- R4-1709908, “Updated PBCH Simulation assumption”, Huawei, HiSilicon