3GPP TSG RAN WG4 Meeting #84bis	R4-1711453

Dubrovnik, Croatia, October 9 – 13, 2017

Agenda Item:	8.20.4.1

Source: 	Ericsson

Title:	On measurements accuracy when using NSSS and NRS

Document for:	Discussion

# Introduction

During the RAN4#84 meeting simulation assumptions for NB-IoT mobility measurements conducted on NSSS were agreed [1].

In this contribution we are providing simulation results for measurement accuracy of received power estimates based on NSSS.

# Simulation assumptions

The following subset of simulation parameters has been used. NOTE 1: Every NSSS occasion has been utilized, hence the Measurement sampling rate is 20ms. NOTE 2: Only NSSS-based measurements have been conducted, since combination of NSSS and NRS either will destroy the statistical properties achievable when using NSSS, or will impose restrictions on physical antennas used for transmissions of NSSS and NRSs, respectively (see [2]). NOTE 3: Since no combination of NSSS and NRS has been simulated, NSSS and NRS power imbalance of 0dB has been used.

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 1 resource blocks | Both RSRP and RSRQ measured over 1 RB |
| System bandwidth | 1 resource blocks |  |
| L1 measurement period | 200ms, 400ms, 800 ms, 1600ms |  |
| Measurement sampling rate | 1 sample/20ms (NOTE 1) | Implementation dependent |
| L3 filtering | Disabled |  |
| Antenna configuration | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern Diversity scheme should be indicated in the simulation. |
| Power imbalance between NSSS and NRS | 0 dB (NOTE 3) | Only applicable for measurement based on combination of NSSS and NRS |
| Mobility | Stationary UEs |  |
| Channel model | AWGN, ETU 1Hz, EPA 1Hz | Models stationary device |
| Measurement type | Synchronization signals only based (NSSS) (NOTE 2) | Companies are encouraged to provide simulation results for measurements based on follows signals. |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15dB, -12.6dB, -6dB | AWGN noise |
| Frequency error modelling | +/-50 Hz | With respect to reference cell |

# Simulation results

Simulation results for each of the propagation conditions and L1 measurement period (L1p) are provided in the following sections, with results for specified SINR points additionally captured in tables.

## AWGN

Figure 1: Measurement accuracy for NSSS-based received power estimates in AWGN propagation condition

Table 1: Measurement accuracy for NSSS-based received power estimates in AWGN propagation condition

| SINR [dB] | L1p [ms] | 5th pt [dB] | 50th pt [dB] | 95th pt [dB] |
| --- | --- | --- | --- | --- |
| -15 | 200 | -0.42 | 1.12 | 2.40 |
|  | 400 | 0.13 | 1.13 | 2.06 |
|  | 800 | 0.45 | 1.16 | 1.78 |
|  | 1600 | 0.66 | 1.15 | 1.60 |
| -12.6 | 200 | -0.58 | 0.66 | 1.72 |
|  | 400 | -0.14 | 0.67 | 1.44 |
|  | 800 | 0.11 | 0.69 | 1.20 |
|  | 1600 | 0.29 | 0.68 | 1.05 |
| -6 | 200 | -0.50 | 0.11 | 0.67 |
|  | 400 | -0.30 | 0.11 | 0.51 |
|  | 800 | -0.17 | 0.12 | 0.39 |
|  | 1600 | -0.08 | 0.11 | 0.30 |

## ETU 1Hz

Figure 2: Measurement accuracy for NSSS-based received power estimates in ETU 1Hz propagation condition

Table 2: Measurement accuracy for NSSS-based received power estimates in ETU 1Hz propagation condition

| SINR [dB] | L1p [ms] | 5th pt [dB] | 50th pt [dB] | 95th pt [dB] |
| --- | --- | --- | --- | --- |
| -15 | 200 | -4.70 | -0.06 | 4.82 |
|  | 400 | -3.72 | 0.17 | 4.34 |
|  | 800 | -2.94 | 0.47 | 3.75 |
|  | 1600 | -2.07 | 0.68 | 3.34 |
| -12.6 | 200 | -6.11 | -0.69 | 4.55 |
|  | 400 | -5.00 | -0.44 | 4.10 |
|  | 800 | -3.96 | -0.10 | 3.48 |
|  | 1600 | -3.02 | 0.12 | 3.01 |
| -6 | 200 | -8.96 | -1.46 | 4.21 |
|  | 400 | -7.15 | -1.18 | 3.82 |
|  | 800 | -5.44 | -0.80 | 3.14 |
|  | 1600 | -4.31 | -0.59 | 2.61 |

## EPA 1Hz

Figure 3: Measurement accuracy for NSSS-based received power estimates in EPA 1Hz propagation condition

Table 3: Measurement accuracy for NSSS-based received power estimates in EPA 1Hz propagation condition

| SINR [dB] | L1p [ms] | 5th pt [dB] | 50th pt [dB] | 95th pt [dB] |
| --- | --- | --- | --- | --- |
| -15 | 200 | -4.77 | -0.21 | 4.90 |
|  | 400 | -3.81 | 0.10 | 4.44 |
|  | 800 | -2.90 | 0.38 | 4.01 |
|  | 1600 | -1.94 | 0.61 | 3.36 |
| -12.6 | 200 | -6.20 | -0.85 | 4.70 |
|  | 400 | -5.18 | -0.52 | 4.24 |
|  | 800 | -4.08 | -0.16 | 3.83 |
|  | 1600 | -2.93 | 0.08 | 3.08 |
| -6 | 200 | -8.98 | -1.62 | 4.51 |
|  | 400 | -7.46 | -1.20 | 4.05 |
|  | 800 | -5.97 | -0.74 | 3.65 |
|  | 1600 | -4.34 | -0.49 | 2.81 |

# Summary and Conclusion

In this contribution we have provided simulation results for received power measurement accuracy for various propagation conditions and L1 measurement periods. It shall be noted that although a short L1 measurement period might be desirable, the fading channels (ETU 1Hz and EPA 1Hz) are slowly fading and hence from that point of view a longer L1 measurement period (but potentially sparser sampling) is desirable in order to capture the average conditions.

# References

- R4-1709045, “Simulation assumptions for measurement accuracy improvements”, Huawei, HiSilicon

- R4-1708317, “On measurements accuracy when using NSSS and NRS”, Ericsson