3GPP TSG-RAN WG4 84Bis	R4-1711576

Dubrovnik, Croatia, 9 – 13 October, 2017

# Source:	Intel Corporation

# Title:	On further discussion in REFSENS for Sub-6GHz

# Agenda Item:	9.4.4.1.1

# Document for:	Approval

# 1	Introduction

RAN4 discussed UE RF requirements for n77, n78, and n79 in RAN4-NR#3 (Nagoya) and WF was agreed [1]. Regarding REFSENS, one issue that needs to be confirmed is whether there is 0.5 dB difference between TDD and FDD mode:

- REFSENS - Filter data satisfying the following conditions will be considered for insertion loss and REFSENS - Attenuation is at least typ. 35 dB for ≤ 2.69 GHz and ≥ 5.15 GHz (35 dB is required for both B46 and 5 GHz WiFi as a consensus in RAN4) - It will be confirmed that IM of TDD bands is 0.5 dB better than that of FDD ones - SNR is being discussed in other documents

This contribution shares our view.

# 2	Discussion

First of all, the origination of the difference 0.5 dB difference comes from existence of a front-end filter. In FDD, a duplexer is required to separate Tx and Rx band each other, while no duplexer is required in TDD. In [2], basic assumption is SNR and IM will take care of all baseband impairments, it should be understood that this kind of impairment, i.e. RF front-end loss, should be included in RF margin which is included in NF budget.

Observation 1: NF difference between TDD and FDD comes from the insertion loss difference of RF front-end filters.

Proposal 1: All RF front-end losses or impairments should be included in NF budget.

Observation 2: 0.5 dB difference front-end filter loss difference was observed between n77 and n78 range.

# 3	Conclusions

In this contribution, we made the following proposals:

Observation 1: NF difference between TDD and FDD comes from the insertion loss difference of RF front-end filters.

Proposal 1: If there is any NF difference originated from RF front-end, it should be included in NF budget.

Observation 2: 0.5 dB difference front-end filter loss difference was observed between n77 and n78 range.

# 4	Reference

- R4-1710031, “WF on Band n77, n78, and n79”, NTT Docomo, RAN4-NR#3, Sept. 2017