3GPP TSG-RAN WG4 Meeting #85                                                             R4-170XXXX

Reno, USA, 27 November - 1 December, 2017

Agenda item:	9.3.1

Source: 	China Unicom, Deutsche Telekom

Title: 	New UE mandatory channel bandwidth for NR band3

Document for:	Approval

Introduction

A WF on band specific channel bandwidth for NR was proposed in RAN4 #NR-AH2 [1]. In this paper, we propose to add a new channel bandwidth for NR band 3 due to the channel allocation of China Unicom (UL:1735-1765 MHz, DL: 1830-1860 MHz).

Discussion

We propose to add a new channel bandwidth of 30MHz for NR Band 3 as UE mandatory channel bandwidth in case of SCS = 15 kHz, 30 kHz, since 60kHz SCS is optional for 1-6GHz bands. All channel bandwidths for band 3 are shown below.

Table 1 channel bandwidths for NR band 3

| NR Band | Data SCS = 15kHz |  |  |  |  |  |  |  | Data SCS = 30kHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | [40MHz] | [50MHz] | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | [40MHz] | [50MHz] | [60MHz] |
| Band 3 | Yes | Yes | Yes | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |
| NOTE: 90% spectrum utilization may not be achieved |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: in case of SCS=15 kHz, 30 kHz, a new UE CBW 30MHz is added for NR Band3 as mandatory. In case of 60 kHz, a new CBW 30MHz is added for NR Band3 as optional.

Conclusions

In this paper, we have the following proposal:

Proposal 1: in case of SCS=15 kHz, 30 kHz, a new UE CBW 30MHz is added for NR Band3 as mandatory. In case of 60 kHz, a new CBW 30MHz is added for NR Band3 as optional.

Reference

- R4-1706982, “WF on band specific UE channel bandwidth”, NTT DOCOMO