3GPP TSG-RAN WG4 Meeting 85	    R4-1712245

Reno, Nevada, USA, 27 Nov. – 1 Dec., 2017

Agenda Item:	9.7.10.1

Source: 	Samsung

Title: 	Updated cell detection evaluation results

Document for:	Discussion

# Introduction

In RAN4 84bis, the simulation results of PSS/SSS detection were summarized in [1]. However, in RAN1 90bis meeting, the PBCH format is changed from 24 PRBs to 20 PRBs, and 8 PRBs of PBCH are padded in the SSS symbol [2]. This could have some impact on the cell detection results since the interference and noise level are slightly different. So in this paper, we update the simulation results based on the new PBCH format, and also make a comparison with the previous simulation results.

# Simulation Assumptions

Figure 2.1 New PBCH format.

The detailed simulation assumptions are listed in Table A.1~4 in Annex A. The new PBCH format is shown in Figure 2.1, where PBCH occupies 240 instead of 288 subcarriers. Consequently, the FFT size is set to be 256. For PSS detection, time-domain correlation is performed, with false alarm rate set to be 0.01%. For SSS detection, correlation is done in the frequency domain, with false alarm rate 0.01%. Besides, correlation is done with received signals from 2 receive antennas seperately, then, the maximum correlation value among the two is selected.

# Simulation results

We first present cell detection simulation results with the new PBCH format (20 RB) for SCS 15 kHz, the results with the old PBCH format (24 RB) is also presented for comparison.

Table 3.1: 90%-tile cell detection delay (number of SS burst periodicities), SINR -6 dB, SCS 15 kHz.

| PBCH Channel | Case 1 |  | Case 2 |  | Case 3 |  | Case 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 RB | 24 RB | 20 RB | 24 RB | 20 RB | 24 RB | 20 RB | 24 RB |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 3 | 3 | 3 | 3 | 3 | 2 | 3 | 2 |
| ETU30 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| TDL-A | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 |
| TDL-B | 3 | 2 | 3 | 2 | 3 | 2 | 3 | 2 |
| TDL-C | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.2: One shot detection probability, SINR -6 dB, SCS 15 kHz.

| PBCH Channel | Case 1 |  | Case 2 |  | Case 3 |  | Case 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 RB | 24 RB | 20 RB | 24 RB | 20 RB | 24 RB | 20 RB | 24 RB |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| EPA5 | 86% | 87.5% | 87.4% | 87% | 86.7% | 87.3% | 86.9% | 87.5% |
| ETU30 | 82.5% | 85.1% | 82.9% | 85.6% | 83.4% | 85.7% | 83.9% | 85.2% |
| TDL-A | 86.8% | 86.9% | 86.7% | 86.6% | 86.9% | 87.4% | 86.7% | 87.8% |
| TDL-B | 86.3% | 87.6% | 86.6% | 87.6% | 86.1% | 88% | 86.7% | 87.6% |
| TDL-C | 86% | 86.7% | 87.3% | 86.2% | 84.8% | 86.1% | 84.9% | 86.2% |
| TDL-D | 99.7% | 99.2% | 98.8% | 99.9% | 92.4% | 98.3% | 99.2% | 98.2% |

Table 3.1, Table 3.2 are the cell detection delay and one shot detection probability, respectively, under SINR -6 dB, SCS 15 kHz. In comparison with our previous results [3], the detection performance degrades, the reason is that we reduce the false alarm rate for SSS from 1% to 0.01% to further reduce the detection complexity. By comparing the detection performance under new PBCH and old PBCH formats, it is seen that the difference is marginal, hence, results in previous meetings can be considered for deciding the cell detection delay requirement. Similar results are seen for 30 kHz, 120 kHz, 240 kHz SCSs as well. To summarize, we have the following observation:

Observation 1: The impact of new PBCH format on cell detection results is limited.

Table 3.3: 90%-tile cell detection delay (number of SS burst periodicities), SINR -6 dB.

| SCS (kHz) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 120 | 240 | 30 | 120 | 240 | 30 | 120 | 240 | 30 | 120 | 240 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 3 | 2 | 3 | 2 | 2 | 3 | 3 | 2 | 3 | 2 | 2 | 3 |
| ETU30 | 2 | - | - | 2 | - | - | 2 | - | - | 2 | - | - |
| TDL-A | 3 | 2 | 2 | 3 | 2 | 2 | 3 | 2 | 2 | 3 | 2 | 2 |
| TDL-B | 3 | 2 | 2 | 3 | 2 | 2 | 3 | 2 | 2 | 3 | 2 | 2 |
| TDL-C | 3 | 2 | 2 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.4: One shot detection probability, SINR -6 dB.

| SCS (kHz) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 120 | 240 | 30 | 120 | 240 | 30 | 120 | 240 | 30 | 120 | 240 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| EPA5 | 86.7% | 88.7% | 82.1% | 87.8% | 88.7% | 83% | 86.2% | 87.3% | 82.8% | 86.6% | 88.1% | 83.1% |
| ETU30 | 79.5% | - | - | 80.8% | - | - | 79.8% | - | - | 80.3% | - | - |
| TDL-A | 84.7% | 87.0% | 82.6% | 84.9% | 87.5% | 83% | 85.2% | 86% | 82.6% | 85.7% | 86.1% | 82.4% |
| TDL-B | 84.7% | 85.8% | 84.4% | 85.5% | 86.1% | 85.1% | 84.9% | 86.2% | 84.5% | 85.2% | 86.1% | 84.6% |
| TDL-C | 85.2% | 85.9% | 86% | 86.1% | 86.7% | 86.9% | 86.2% | 86.1% | 86.2% | 86.3% | 86.4% | 86.5% |
| TDL-D | 99.6% | 99.7% | 99.8% | 98.8% | 99% | 99.2% | 92% | 92.1% | 92.5% | 99.2% | 99.3% | 99.4% |

Table 3.3, Table 3.4 present the cell detection delay and one shot detection probability, respectively, with the new PBCH format for SCS 30 kHz, 120 kHz, and 240 kHz. It is seen that under SINR -6 dB, 3 shots are enough to achieve the 90%-tile cell detection probability across all simulated scenarios.

Observation 2: With -6 dB SINR, 3 shots are necessary to achieve 90%-tile cell detection probability for all scenarios.

# Conclusion

In this paper, we updated the link level simulation results for PSS/SSS detection based on the new PBCH format. By comparing the results for all scenarions, we have the following observations:

Observation 1: The impact of new PBCH format on cell detection results is limited.

Observation 2: With -6 dB SINR, 3 shots are necessary to achieve 90%-tile cell detection probability for all scenarios.

# Reference

- R4-1711691, Summary of link level simulation result of PSS/SSS detection and PBCH, Huawei, HiSilicon

- Chairman’s notes RAN1 90 bis final

- R4-1709877, Cell detection evaluation-update, Samsung

- R4-1708698, Updated link level simulation assumption of cell detection in NR, Huawei, HiSilicon

# Annex A: Simulation Assumptions [4]

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25; FSS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Es/Noc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 CDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 (secondary priority) |  |
| Noc Model | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated (except for CDL model) Option 1: With receiver beamforming Option 2: Without receiver beamforming) |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |