TSG-RAN Working Group 4 (Radio) 85	R4-1712264

Reno, USA, 27 Nov - 1 Dec, 2017

Source:	Samsung

Title:	Discussion on NR UE EVM

Agenda item:	9.4.3.2

Document for:	Discussion

# Background

In RAN4 #84bis, NR UE EVM measurement is discussed in [1]. There are several issues covered:

- Tx LO leakage

- Common phase error (CPE) compensation for EVM measurement

- EVM window size

In this paper, we share our views on these issues.

# Tx LO leakage

In LTE, Tx LO leakage is removed in RF compensation block before FFT. The measured EVM value excludes Tx LO leakage impact. The measured LO leakage value will be used in IBE test. On the other hand, if Tx LO leakage is included in measured EVM value, the amount of LO leakage does not change very much with UE output power. Thus, for low output power, UE is challenging to meet the EVM requirement, if LO leakage is included. Therefore, we have the following proposal

Proposal 1: For NR, LO leakage needs to be removed before measuring EVM value, and measured LO leakage is included for IBE measurement.

There are two possible ways to remove LO leakage: Option 1: in RF compensation block before FFT (similar to LTE); Option 2: exclude a dedicated subcarrier after FFT, e.g., DC subcarrier. To compare between these two options, we need to look at LO position for NR. In 2017 Jan RAN1 AdHoc meeting, the following agreements have been reached in RAN1[2].

- Handling of transmitter DC subcarrier at the transmitter

  - DL

    - UE may assume transmit DC subcarrier at the transmitter (gNB) side is modulated i.e., data is neither rate-matched nor punctured.

      - Signal quality requirement (e.g., EVM) corresponding to DC subcarriers is up to RAN4.

  - UL

    - Transmit DC subcarrier at the transmitter (UE) side is modulated i.e., data is neither rate-matched nor punctured.

      - Signal quality requirement (e.g., EVM) corresponding to DC subcarriers is up to RAN4.

    - The transmitter DC subcarrier at the transmitter (UE) side should avoid collisions at least with DMRS if possible

    - The specification should define at least one particular subcarrier as the candidate position of DC subcarrier, e.g., DC subcarrier is located at the boundary of PRBs

    - This should be considered in the RS design for NR

    - Specify means for the receiver to determine DC subcarrier location

      - This involves semi-static signalling from UE and also standard specified DC subcarrier location

      - FFS how to determine and how to indicate DC subcarrier location in the case of bandwidth adaptation

- Note that above DC subcarrier can be interpreted as DC subcarrier candidate

We could see that for UL, RAN1 encourages receiver to demodulate DC subcarrier, and receiver shall assume that transmitter modulates symbol on DC subcarrier. In that sense, excluding DC subcarrier from EVM measurement does not follow RAN1 conclusion.

So far, there is no explicit LO position defined in NR. For LTE refarming band with UL sharing, LO position may overlap with LTE LO position in the middle of subcarrier grid. In addition, another related issue is RB grid alignment among multiple numerologies. No conclusion has been reached for RB grid alignment [3]. To illustrate the issue,  Proposal 1 from [4] is copied as follow in Table 1. We could see that there is a frequency offset for higher numerology compared to the lowest numerology. And the offset could be none multiples of subcarriers of corresponding numerology. If we place UE Tx LO at the center of transmission BW of the lowest numerology, it is likely LO position is not at the integer multiple subcarriers for higher numerology. For aforementioned cases, it is impossible to remove LO leakage after FFT by notching a dedicated subcarrier.

Observation 1: Removing LO leakage by notching a dedicated subcarrier after FFT does not follow RAN1 agreement and depends on RB grid alignment discussion.

Table 1. Offset of the center frequency of the transmission bandwidth configuration for worst case guard band (100 kHz channel raster) [4]

On the other hand, removing LO leakage before FFT in RF compensation block can be based on hypothesis of LO positions and values. It could potentially hurt data if LO position is overlapping with a subcarrier. The degradation can be relaxed by using longer observation window e.g., multiple slots, to measure LO leakage.

It is our understanding that how to remove LO leakage in EVM measurement is implementation specific to test box vendors.

# CPE compensation

In FR2, phase noise could be a large contributor to EVM for higher than 64QAM modulation. RAN1 introduces PTRS to remove CPE part of phase noise. CPE is compensated for each OFDM symbol. We did a simulation comparison on EVM impact by using PTRS compensation. The settings are summarized as follows

- -24dBc PN profile [5].

- 64QAM, 200MHz CHBW full allocation.

- The same PTRS density between OFDM and DFT-s-OFDM. A total of 16 PTRS per symbol. For DFT-s-OFDM, 16PTRS are evenly distributed in four chunks in time-domain (RAN1 agreed that PTRS is pre-DFT inserted for DFT-s-OFDM). For OFDM, 16PTRS are assigned as 1PTRS to each RB and distributed evenly in edge RBs.

- Results based on 1000 random waveform generations.

Table 2. EVM performance with and without CPE compensation using PTRS

| EVM [dB] | no CPE comp | with per-symbol CPE comp | Gain [dB] |
| --- | --- | --- | --- |
| OFDM | 25.1(0.9) | 26.3(0.7) | 1.2 |
| DFT-s-OFDM | 25.2(1.1) | 25.9(1.1) | 0.7 |

We could see without PTRS CPE compensation, EVM is around 25dB. Channel estimation (CE) is able to capture part of CPE, but since CE is per-slot based, gain is limited. CPE compensation based on PTRS for each symbol can further improve EVM for 1dB. Note that 64QAM EVM requirement is around 22dB, it is very critical to adopt per-symbol wise CPE compensation using PTRS to improve measured EVM.

Proposal 2: For NR FR2, Per-symbol CPE compensation is applied for EVM measurement, if PTRS is present

There are several ways to perform per-symbol CPE compensation. CPE compensation can be done in RF compensation block before FFT based on hypothesis. We provide one example of CPE compensation after FFT as shown in Figure 1. Since PTRS is inserted pre-DFT for DFT-s-OFDM, it is natural to do the compensation after IDFT.

Figure 1. CPE compensation using PTRS for DFT-s-OFDM. New CPE compensation blocks are highlighted in red.

The frequency domain equalization function EC(f) can still be based on average of symbols in one slot since CPE is  removed by MSf,texp-jϕ(t).

ECf=symbs∈slot NSf,t*NSf,tsymbs∈slot NSf,t*MSf,texp-jϕ(t)

where MSf,t and NSf,t denote measured symbol and nominal symbol, respectively, on Frequency f and Symbol t. ϕ(t) denotes estimated CPE for symbol t.

# Conclusions

Proposal 1: For NR, LO leakage value needs to be excluded from measured EVM value, and included for IBE measurement.

Observation 1: Removing LO leakage by notching a dedicated subcarrier after FFT does not follow RAN1 agreement and depends on RB grid alignment discussion.

Proposal 2: For NR FR2, Per-symbol CPE compensation is applied for EVM measurement, if PTRS is present

# References

[1]	R4-1711395, “On UE EVM for NR”, Qualcomm Incorporated

[2]	R1-1701553, ”Final Report of 3GPP TSG RAN WG1 #AH1_NR v1.0.0”

[3]	R4-1711736, “WF on NR spectrum utilization”, ZTE

[4]	R4-1708606, “NR transmission bandwidth configuration for MPR evaluation”, Qualcomm Incorporated

[5]	R4-1703528, “Phase noise model for above 6 GHz”, Huawei, Hisilicon