3GPP TSG-RAN WG4 Meeting#85                                   R4-1712267

Reno, US, 27 Nov - 01 Dec, 2017

Agenda Item:   9.3.2.2

Source:        Samsung

Title:          Discussion on NR sync raster for initial cell search

Document for:  Discussion

Introduction

As agreed in [1] the channel raster agreement are summarized as below

- Below 2.4GHz band: 100KHz

- Above 2.6GHz band including band n41: SCS based(15KHz)

- MMW band: SCS based (60KHz)

Regarding the use case of channel raster in NR phase our current understanding is that channel raster is at least used as the minimum granularity of shift/offset to the lower edge of specific NR bands that operator can allocate to their own spectrum (system bandwidth). Furthermore, according to discussion in RAN4#84bis introduction of ARFCN according channel raster would be needed but corresponding specification is still open.

With above agreement and common understanding of channel raster companies provide proposal on sync raster based on below agreement in [1]

- Sync raster will be defined such that there is a minimum number of entries for each band

- Sync raster entries will be included in the specifications for each band

- Sync raster entries will be defined for initial system acquisition, sync blocks can be transmitted in other frequency locations if the position is signaled to the UE

Besides that, floating sync mechanism is introduced to resolve the problem of multiple sync entries due to ensure PRB alignment between SSB and data. However, it should be clarified that it could not save number of sync entries if too fine sync raster (compared with 100 kHz) is assumed. In the other word, this approach would reduce the number of sync entries only for the case with sparse sync raster.

With above agreement, the rationale to deduce sync raster is shown roughly as below:

- First SSB in one band(offset to the lower edge): [TMBW of minimum bandwidth]-[PBCH bandwidth]+ [guard band of minimum channel bandwidth]

- Sync raster for following SSB: [TMBW of minimum bandwidth]-[PBCH bandwidth]+ [channel raster]

It should be noted that the sync raster derived by this approach would be the maximum one based on agreed SU of minimum channel bandwidth, PBCH size and channel raster. This would be the maximum value mathematically and theoretically. Hence smaller raster would be still valid if it is selected on integer multiple of channel raster and RE size. Furthermore, considering Network reliability and forward compatibility it is proposed to reserve room when decide key system parameter such as sync raster.

Based on above agreement, assumption and consideration, further discussion on sync channel raster for initial cell search is provided in this contribution.

Discussion

In this contribution, the NR bands are categorized as below based on calculation summarized in table 1 and 2. And no SDL and SUL bands are considered here.

- FR1 below 2.4GHz normal case:

This band class includes NR band n1, n2, n3, n8, n20, n28, n50, n51, n70, n71 and n74 with system parameters of 5MHz minimum channel bandwidth, 15 kHz default SSB SCS and 100 kHz channel raster. Based on the equation aforementioned in introduction, the theoretical maximum sync raster would be 1MHz. Then there would be two alternatives for sync raster for this class:

- Alt 1: 1MHz sync raster with 2 steps repetition with 100 kHz channel raster to keep the RE alignment

- Alt 2: 100 kHz sync raster same as LTE which will simplify the specification but the sync entry# will be 3 times of Alt1.

Observation 1: two alternatives could be considered for sync raster of NR band n1, n2, n3, n8, n20, n28, n50, n51, n70, n71 and n74.

- FR1 below 2.4GHz special case:

This band class includes NR band n5 and n66 which allow 15KH default SSB SCS for 5MHz minimum bandwidth, 30 kHz default SSB SCS for 10MHz channel bandwidth and 100 kHz channel raster. Based on the equation aforementioned in introduction, the theoretical maximum sync raster would be 1MHz for 15 kHz SCS or 1.54MHz for 30 kHz SCS case. Then there would be three options for sync raster for this class:

- Option1: decide the sync raster independently for each default SCS case based on calculation.

- Option2: fix the first SSB location with offset to band lower edge in the range of [0.32, 1.12], and decide sync raster based on 1MHz. The same sync entry pattern could be applied to both default SCSs case with almost half sync entry# of option 1.

- Option3: 100KHz sync raster same as LTE which will simplify the specification with less than 2 times sync entry# of option 1

Observation 2: 3 options can be candidates for sync raster of band n5 and n66.

- FR1 2.4GHz bands

This band class includes NR band n7 and n38 with system parameters of 5MHz minimum channel bandwidth and 15 KHz default SSB SCS. The remaining issue is the channel raster for these two bands are not explicitly decided in RAN4. Considering that these two bands are in the same frequency range of band n41, in our calculation in table 1, 15 kHz channel raster is assumed, which results in 0.915MHz maximum sync raster.

Observation 3: the channel raster for band n7 and n38 is not explicitly decided yet.

- FR1 above 2.6GHz bands

This band class includes NR band n41, n77, n78 and n79 with 15KHz channel raster and at least 10MHz minimum channel bandwidth. As shown in table 1, further discussion on default SSB SCS is still needed for band n41.

Observation 4: Further discussion on default SSB SCS is still needed for band n41.

- FR2 bands

FR2 includes NR band n257, n258 and n260 currently with 120 kHz default SSB SCS for 50MHz minimum bandwidth, 240 kHz default SSB SCS for 100MHz channel bandwidth and 60KHz channel raster. The calculation for this frequency range is shown in table 2. The criteria of option 2 for band n5 and n66 could be considered for band n257, n258 and n260. The possible candidate could be as below:

- Reuse the same sync entry location for both 120KHz and 240KHz SSB SCS

- Sync raster : [16.8MHz]

- First sync entry location offset to band lower edge: [18.76MHz]

Observation 5: reuse the sync entry locations for 120 kHz and 240 kHz could be candidate solution to resolve multiple default SSB SCS for FR2.

Conclusion

In this contribution we further analyze the sync raster as have below observations:

Observation 1: two alternatives could be considered for sync raster of NR band n1, n2, n3, n8, n20, n28, n50, n51, n70, n71 and n74.

Observation 2: 3 options can be candidates for sync raster of band n5 and n66.

Observation 3: the channel raster for band n7 and n38 is not explicitly decided yet.

Observation 4: Further discussion on default SSB SCS is still needed for band n41.

Observation 5: reuse the sync entry locations for 120 kHz and 240 kHz could be candidate solution to resolve multiple default SSB SCS for FR2.

One additional issue needs to be considered is in the decision of sync raster is that the Network reliability and forward compatibility. All the roughly calculation shown in annex is just based on theoretical maximum sync raster but not take into account this aspect.

Reference

[1] R4-1711975, WF on Channel Raster for NR

[2] R4-1708850, WF on Channel Raster for NR

[3] R4-1711817, Way Forward on Minimum CHBW and SCS for SS/PBCH

[4] R1-1719165, Reply LS on NR minimum carrier bandwidth and SS block numerology

[5] R4-1709075, Way forward on spectrum utilization

Annex

Table 1:  NR bands in FR1.

| Band number | DL | Minimum channel BW(MHz) | Default SCS(kHz) | Channel raster | Maximum Sync raster | Roughly sync entry# |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 2110 – 2170 MHz | 5 | 15 | 100kHz | 1MHz | Alt1: 171 |
|  |  |  |  |  |  | Alt2:550 |
| n2 | 1930 – 1990 MHz | 5 | 15 | 100kHz | 1MHz | Alt1: 171 |
|  |  |  |  |  |  | Alt2:550 |
| n3 | 1805 – 1880 MHz | 5 | 15 | 100kHz | 1MHz | Alt1: 171 |
|  |  |  |  |  |  | Alt2:550 |
| n5 | 869 – 894MHz | 5 | 15 | 100kHz | 1MHz | Option1: 105 Option2: 66 Option3: 200 |
|  |  | 10 | 30 |  | 1.54MHz |  |
| n7 | 2620 – 2690 MHz | 5 | 15 | 15KHz | 0.915MHz | 73 |
| n8 | 925 – 960 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:96 |
|  |  |  |  |  |  | Alt2:300 |
| n20 | 791– 821MHz | 5 | 15 | 100kHz | 1MHz | Alt1:81 |
|  |  |  |  |  |  | Alt2:250 |
| n28 | 758 – 803 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:126 |
|  |  |  |  |  |  | Alt2:400 |
| n38 | 2570 – 2620 MHz | 5 | 15 | 15KHz | 0.915MHz | 51 |
| n41 | 2496 – 2690 MHz | 10 | 15KHz | 15KHz | 5.775MHz | 33 |
|  |  |  | 30KHz |  | 1.455MHz | 130 |
| n50 | 1432 – 1517 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:246 |
|  |  |  |  |  |  | Alt2:800 |
| n51 | 1427 – 1432 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:1 |
|  |  |  |  |  |  | Alt2:1 |
| n66 | 2110 – 2200 MHz | 5 | 15 | 100kHz | 1MHz | Option1:318 Option2:201 Option3:650 |
|  |  | 10 | 30 |  | 1.54MHz |  |
| n70 | 1995– 2020 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:66 |
|  |  |  |  |  |  | Alt2:200 |
| n71 | 617 – 652 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:96 |
|  |  |  |  |  |  | Alt2:300 |
| n74 | 1475 – 1518 MHz | 5 | 15 | 100kHz | 1MHz | Alt1:120 |
|  |  |  |  |  |  | Alt2:380 |
| n77 | 3.3 – 4.2 GHz | 10 | 30 | 15KHz | 1.455MHz | 614 |
| n78 | 3.3 – 3.8 GHz | 10 | 30 | 15KHz | 1.455MHz | 339 |
| n79 | 4.4 – 5.0 GHz | 40 | 30 | 15KHz | 30.975MHz | 18 |

Table 2: New NR bands in FR2.

| Band number | DL | Minimum channel BW(MHz) | Default SCS(kHz) | Channel raster | Sync raster |
| --- | --- | --- | --- | --- | --- |
| n257 | 26.5 –29.5 GHz | 50 | 120 | 60kHz | 17.34MHz |
|  |  | 100 | 240 |  | 37.5MHz |
| n258 | 24.25 – 27.5 GHz | 50 | 120 | 60kHz | 17.34MHz |
|  |  | 100 | 240 |  | 37.5MHz |
| n260 | 37–40 GHz | 50 | 120 | 60kHz | 17.34MHz |
|  |  | 100 | 240 |  | 37.5MHz |