3GPP TSG-WG RAN4 Meeting #85	R4-1712537

# Reno, Nevada, USA 27th Nov – 1st Dec, 2017

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.715-05-01: CA_5DL_2A-48C-48C_1UL_BCS0

Agenda Item: 			8.5.2

Document for:	Approval

# Introduction

This TP introduces 5DL/1UL CA configuration CA_2A-48C-48C in to the TR 36.715-05-01

**************************** Start of changes ********************************************

## 5.X	CA_2A-48C-48C_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 5DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-48C-48C | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 2 and Band 48 CA with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |
| 48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 |

It can be seen from Table 5.X.2-1 that the second harmonic of band 2 falls in band 48 only for the channel edge.

### 5.X.3	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 2 and Band 48 for 1UL/5DL CA_2A-48C-48C, the TIB,c and RIB,c values are shown in Table 5.X.3-1 and Table 5.X.3-2, respectively, as derived from CA_2A-48A.

Table 5.X.4-1: ΔTIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-48C-48C | 2 | 0.6 |
|  | 48 | 0.8 |

Table 5.X.4-2: ΔRIB,c for 5DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-48C-48C | 2 | 0.2 |
|  | 48 | 0.5 |

### 5.X.4	REFSENS requirements

For the REFSENS of B48 for CA_2A-48C-48C_1UL_BCS0 due to harmonic issue, the exceptions are shown in Table 5.X.4-1 and Table 5.X.4-2.

Table 5.X.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-48C-48C8,9 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C-48C10 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. … NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42C, CA_1A-3A-19A-42C, CA_1A-3A-21A-42C, CA_1A-3A-42C, CA_3A-28A-42C, CA_3A-19A-42C, CA_3A-28A-41A-42C, CA_3A-28A-41C-42A, and CA_2A-48C-48C. NOTE 9: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 5.X.4-2: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_2A-48C-48C | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |

**************************** End of changes ********************************************