3GPP TSG-RAN WG4 Meeting #85	R4-1712771

Reno, NV, USA, 27 November – 1 December 2017

Agenda Item:	6.3.3.2

Source:	Ericsson

Title:	MPDCCH simulation result for RLM with 2Rx UE

Document for:	Discussion

# 1	Introduction

RAN4#84bis agreed with the CR on RLM tests for non-BL/CE [1], but the SNR test points are still TBD. This contribution provides the simulation results to decide the SNR test points.

# 2	Discussion

## 2.1	Simulation parameters

Table 1 shows the simulation parameter we used for MPDCCH simulation. We reused the parameter used for BL/CE UE RLM tests except for the antenna configuration and the pair of aggregation level and repetition level.

Table 1	Simulation parameters for RLM for CEModeB UE with 2Rx.

| Parameter | MPDCCH CE Mode A |
| --- | --- |
| DCI format | DCI Format 6-1A |
| System Bandwidth | 10 MHz |
| Channel model | AWGN ETU30 |
| Antenna configuration | 2x2 |
| Number of information bits (without 16 bits CRC) | FDD: 29 bits |
| Antenna correlation | Low |
| Aggregation level and Repetition level | OOS: (24,4), (16,2) IS: (8,2), (4,1) |
| Starting OFDM symbols (CFI) | 2 |
| Frequency hopping | OFF |
| Number of PRB | 4 for Aggregation levels 4, 8, and 16 2+4 for Aggregation level 24 |
| Transmission type configured to UE | Distributed |
| DMRS scrambling sequence initialization parameter for UE-SS | PCID = 1 |
| Channel Estimation | DMRS based |
| UE residual frequency error | 50 Hz |

## 2.2	Simulation results

Figure 1 and Figure 2 show the ideal MPDCCH simulation with AWGN and ETU30 respectively. The required SNR values at 10% BLER (Out-of-synch) and 2% BLER (In-synch) are summarized in Table 2 and Table 3, respectively.

Figure 1	MPDCCH simulation result with AWGN.

Figure 2	MPDCCH simulation result with ETU30.

Table 2	Required SNR [dB] to achieve CE Mode A UE MPDCCH BLER=10% with 2 receive antenna configuration.

|  | Out-of-synch: SNR@BLER=10% |  |
| --- | --- | --- |
| (AL, Rmax) | AWGN | ETU30 |
| (24, 4) | -13.4 | -12.8 |
| (16, 2) | -12.1 | -9.6 |

Table 3	Required SNR [dB] to achieve CE Mode A UE MPDCCH BLER=2% with 2 receive antenna configuration.

|  | In-synch: SNR@BLER=2% |  |
| --- | --- | --- |
| (AL, Rmax) | AWGN | ETU30 |
| (8, 2) | -9.2 | -5.6 |
| (4, 1) | -4.6 | -9.4 |

# 3	References

R4-1710452, “CR for RLM tests for non-BL/CE UE (R13)”, Qualcomm.