3GPP TSG-RAN WG4 Meeting #85	R4-1713137

Reno, USA, 27 November – 1 December 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	feNB-IoT measurements in-band deployment

Agenda item:	8.20.4

Document for:	Discussion

# 1	Introduction

In RAN4#84bis meeting in Dubrovnik RAN4 continued the discussions related to feNB-IoT work. One outcome from the meeting was an agreement that RAN4 will not further study work on combining NRS and NSS measurements.  I.e. RAN4 will focus on measurement improvements based only on UE using the NSS signal for RRM measurements. Several companies provided simulation results based on the simulation assumptions for evaluating NRSRP measurement performance with NRSRP as agreed in the Berlin meeting. In this paper, we provide summary of our simulation results provided in [2] and discuss on the next steps.

# 2	Discussion

Firstly, we take a look at the existing NRSRP measurement accuracy requirements as agreed for NB-IoT Rel-13. These requirements were developed based on NRS and inband deployment. In Rel-13 RAN4 agreed to have accuracy requirement for UEs under normal and enhanced coverage as follows:

### 8.14.2	NB-IoT intra frequency measurements under normal coverage

#### 8.14.2.1	NB-IoT intra frequency measurements when no DRX is used

In the RRC_CONNECTED state the measurement period for intra frequency measurements is 800ms. The NRSRP measurement accuracy shall be as specified in the sub-clauses 9.1.22.1.

### 8.14.3	NB-IoT intra frequency measurements under enhanced coverage

#### 8.14.3.1	NB-IoT intra frequency measurements when no DRX is used

In the RRC_CONNECTED state the measurement period for intra frequency measurements is 1600ms. The NRSRP measurement accuracy shall be as specified in the sub-clauses 9.1.22.1.

Besides the non-DRX requirements also requirements for UE in DRX were defined. The actual measurement accuracy requirements are listed in table 9.1.22.1-1 below.

Table 9.1.22.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1 for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 8 | 11 | -6 dB | NFDD_G | N/A | [-70] | [-50] |
| 10.3 | 13.3 | -15 Ês/Iot -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 12.3 | 15.3 | -15 Ês/Iot -6 dB | NFDD_G | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

The UE is informed about any power offset between the NRS and the NB-SSS signal. This means that it is possible to utilize the NB-SSS signal for NRSRP measurements.

Using the NB-SSS signal for RRM measurements provides a significant increase in the number of reference signals available for the UE for measurements which will enable better measurement accuracy on UE side. Next, we summarize the achievable NRSRP measurement accuracy using the NB-SSS signal based on [2].

# 3	Simulation Results

## 3.1	Simulation Results

We have used in-band deployment scenario, with puncturing of the NB-SSS by 2Tx CRS. We have used 40ms sampling rate and average all samples over the L1 measurement period. NB-SSS is transmitted from port 0 only.

In the following sections present the results for NRSRP. All results are without impairment margin (except of course the reference legacy requirements numbers).

## 3.2	Simulation Results

### 3.2.1	NRSRP -6 dB

The L1 measurement period when under normal coverage is 800ms. Comparing the results from [2] with the current requirements, we see a significant improvement in the measurement accuracy (note: NB-SSS results are without impairments):

| SNR [dB] | Signal | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -6 | NB-SSS | 800 | AWGN | -0.99 | -0.36 | 0.27 | 0.99 | 1.26 |
| -6 | NRS | 800 | AWGN |  |  |  | 6 |  |

### 3.2.2	NRSRP -15 dB

The L1 measurement period when under enhanced coverage is 1600ms. Comparing the results from [2] with the current requirements, we see a significant improvement in the measurement accuracy (note: NB-SSS results are without impairments):

| SNR [dB] | Signal | Meas period [ms] | Channel model | 5% | 50% | 95% | Absolute accuracy | Relative Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -15 | NB-SSS | 1600 | AWGN | -3.47 | 0.09 | 2.06 | 3.47 | 5.53 |
| -15 | NRS | 1600 | AWGN |  |  |  | 10.3 |  |

## 3.2	Simulation Results Discussion

Based on these results it is clear that the UE NRSRP measurement accuracy when using the NB-SSS is significantly improved compared to legacy measurement accuracy when using the NRS. For the normal coverage mode, the improvement is in the order of around 2-3dB when accounting the UE impairments of ~2dB. For enhanced coverage mode, the accuracy improvement is ever bigger – around 4-5dB when accounting the UE impairments of ~2dB.

Proposal 1: NRSRP measurement accuracy in normal coverage can be improved by 2dB when using NSSS compared to using NRS.

Proposal 2: NRSRP measurement accuracy in enhanced coverage can be improved by 4.5dB when using NSSS compared to using NRS.

This would in practice lead to NRSRP measurement accuracy as follows assuming accuracy is improved with 2dB for normal coverage and 4.5 for enhanced coverage:

Table 9.1.22.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1 for HD-FDD

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 4 | 7 | -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 6 | 9 | -6 dB | NFDD_G | N/A | [-70] | [-50] |
| 5.8 | 8.8 | -15 Ês/Iot -6 dB | NFDD_G | [-118] | N/A | [-70] |
| 7.8 | 10.8 | -15 Ês/Iot -6 dB | NFDD_G | N/A | [-70] | [-50] |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

In addition to discussing the NRSRP accuracy RAN4 will of course also have to discuss the NRSRQ accuracy requirements. In [3] we have provided a CR capturing the above NRSRP accuracy requirements.

# 4	Measurement metrics

As mentioned earlier there will be a need to update the NRSRP and NRSRQ measurement metric definition in 36.214. It would be necessary to account for both existing and new reference signal.

| Definition | Narrowband Reference signal received power (NRSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry either narrowband specific reference signals or the resource elements that carry the narrow band secondary synchronization signal within the considered measurement frequency bandwidth. For NRS based NRSRP determination the narrowband reference signals for the first antenna port (R0 or R1000) according to TS 36.211 [3] shall be used. If the UE can reliably detect that a second antenna port (R1 or R1001) is available it may use the second antenna port in addition to the first antenna port to determine NRSRP. For narrow band secondary synchronization based NRSRP determination the narrowband reference signals for the first antenna port (R0) according to TS 36.211 [3] shall be used. The reference point for the NRSRP shall be the antenna connector of the UE. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, |

NRSRQ definition would need a similar update. Additionally, when discussing the NRSRQ RAN4 would need to discuss where the UE should measure the NRSSI. Measuring the NRSSI using the same resource as the NSSS might bias the results. In [4] we have provided a draft LS to RAN1 for updating the definition.

Finally, it would also necessary for RAN4 to discuss whether UE support of NB-SSS signal for measurements could be allowed also for pre-Rel-15 UE as early implementable solution?

Proposal 3: RAN4 should discuss of using NSSS signal for measurement can be allowed for early UE implementation?

# 4 Conclusion

In this paper, we provide summary of our simulation results evaluating the NRSRP measurement performance improvement when NRSRP are based on NB-SSS measurement alone. Based on these simulation results it is clear that the measurement accuracy using the NB-SSS gives significantly improvement gain in the NRSRP measurement accuracy compared to legacy when using only the NRS.

Proposal 1: NRSRP measurement accuracy in normal coverage can be improved by 2dB when using NSSS compared to using NRS.

Proposal 2: NRSRP measurement accuracy in enhanced coverage can be improved by 4.5dB when using NSSS compared to using NRS.

Proposal 3: RAN4 should discuss of using NSSS signal for measurement can be allowed for early UE implementation?

Additionally, we provide text proposal for updated measurement accuracy numbers and new NRSRP measurement definition for 36.214. CR [3] and draft LS [4] have been provided.

# References

# [1] R4-1709045, Simulation assumptions for feNB-IoT in-band deployment, Nokia

# [2] R4-1710711, RRM measurement performance for feNB-IoT in-band deployment; Nokia, Nokia Shanghai Bell

# [3] R4-1713138, CR Introducing Measurement accuracy for UE Category NB1, Nokia, Nokia Shanghai Bell

# [4] R4-1713139, LS on NSSS based NRSRP measurement definition, Nokia, Nokia Shanghai Bell

# Annex

### Simulation assumptions for feNB-IoT in-band deployment [1]

| Parameters | Value | Comments |
| --- | --- | --- |
| Measurement bandwidth | 1 resource blocks | Both RSRP and RSRQ measured over 1 RB |
| System bandwidth | 1 resource blocks |  |
| L1 measurement period | 200ms, 400ms, 800 ms, 1600ms |  |
| Measurement sampling rate | 1 sample/40ms | Implementation dependent (NOTE 1) |
| L3 filtering | Disabled |  |
| Antenna configuration | 2TX, 1RX | Puncturing of NB-PSS and NB-SSS in in-band operation when colliding with 2TX CRS pattern Diversity scheme should be indicated in the simulation. |
| Power imbalance between NSSS and NRS | -3dB, 0 dB, 3 dB | Only applicable for measurement based on combination of NSSS and NRS |
| Mobility | Stationary UEs, |  |
| Channel model | AWGN, ETU 1Hz, EPA 1Hz | Models stationary device |
| Measurement type | Synchronization signals only based (NSSS), Combination of NRS and synchronization signals (NSSS) | Companies are encouraged to provide simulation results for measurements based on follows signals. |
| CP length | Normal |  |
| Carrier frequency | 2 GHz |  |
| Ec/Iot | -15dB, -12.6dB, -6dB | AWGN noise |
| Frequency error modelling | +/-50 Hz | With respect to reference cell |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results NOTE 2: Companies are requested to provide the details of the RS averaging techniques for interpretation and comparison of the results. |  |  |

The NB-SSS is transmitted from port 0.