3GPP TSG-RAN4 Meeting #85 	R4-1713853

# Reno, Nevada, US, 27 Nov – 1 Dec, 2017

Agenda item:	8.28.1.1

Source:	Intel Corporation

Title:	Further discussion on network based CRS-IM RRM

Document for:	Discussion

# Introduction

In RAN4 #84bis meeting the warm-up and cool-down mechanism was discussed but no consensus was reached. The warm-up and cool-down subframes are the subframes with full bandwidth CRS and therefore it’s very important to design the number of warm-up and cool-down subframe number for mitigating the impact to legacy UEs during the CRS muting. In this contribution, we would like to continue the analysis of [1] and propose the number of warm-up and cool-down subframes for different scenarios.

# Criteria for warm-up and cool-down subframe design

CRS muting is a big change to network side and also it will bring big challenge to the legacy UE implementation if CRS is muted on any subframe that UE is used for any kind of implementation. In order to guarantee the backward compatibility to legacy UE, some possible methods were mentioned in RAN4 #84 meeting, one of options is to use “AllowedMeasBandwidth” to indicate CRS muting is used or not, but the current definition of this IE is used to indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104, and it can only be used to imply the carrier measurement bandwidth of neighbour cell rather than serving cell. So the most possible solution to mitigate the impact to legacy UE is: to reserve some warm-up and cool-down subframes to make sure legacy UE is able to manipulate the implementation on full bandwidth CRS before or after certain specified behaviour, e.g. random access. The criteria for warm-up and cool-down subframes design is to guarantee the backward compatibility, that is, the warm-up and cool-down subframes design should guarantee that CRS muting is fully transparent to legacy UE and will not cause any impairment to legacy UE.

Note: Here the legacy UE means the R14-and-before UE.

Proposal 1:  The criteria for warm-up and cool-down subframes design is that it should guarantee that CRS muting is fully transparent to legacy UE and will not cause any degradation to legacy UE.

It is also important to note that it’s really difficult to design different warm-up and cool-down subframe numbers for legacy UE and R15 UE respectively for some scenarios. The reason is: in the network coverage there might be mixed UEs(legacy UE  and R15 UE); even though network can know the current active UEs are all R15, it’s very hard to say there is no legacy UE who is trying to access the network. If there is at least one legacy UE who tries to access network (this behavior is out of NW knowledge), the short WU/CD will also degrade the performance of this legacy UE (e.g. performance of SI reading, RACH) and in some cases it may increase the UE power consumption. So the warm-up and cool-down subframe design shall be a network behaviour and shall not be differentiated among different release UEs for some certain scenarios (e.g. RACH, SI reading), and in those scenarios the WU/CD design shall be based on the assumption of legacy UE. .

Proposal 2:  The warm-up and cool-down subframe design shall be based on the assumption of legacy UE for certain scenarios, e.g RACH and SI reading.

However, if considering the requirement design, we may have different prerequisite for different release UE to fulfil the corresponding requirements, and that is, we may need to have some condition clarification before each corresponding requirement, for example:

The requirements in clause xxx shall apply for the UEs supporting the network based CRS interference mitigation handling, and CRS muting handling features. Moreover, the requirements shall be satisfied provided that the following additional conditions are fulfilled: - The number of warm-up and cool-down subframes with full bandwidth CRS is M and N respectively if CRS muting is used.

The number of warm-up and cool-down subframes for legacy UE to fulfil the legacy requirements is probably more than R15 UE, which is because R15 UE may be able to aware the CRS muting is used or not.

So if network implemented less subframes than the subframe number defined in prerequisite of RRM requirement, UE is not required to fulfil the corresponding requirement. But it may need more consideration on how to reflect those prerequisites for the previous release specification, especially the frozen releases. Maybe capturing the conditions in TS36.307 is one of the options as well.

Proposal 3: The prerequisite clarification for applying the legacy RRM requirements to legacy UE when CRS muting is used might be captured in TS36.307.

# Warm-up and cool-down subframe design for legacy UE

As summarized in the previous WF in RAN4 #84, in this section we analyse the warm-up and cool-down subframes scenario by scenario.

- IDLE Full BW CRS shall be assumed during: - All configured paging occasions and also [TBD subframes] for warm up & cool down - SI acquisition (SIB1 and SI-window) and also [TBD subframes] for warm up & cool down - RA procedure (at least partly), e.g.: - Prio to RA transmission occasions, [TBD subframes] - During Msg2 monitoring and also [TBD subframes] for warm up - During Msg4 monitoring and also [TBD subframes] for warm up eDRX: - 6 RB CRS may be assumed outside PTW (inside PTW is according to the above) - FFS additional warm up before the PTW - CONNECTED - DRX is configured: full BW CRS in serving cell during - UE Active Time and also [TBD subframes] for warm up & cool down - RA (at least partly) - on-going retransmissions in UL and DL, in semi-persistently scheduled DL resources, in the resources with HARQ feedback - SCell activation period and when the SCell is activated except for the SCell deactivation period - RSTD: existing requirements apply, under full BW CRS in positioning subframes

## Paging occasions

For paging occasions UE is expected to monitor PDCCH during the on-duration time of DRX in order to check for the presence of a paging message and if the PDCCH indicate that a paging message is present, then UE needs to decode the paging message carried on PDSCH. In order to monitor the PDCCH and decode the corresponding PDSCH, UE may need some pre-processing before the possible paging occasion subframe, which includes e.g. AGC estimation and timing/frequency tracking. The algorithm for AGC estimation may differ among different companies, but the general idea shall be similar, that is, to maintain a constant signal level at the amplifier output to make sure the good reception of DL data. So in order to mitigate the impairment to legacy UE AGC, at least one subframe with full bandwidth shall be reserved. In CA activation requirement, 20ms is assumed for CRS timing and frequency tracking where TDD configuration 0 is considered as the worst case, and that mean in CONNECTED mode at least 4 subframes shall be used for timing and frequency tracking. Even though the higher SINR may shorten the timing/frequency tracking delay, we still need to focus on the worst case to reserve sufficient subframes for UEs with all conditions and then we choose to use 4 subframes. We propose to use the similar approach here, and propose that at least 4 subframes shall be reserved for timing and frequency tracking with full bandwidth CRS. So considering 1 subframe margin, in total the warm-up subframe number is at least 6 for non-MBSFN subframe case.

For cool-down subframes, some of them might be used for continuous channel estimation, and the subframe number used for channel estimation is significant rely on which algorithm is used, and here in order to consider the margins and the possible algorithm, we propose at least 1 subframes shall be used for cool-down.

Besides the paging in IDLE mode, paging is also used in CONNECTED mode for e.g. CMAS(commercial mobile alert system)/ETWS(Earthquake and tsunami warning system); so the full BW CRS shall be assumed during all configured paging occasions in both IDLE and CONNECTED.

Proposal 4: the full BW CRS shall be guaranteed during all configured paging occasions in both IDLE and CONNECTED.

Proposal 5: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before all configured paging occasions, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after all configured paging occasions, in both IDLE and CONNECTED.

## SI acquisition

UE needs to read SIB1 and SIB2 before the random access, so the warm-up can cool-down subframes are necessary for all the SIB1 and SIB2 occasions. Moreover, it is up to UE implementation to read the other SIBs ant any available SI occasion, and therefore it’s necessary to reserve sufficient warm-up and cool-down subframes for those SI acquisition as well. SIB1 and all the SI windows for other SIBs are carried on PDSCH, and therefore the same design of paging reception can be reused for SI acquisition.

Besides the SI acquisition in IDLE mode, SI acquisition is also need to be considered in the CONNECTED mode, and also if the RLF happens then the SI acquisition is needed during the RRC reestablishment.

Proposal 6: the full BW CRS shall be guaranteed during all SI acquisition windows (all SIBs) in all cases (including IDLE, CONNECTED and other cases).

For SI acquisition, the number of warm-up/cool-down subframes for IDLE and CONNECTED modes could be same, since in CONNECTED mode the SI acquisition may happen when RLF is triggered (which is quite similar as IDLE condition).

Proposal 7: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before SIBs, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after SIBs, in all cases. SIBs here includes SIB1 and all the SI windows.

## Random access

Random access may be needed in both IDLE and CONNECTED modes, and moreover, during initial access or reestablishment. Before RA the timing information is very important for UL transmission of the preamble, and the related requirement is defined in TS36.133 as,

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies when it is the first transmission in a DRX, eDRX_CONN cycle for PUCCH, PUSCH and SRS or it is the first transmission after RACH-less handover or it is the PRACH transmission.

The Te is partially related with DL BW which is assumed to carry CRS on full BW. So in this case, we need to consider the subframes before the RACH for timing estimation, and therefore considering the implementation and some margin, we propose to reserve 6 subframes with full BW CRS before each RA in all cases (including IDLE, CONNECTED and other cases).

Proposal 8: At least [6] subframes with full bandwidth CRS shall be reserved before RA transmission occasions in all cases.

For msg2 and msg4 monitoring, the mechanism shall be similar as PDSCH reception (analysed in paging reception) and both warm-up and cool-down are needed.

Proposal 9: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before msg2&4 reception, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after msg2&4 reception, in all cases.

## UE active time in C-DRX

The active time defintion is specified in TS36.321:

When a DRX cycle is configured, the Active Time includes the time while: - onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimer or drx-ULRetransmissionTimer or mac-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or - an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer for synchronous HARQ process; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity (as described in subclause 5.1.4).

The on-going retransmissions in UL and DL in the  resources with HARQ feedback are already taken into account in the active time of CDRX. And the UE is not require to monitor the PDCCH outside the active time and that means UE is also not required to receive PDSCH out side the active time. So the full BW CRS shall be guaranteed during UE active time.

For CQI reporting in CDRX, the UE behaviour is required as below(TS36.321):

- if CQI masking (cqi-Mask) is setup by upper layers: - in current subframe n, if onDurationTimer would not be running considering grants/assignments/DRX Command MAC control elements/Long DRX Command MAC control elements received until and including subframe n-5 when evaluating all DRX Active Time conditions as specified in this subclause, CQI/PMI/RI/PTI/CRI on PUCCH shall not be reported. - else: - in current subframe n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC control elements/Long DRX Command MAC control elements received and Scheduling Request sent until and including subframe n-5 when evaluating all DRX Active Time conditions as specified in this subclause, CQI/PMI/RI/PTI/CRI on PUCCH shall not be reported.

The main idea is: if CDRX is eanbled and UE is in sleeping due to CDRX acitivity, UE is not required to send CSI(CQI /PMI /RI).

However, in C-DRX case, the AGC and time/frequency tracking may need to be pre-processed, and UE will also reserve some subframes after the on-duration time for certain UE implementations of channel estimation and some margins reagrdless of whether other timer is triggered or not. The early warm-up and later sleep-down subframes may be varied according to the SINR condition and DRX cycle length, but as a worst case, we propose to reserve 10 subframes for warm up and 6 subframes for cool down before and after UE on-duration time in C-DRX.

Proposal 10: At least [10] subframes with full bandwidth CRS shall be reserved for warm up before UE on-duration time in C-DRX, and at least [6] subframes with full bandwidth CRS shall be reserved for cool down after UE on-duration time in C-DRX.

## Scheduling request(SR) in CDRX

For SR transmission it may be carried on PUCCH or on RACH (if there is no PUCCH resource avaiable, defined in TS36.300), but the warm-up and coold mechanism for SR over RACH can be considered as normal RACH case. Here we focus on SR over PUCCH, in CDRX, UE can send SR even when it is outside the active time, however once UE sends SR to eNB, it should monitor PDCCH continuously till it gets DCI0. Before the UE send SR, UE may need subframes (6 subframs) to derive/calibrate the UL timing to make sure a synchronized trasmission to eNB, and after receive the PDCCH of UL grant UE may also need subframe (1 subframe) of cool-down for some channel estimation implementation.

Proposal 11: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before UE SR-over-PUCCH, and at least [1] subframes with full bandwidth CRS shall be reserved for cool down after UE received corresponding UL grant, and full bandwidth CRS shall be guaranteed from UE SR-over-PUCCH to the corresponding UL grant reception.

## RSTD measurement in CDRX

In RSTD measurement, to generate an accurate measurement window to receive PRS/CRS from positioning cells, the UE may first need to derived the timing of reference cells (e.g. serving cell), and then use the prs-SubframeOffset and/or expectedRSTD to make sure the target cell PRS/CRS can be covered by this window. So that means before the PRS/CRS reception UE may also need warm-up subframes to tracking the reference cell DL timing and also to set proper AGC. So we propose to reserve 6 subframes before RSTD measurement occaisions for warm-up.

Proposal 12: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before RSTD measurement occasions, and full bandwidth CRS shall be guaranteed during all RSTD measurement occasions.

Based on the analysis above, the following table is proposed for the warm-up and cool-down mechanism:

Table 1. Warm-up and cool-down mechanism summary for legacy UE

| Scenario | Mode | Warm-up subframes | Cool-down subframes | Other full BW CRS occasions |
| --- | --- | --- | --- | --- |
| All configured paging occasions | IDLE, CONNECTED | [6] | [1] | All paging occasions |
| SI acquisition (SIB1 and SI-window) | IDLE, CONNECTED | [6] | [1] | All SI reading windows(incl. SIB1 and all other SIBs) |
| Prior to RA transmission occasions | IDLE, CONNECTED | [6] | [0] | In Connected mode from RAR window to HO complete; In IDLE mode, from RAR window to RRC configuration complete |
| Msg2 monitoring duration | IDLE, CONNECTED | [6] | [1] |  |
| Msg4 monitoring duration | IDLE, CONNECTED | [6] | [1] |  |
| On-duration of CDRX | CONNECTED | [10] | [6] | During active time |
| SR over PUCCH in CDRX | CONNECTED | [6] before SR | [1] after UL grant | From UE SR-over-PUCCH to the corresponding UL grant reception |
| RSTD measurement | CONNECTED | [6] | [0] | All RSTD measurement occasions |

On the other hand, for R15 and later UE in the network, if those UEs can have the CRS muting information from network then they may be able to work well under shorter warm-up and cool-down subframe design, compared with legacy UE.

# Conclusion

In this contribution, we would like to analyse and propose the number of warm-up and cool-down subframes for different scenarios.

Proposal 1:  The criteria for warm-up and cool-down subframes design is that it should guarantee that CRS muting is fully transparent to legacy UE and will not cause any degradation to legacy UE.

Proposal 2:  The warm-up and cool-down subframe design shall be based on the assumption of legacy UE for certain scenarios, e.g RACH and SI reading.

Proposal 3: The prerequisite of warm-up and cool-down subframes for the corresponding RRM requirements in specification might be different for previous release UEs and R15 UEs.

Proposal 4: the full BW CRS shall be guaranteed during all configured paging occasions in both IDLE and CONNECTED.

Proposal 5: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before all configured paging occasions, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after all configured paging occasions, in both IDLE and CONNECTED.

Proposal 6: the full BW CRS shall be guaranteed during all SI acquisition windows (all SIBs) in all cases (including IDLE, CONNECTED and other cases).

Proposal 7: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before SIBs, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after SIBs, in all cases. SIBs here includes SIB1 and all the SI windows.

Proposal 8: At least [6] subframes with full bandwidth CRS shall be reserved before RA transmission occasions in all cases.

Proposal 9: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before msg2&4 reception, and at least [1] subframe with full bandwidth CRS shall be reserved for cool down after msg2&4 reception, in all cases.

Proposal 10: At least [10] subframes with full bandwidth CRS shall be reserved for warm up before UE on-duration time in C-DRX, and at least [6] subframes with full bandwidth CRS shall be reserved for cool down after UE on-duration time in C-DRX.

Proposal 11: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before UE SR-over-PUCCH, and at least [1] subframes with full bandwidth CRS shall be reserved for cool down after UE received corresponding UL grant, and full bandwidth CRS shall be guaranteed from UE SR-over-PUCCH to the corresponding UL grant reception.

Proposal 12: At least [6] subframes with full bandwidth CRS shall be reserved for warm up before RSTD measurement occasions, and full bandwidth CRS shall be guaranteed during all RSTD measurement occasions.

# References

[1] R4-1710363, “On warm-up and cool-down subframes for network based CRS-IM”, Intel, RAN4 #85