3GPP TSG RAN WG4 Meeting #85                         R4-1714126

Reno, USA, Nov 27th-Dec 1st, 2017

Source: 	NEC

Title:		      Proposal on NR BS EVM Requirement for FR1

Agenda Item:    9.5.2.4.1.1

Document for:   Approval

# Introduction

In the last RAN4 meeting, a WF [1] on simulation assumption for NR BS EVM requirements was approved. In this contribution, we provide our simulation results on NR BS EVM performance of frequency range 1 (FR1), and make a proposal.

# Discussion

CRS have been used for channel estimation in LTE DL. In NR, UE-specific DMRS will be used to estimate the DL channels. The NR DL channel estimation performance compared with LTE would be different due to the different reference signals. So, it’s necessary to evaluate NR DL link performance to check if the EVM values defined for LTE BS can be reused for NR BS.

## 2.1 Simulation assumptions

For FR1, RAN1 hasn't finalized the final DMRS patterns yet. In this contribution, we evaluate the EVM impacts together with two selected DMRS density patterns [2] illustrated in Figure 1. The EVM values can be obtained from Table 1 below as per the WF mentioned. In addition, the link level simulation assumptions can be found in Table 2 below.

Figure 1, DMRS pattern assumptions

Table 1, EVM for NR

| Modulation scheme for NR-PDSCH | Required EVM [%] measured over all allocated resource blocks |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |

Table 2, Link level simulation assumption

| Carrier Frequency | 4GHz |
| --- | --- |
| Channel Model | PedA |
| Bandwidth | 10MHz |
| Subcarrier Space | 15KHz |
| Coding Scheme | LDPC |
| Code Rate | QPSK 1/5 16QAM 2/5 64QAM 2/3 256QAM 5/6, |
| HARQ | 0 |

## Simulation results

In the EVM simulation, throughput with target EVM and ideal case (EVM 0) are compared. They are shown in Figure 2 and Figure 3. Figure 2 shows the EVM impacts on throughput performance for NR DMRS Pattern1. It can be observed that with EVM or not, the throughput difference becomes larger with the increase of modulation order. . For DMRS pattern 2 in figure 3, similar performance can be observed. To further check the difference, we list the throughput loss at 95% max throughput over different modulation schemes and DMRS patterns in Table 3. As the simulation results show, the throughput losses are under 5% for both NR DMRS pattern 1 and pattern 2 except the case where 256QAM and DMRS pattern 1 were assumed. So the LTE EVM requirements are tight enough for NR DMRS pattern 2. But for NR DMRS pattern 1, throughput loss of EVM 3.5% at 256QAM is 8.3624% which doesn’t achieve the 5% threshold in our simulation.

Observation 1: DMRS pattern 1 can’t achieve the 5% threshold and NR DMRS pattern 2 is suggested to be used when consider FR1 NR BS EVM requirements.

And, we propose,

Proposal 1: If DMRS pattern 2 or denser DMRS pattern is adopted," reuse LTE EVM requirements for FR1 NR BS. If DMRS pattern 1 is adopted, revisit EVM requirements for FR1 NR BS.

Table 3 Throughput loss compared with EVM 0

| Modulation [EVM %] | QPSK[17.5] | 16QAM[12.5] | 64QAM[8] | 256QAM[3.5] |
| --- | --- | --- | --- | --- |
| NR DMRS pattern 1 | 1.2869% | 1.2959% | 4.6738% | 8.3624% |
| NR DMRS pattern 2 | 1.1826% | 1.1380% | 2.9577% | 3.4916% |

Figure 2 EVM impacts on throughput performance, NR DMRS Pattern1

Figure 3 EVM impacts on throughput performance, NR DMRS Pattern2

# Conclusion

In this contribution, we provide our initial simulation results on BS EVM requirements for FR1. From the simulation results, we propose that,

Observation 1: DMRS pattern 1 can’t achieve the 5% threshold and NR DMRS pattern 2 is suggested to be used when consider FR1 NR BS EVM requirements.

Proposal 1: If DMRS pattern 2 or denser DMRS pattern is adopted," reuse LTE EVM requirements for FR1 NR BS. If DMRS pattern 1 is adopted, revisit EVM requirements for FR1 NR BS.

# Reference

[1] R4-1711777 ‘WF on simulation assumption for NR BS EVM requirement’, Huawei, HiSilicon

[2] R4-1711162 ‘Simulation Results of DM-RS patterns on EVM for FR1’, Ericsson

[3] RAN4#84bis Main Session Chairman Notes