3GPP TSG-RAN WG4 Meeting #86	R4-1801774

# Athens, Greece, 26 Feb - 2 Mar, 2018

Agenda item:	7.4.9.5

Source: 	Intel Corporation

Title: 	MPR evaluation results for FR2

Document for:	Approval

# Introduction

In this contribution, we provide FR2 MPR values based on WF [1] approved in AH1801 meeting.

# Discussion

- Goal is to provide a simple method to account for MPR for wider BW’s.

- A single MPR table is defined for all UEs whether they support the PA calibration gap or not;

  - NOTE: whether the PA calibration gap is introduced is addressed in a separate WF

- MPR 0dB waveform is 100MHz/60K QPSK DFT-S-OFDM 128RB0 to allow IBE measurement; 132RB is full allocation) [1] [5]

- Simulation Assumptions for EVM, SEM, ACLR are outlined in [6]

- For 50M/100M/200M assign a single MPR value relative to MPR 0 for each waveform and modulation.

- For 400M, add MPR offset based on practical implementation

  - Companies are encouraged to measure the effect large BW on MPR

- MPR, Xn, [n=1..8] values are relative to MPR 0dB.

- Y dB, is the MPR delta due to excess BW.

|  |  | MPR |  |
| --- | --- | --- | --- |
| Waveform | Modulation | 50M/100M/200M | 400M |
| DFT-s-OFMD | shaped pi/2 BPSK | <=X1 | <=X1+Y |
| DFT-s-OFMD | pi/2 BPSK | <=X2 | <=X2+Y |
| DFT-s-OFMD | QPSK | <=X3 | <=X3+Y |
| DFT-s-OFMD | 16QAM | <=X4 | <=X4+Y |
| CP-OFDM | QPSK | <=X5 | <=X5+Y |
| CP-OFDM | 16QAM | <=X6 | <=X6+Y |
| DFT-s-OFMD | 64QAM | <=X7 | <=X7+Y |
| CP-OFDM | 64QAM | <=X8 | <=X8+Y |

In San Diego meeting, the WF on mmW MPR [1] was approved and the following agreements were reached.

Based on above agreed MPR table format, we proposed our MPR evaluation results in the following sections.

## Simulation assumptions

Based on our mmWave PA model, the MPR evaluation was performed by using 8 PAs, 4 for each polarizations within an antenna array panel.

Our simulation assumptions are captured below, following the spec [2], otherwise specified:

- EVM requirement: table 6.4.2.1.1-1 [2].

| Parameter | Unit | Average EVM Level (%) |
| --- | --- | --- |
| Pi/2 BPSK | % | 30 |
| Pi/2 BPSK with pulse shaping | % | 30 |
| QPSK | % | [17.5] |
| 16 QAM | % | [12.5] |
| 64 QAM | % | 8 |

- IBE requirement: table 6.4.2.3.1-1 [2], except general term in IBE which followed IBE modifications agreed in AH1801 meeting.

- ACLR = 17dBc

- The maximum transmission BW configuration NRB:  Table 5.3.2-1 in [2]

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

- ACLR measurement BW is centered at channel center and is determined by Channel BW – 2* the minimum guardband between 60KHz SCS and 120KHz SCS for that channel BW. The minimum guardband in KHz is from table 5.3.3-1 [2].

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N.A |
| 120 | 1900 | 2420 | 4900 | 9860 |

- SEM requirement:  table 6.5.2.1.1-1 in [2], where ΔfOOB is up to 200% channel BW away from +/- channel edge.

| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |

## Simulation results

The simulation results for BW = 50/100/200 MHz are attached in the Appendix. We further verified the simulation results by real PA measurements and provided necessary justifications since simulation platform could not capture all non-linearity in real product design. For 400MHz BW, our measurement showed that Y=1dB is required due to increased BW larger than 200MHz. Our proposed FR2 MPR values are listed in the following table.

Table 1. Proposed FR2 MPR values

| WF type | modulation | MPR for BW = 50/100/200MHz | BW = 400MHz with Y=1dB |
| --- | --- | --- | --- |
| DFT-s-OFDM | Pulse Shaped Pi/2 BPSK | [TBD] | [TBD] |
|  | Pi/2 BPSK | 1.5 | 2.5 |
|  | QPSK | 1.5 | 2.5 |
|  | 16QAM | 3.0 | 4.0 |
|  | 64QAM | 5.0 | 6.0 |
| CP-OFDM | QPSK | 3.5 | 4.5 |
|  | 16QAM | 5.0 | 6.0 |
|  | 64QAM | 7.0 | 8.0 |

For pulse shaped Pi/2 BPSK, MPR values need further investigations since Zadoff Chu DMRS symbols limit the benefit of pulse shaped Pi/2 BPSK.

# Conclusion

In this contribution, we propose FR2 MPR values in table 1.

# References

[1] R4-1801192, ‘WF on mmW MPR’, Qualcomm.

[2] 3GPP TS 38.101-2 V1.0.0 (2017-12)

# Appendix

Attached is the file containing MPR simulation results.