3GPP TSG-RAN WG4 Meeting #86	R4-1802125

Athens, Greece, 26th Feb – 2nd Mar 2018

Source: 	Nokia

Title: 					TP to TR 36.761: UE transmitter characteristics for Extended-Band 12

Agenda Item: 			6.12.2

Document for:	Approval

# 1	Introduction

This contribution is a

**************************** Start of the TP *****************************************

## 8.1	UE aspect issues

### 8.1.1	UE transmitter characteristics

In this clause extended band 12 specific transmitter requirements are addressed, all other transmitter requirements are band agnostic.

#### 8.1.1.1	UE maximum output power

Due to almost identical band arrangement with band 12 extended band 12 maximum output power class requirement is defined to be same as band 12.

Table 8.1.1.1-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| ext -12 |  |  |  |  | 23 | ±22 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

Similarly for UE categories M1, M2, NB1 and NB2 the extended band 12 requirements for maximum output power are same as for band 12 for these categories.

#### 8.1.1.2	UE maximum output power with additional requirements

Additional SEM defined for band 12 applies also for extended band 12 hence NS_06 is also applicable to extended band 12 with an exception that only channel bandwidths of 5 and 10 MHz are specified for this band. No A-MPR is necessary.

Table 8.1.1.2-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
|  |  | ext-12 | 5, 10 | Table 5.6-1 | N/A |

Similarly for UE categories M1, M2, NB1 and NB2 the extended band 12 requirements for additional maximum output power reduction are same as for band 12 for these categories.

#### 8.1.1.3		Spurious emission band UE co-existence

Extended band 12 will be used in same geographical regions as band 12 thus same UE to UE co-existence requirements apply. Extended band 12 is also added as protected band for those bands and CA configurations that protect band 12.

Table 8.1.1.3-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| ext-12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 51, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

**************************** End of the TP *****************************************