3GPP TSG-RAN WG4 Meeting #86	R4-1802827

February 26th – March 2nd, 2018

Athens, GR

Agenda item:	7.4.9.2

Source: 	Qualcomm Incorporated

Title: 	On UE peak EIRP requirement for FR2 - impact on UL network performance

Document for:	Approval

# Introduction

In RAN4 #85, the UE power class for frequency range 2 (FR) NR was extensively discussed. With specific reference to the peak EIRP requirement, a Way Forward (WF) was approved defining the range 21.2 to 25.2 dBm for the 28 GHz band [1]. Although further happened in RAN4 NR Ad Hoc #4, no consensus was reached about the final minimum peak EIRP requirements.

In this contribution we extend our analysis in [9], providing a set of simulation results showing the impact of peak EIRP on the overall NR UL performance in 28GHz band. The assumptions adopted in this study are listed in [2]. The final goal of this paper is to provide information useful to finalize the peak EIRP requirement, i.e. to select a peak EIRP value within the agreed range.

Based on the outcome of the simulation campaign we make a compromised proposal for the minimum peak EIRP requirement in FR specifications.

# Discussion

After a very long debate in RAN4 #84, in [1] the following was agreed:

- The handheld UE peak EIRP range is defined as follows and will be captured in TS38.101-2 in this meeting

  - For 28 GHz: [21.2-25.2] dBm

  - For 39 GHz: [19.4-23.7] dBm

The ranges in square brackets are already implemented in the first release of UE RF technical specification for FR2 [3].

Furthermore, it is agreed “companies are encouraged to provide additional analysis with the intention to finalize the UE peak EIRP requirement”. The goal of the study presented in this contribution is to understand how NR network performance in millimeter wave (mmW) frequency range is sensitive to the UE peak EIRP requirement.

In the following sections, after a brief introduction of the adopted simulation assumptions, we will show UL throughput and coverage performance considering three different deployment scenarios, namely indoor office (InH), dense urban (UMi) and for Urban Macro (UMa).

## Simulation assumptions

The simulation assumptions adopted in this contribution are based on the ones agreed during NR study item and captured in TR 38.803 [4]. However, we propose to improve some of the assumptions in order to get a more typical deployment and UE behavior. The full set of assumptions which are the basis of this study is described in [2], where a rational for every modification compared to TR 38.803 is given. In Table 1, a summary of the key settings for each of the deployment scenario under analysis is reported.

Table 1. Table 2. Parameters for different deployment scenarios.

| InH | UMi | UMa |
| --- | --- | --- |
| EIRP = [21.2 - 25.2] dBm | EIRP = [21.2 - 25.2] dBm | EIRP = [21.2 - 25.2] dBm |
| As defined in TR 38.803 | Hexagonal Layout ISD from 100m to 400m | ISD from 200m to 500m |
|  | 10m antenna height | 25m antenna height |
|  | 10m UE to BS minimum 2D distance | 25m UE to BS minimum 2D distance |
|  | Indoor/Outdoor UE ratio 0%, 20% and 80% | Indoor/Outdoor UE ratio 0%, 20% and 80% |
|  | All other parameters as defined in TR 38.803 | All other parameters as defined in TR 38.803 |

Moreover, in order to characterize UE behavior more accurately, for all deployment scenarios the following modification compared to TR 38.803 are considered:

- UE resource allocation: 20 MHz (200MHz will be provided later or in future meetings)

- Power control SINR target: 22 dB

- UE elevation follows a Gaussian distribution centered at 45° with a standard deviation σ=10°

Since at mmW frequencies losses deriving from hand blockage become relevant, an angular blockage model has been adopted with details broadly discussed in [2] and [5]. In particular, UEs were considered to be held in Portrait mode and subject to a log-normal additional loss with mean μ=15.26 dB and  σ=3.8 dB. Such mean and standard deviation values were derived from a fitting of a measurement campaign [5] and capture a more realistic view of diffraction loss with hand in many real use-cases.

## Simulation results

In the following sections, we consider simulation results for all three deployment scenarios. The focus is on the impact of UE peak EIRP on NR network performance. The metrics adopted are the following:

- SINR

- Throughput (both 50% and 5% percentiles)

- Outage. As specified in TR 38.803 a UE is considered in outage when UL SINR is less than 10dB

In order to present the results in a compact format we will show the loss in performance of 21.2dBm EIRP compared to 25.2dBm.

### Results InH

Figure 1 shows the SINR Cumulative Distribution Function (CDF) in an indoor setting. The comparison is between UE peak EIRP of 21.2 dBm and UE peak EIRP of 25.2 dBm, considering an UL transmission bandwidth of 20MHz. In this scenario, the difference between 21.2dBm and 25.2dBm is negligible. These results differ from our analysis in [6] where we also showed degradation 50% and 5% degradation. Compared to our previous analysis, results presented in this paper consider only one angular region blocked by the hand. As a consequence, the results can be explained as follows:

- When a fixed (over the entire sphere) blockage is considered, co-channel interference becomes negligible and the system becomes noise limited. Under these conditions peak EIRP has a large impact even in InH as shown in [6]

- When only one angular region is selected and two panels are available, the best panel is always selected, therefore the actual loss in the link is much lower compared to a full blockage. In this case the system becomes less sensitive to peak EIRP as shown in Figure 1.

Because of the last bullet, it is expected that spherical coverage has significant impact in InH as well. This is consistent with what we observed in [7].

Figure 1. SINR CDF comparison between 21.2 and 25.2 dBm UE peak EIRP in InH

Also, it is worth noticing that we are considering 20MHz resource allocation. Increasing the BW allocation will make the system more sensitive to the need of higher EIRP.

### Results UMi

In this Section, we present SINR CDF results for UMi scenario. We consider different Inter-Site Distance (ISD) and indoor UE ratios, as defined in Section 2.1.

Figure 2 shows a comparison between SINR CDFs for two different UE peak EIRP values, i.e. 21.2 dBm and 25.2 dBm, when all UEs are outdoor. As expected, the difference changes with different ISD values with major variation for ISD = 400m. In Figure 3 and Figure 4 same comparison is made when 20% and 80% of UEs, respectively, are indoor. Network performance generally worsens with an increase in the number of UEs in outage. Such differences are summarized in the subsequent tables.

The following observations can be made:

- throughput degradation is observed starting for medium/large ISD, both 50%-tile and 5%-tile. In these cases the system is noise limited.

- the performance degradation due to lower EIRP increases when indoor UE are considered. This is due to the fact that higher EIRP can help compensate penetration loss for some UEs.

- the benefit of higher EIRP is clearly visible once the system starts to be noise limited. Even considering small resource allocation, higher EIRP helps improve system performance.

Figure 2. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – all UEs are outdoor

Figure 3. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – 20% of UEs are indoor

Figure 4. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – 80% of UEs are indoor

Table 3. 50%-tile SINR loss [dB] compared to 25.2 dBm UE peak EIRP

|  | 50%-tile SINR loss [dB] compared to 25.2 dBm EIRP |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 100m | ISD = 200m | ISD = 300m | ISD = 400m |
| 21.2 dBm - 0% | 0.00 | 0.21 | 2.72 | 3.67 |
| 21.2 dBm - 20% | 0.02 | 0.90 | 3.29 | 3.84 |
| 21.2 dBm - 80% | 3.82 | 3.90 | 3.92 | 3.94 |

Table 4. 50%-tile throughput loss compared to 25.2 dBm UE peak EIRP

|  | 50%-tile throughput loss compared to 25.2 dBm EIRP |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 100m | ISD = 200m | ISD = 300m | ISD = 400m |
| 21.2 dBm - 0% | 0% | 1% | 13% | 22% |
| 21.2 dBm - 20% | 0% | 4% | 18% | 28% |
| 21.2 dBm - 80% | 25% | 44% | 55% | 100% |

Table 5. 5%-tile SINR loss [dB] compared to 25.2 dBm UE peak EIRP

|  | 5%-tile SINR loss compared to 25.2 dBm EIRP [dB] |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 100m | ISD = 200m | ISD = 300m | ISD = 400m |
| 21.2 dBm - 0% | 0.10 | 3.19 | 3.68 | 3.73 |
| 21.2 dBm - 20% | 3.97 | 3.79 | 3.83 | 3.81 |
| 21.2 dBm - 80% | 3.85 | 3.86 | 3.90 | 3.95 |

Table 6. 5%-tile throughput loss compared to 25.2 dBm UE peak EIRP

|  | 5%-tile throughput loss compared to 25.2 dBm EIRP |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 100m | ISD = 200m | ISD = 300m | ISD = 400m |
| 21.2 dBm - 0% | 1% | 27% | 46% | 53% |
| 21.2 dBm - 20% | 54% | N.A. | N.A. | N.A. |
| 21.2 dBm - 80% | N.A. | N.A. | N.A. | N.A. |

Table 7. Outage comparison for different UE peak EIRP values and different % of indoor UE

|  | Outage |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 100m | ISD = 200m | ISD = 300m | ISD = 400m |
| 25.2 dBm - 0% | 0% | 1% | 1% | 2% |
| 21.2 dBm - 0% | 0% | 1% | 1% | 3% |
| 25.2 dBm - 20% | 3% | 7% | 11% | 13% |
| 21.2 dBm - 20% | 4% | 9% | 13% | 16% |
| 25.2 dBm - 80% | 7% | 25% | 38% | 48% |
| 21.2 dBm - 80% | 13% | 32% | 46% | 55% |

Also in this case, we only considered 20MHz resource allocation. Increasing the BW allocation will make the system more sensitive to the need of higher EIRP, especially if high throughput is required.

### Results UMi – Micro cells layout

In [8] it was agreed to use the scenario described in [4] for dense urban NW simulations. We argued several times that this scenario is not really meaningful since it does not represent a typical dense urban deployment. Regardless the specific cell layout, which is somehow arbitrary, an observation can be made on the UE-BS 2D distances:

- For the UMi deployement defined in TR 38.803, the average distance between UE and its closest BS is about 26 meters according to our simulations.

- On the other hand, for the UMa scenario with 200m ISD, the same distance is about 75 meters.

As a consequence, with such short minimum distance it is impossible to stress the validity of peak EIRP requirement. This is the reason why we preferred to carry out a sensitivity analysis based on variable ISD even for the dense urban deployment.

### Results UMa

In this section, UMa scenario is discussed. The analysis is similar to UMi in Section 2.2.2 but based on UMa deployment assumptions.

Figure 5 shows a comparison between SINR CDFs for two different UE peak EIRP values, i.e. 21.2 dBm and 25.2 dBm, when all UEs are outdoor while Figure 6 and Figure 7 are for the cases when 20% and 80% of UEs are indoor, respectively. 
Such differences are summarized in the following Tables.

The same observations made for UMi case apply here.

Figure 5. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – all UEs are outdoor – UMa

Figure 6. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – 20% of UEs are indoor – UMa

Figure 7. SINR CDF comparison between UE peak EIRP 21.2 dBm (continuous line) and UE peak EIRP 25.2 dBm (dashed line) for different ISD values – 80% of UEs are indoor – UMa

Table 8. 50%-tile SINR loss [dB] compared to 25.2 dBm UE peak EIRP

|  | 50%-tile SINR loss compared to 25.2 dBm EIRP [dB] |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 200m | ISD = 300m | ISD = 400m | ISD = 500m |
| 21.2 dBm - 0% | 0.04 | 0.90 | 3.39 | 3.85 |
| 21.2 dBm - 20% | 0.14 | 2.57 | 3.80 | 3.89 |
| 21.2 dBm - 80% | 3.92 | 3.91 | 3.96 | 3.97 |

Table 9. 50%-tile throughput loss compared to 25.2 dBm UE peak EIRP

|  | 50%-tile throughput loss compared to 25.2 dBm EIRP |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 200m | ISD = 300m | ISD = 400m | ISD = 500m |
| 21.2 dBm - 0% | 0% | 4% | 18% | 25% |
| 21.2 dBm - 20% | 1% | 12% | 23% | 32% |
| 21.2 dBm - 80% | 25% | 44% | 55% | 100% |

Table 10. 5%-tile SINR loss [dB] compared to 25.2 dBm UE peak EIRP

|  | 5%-tile SINR loss compared to 25.2 dBm EIRP [dB] |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 200m | ISD = 300m | ISD = 400m | ISD = 500m |
| 21.2 dBm - 0% | 2.77 | 3.72 | 3.82 | 3.83 |
| 21.2 dBm - 20% | 3.89 | 3.84 | 3.83 | 3.86 |
| 21.2 dBm - 80% | 3.88 | 3.90 | 3.93 | 3.97 |

Table 11. 5%-tile throughput loss compared to 25.2 dBm UE peak EIRP

|  | 5%-tile throughput loss compared to 25.2 dBm EIRP |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 200m | ISD = 300m | ISD = 400m | ISD = 500m |
| 21.2 dBm - 0% | 18% | 40% | 51% | 55% |
| 21.2 dBm - 20% | 100% | N.A. | N.A. | N.A. |
| 21.2 dBm - 80% | N.A. | N.A. | N.A. | N.A. |

Table 12. Outage comparison for different UE peak EIRP values and different % of indoor UE

|  | Outage |  |  |  |
| --- | --- | --- | --- | --- |
| Max EIRP [dBm] - % indoor UE | ISD = 200m | ISD = 300m | ISD = 400m | ISD = 500m |
| 25.2 dBm - 0% | 0% | 1% | 1% | 2% |
| 21.2 dBm - 0% | 1% | 1% | 2% | 4% |
| 25.2 dBm - 20% | 5% | 8% | 11% | 13% |
| 21.2 dBm - 20% | 7% | 10% | 13% | 17% |
| 25.2 dBm - 80% | 17% | 29% | 39% | 47% |
| 21.2 dBm - 80% | 24% | 36% | 47% | 55% |

# Conclusions

In this contribution, we presented simulation results showing the impact of UE peak EIRP on NR network performance. Based on the outcome of the study, we made the following observations:

- For InH deployment, 25.2 dBm EIRP does not bring significant gain. In this scenario, spherical coverage is the most relevant aspect as observed in [7]. If additional losses are considered, even in InH higher EIRP brings gain.

- For UMi and UMa:

  - When lower EIRP is considered, throughput degradation is observed starting for medium/large ISD, at both 50%-tile and 5%-tile. In such cases, the system is noise limited.

  - The performance degradation increases when indoor UEs are considered. This is due to the fact that higher EIRP can help compensate penetration losses for some UEs.

  - Even considering small resource allocation, higher EIRP helps improve system performance.

- All the simulations presented in this paper were based on 20MHz UL resource allocation. Larger allocation will make the system even more sensitive to peak EIRP, especially if high UL throughput is required.

- In summary, the benefit of higher EIRP is clearly visible once the system starts to be noise limited. In particular, in a pure noise limited scenario the 4dB gap in EIRP between 21.2-25.2dBm directly translates into 4dB gap in SINR.

Although the simulations presented in this paper show the need of 25.2dBm min EIRP, we share the concern of industrial design being important indirect factor for the success of cellular NR deployments and therefore we are ready compromise on the performance up to certain degree to enable progress in RAN4:

Proposal: RAN4 to adopt compromised proposal for peak EIRP 23.2 dBm @ 28 GHz and 21.5 dBm @ 39 GHz.

# Reference

- R4-1714445, “WF on pi/2 BPSK spectrum shaping and power class in FR2“, Intel Corporation, Apple, Qualcomm, IITH, LGE, Nokia, Huawei, Motorola Mobility.

- R4-1800861, “Simulation assumptions for UE max EIRP and spherical coverage evaluation”, Qualcomm Incorporated.

- 3GPP TS 38.101-2 V15.0.0 (2017-12), “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 15)”.

- 3GPP TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, V14.0.0.

- “Statistical Blockage Modeling and Robustness of Beamforming in Millimeter Wave Systems”, Vasanthan Raghavan, Lida Akhoondzadeh-Asl, Vladimir Podshivalov, Joakim Hulten, M. Ali Tassoudji, Ozge Hizir Koymen, Ashwin Sampath, and Junyi Li. Submitted to IEEE Transactions on Antennas and Propagation, available at: https://arxiv.org/abs/1801.03346.

- R4-1800864, “Preliminary results on mmW NR Coverage and Capacity: sensitivity to UE EIRP”, Qualcomm Incorporated.

- R4-1802828, “Evaluation of spherical coverage impact on NR network performance”, Qualcomm Incorporated.

- R4-1801198, “WF on network performance analysis for spherical coverage”, Samsung.

- R4-1800862, “On mmW NR Coverage and Capacity - Sensitivity to UE peak EIRP”, Qualcomm Incorporated.

- R4-1801953, “FR2 Peak EIRP”, Qualcomm Incorporated.