3GPP TSG-RAN WG4 Meeting #86	R4-1802887

Athens, GR, 26th Feb. – 2nd March, 2018

Source:	NTT DOCOMO, INC.

Title:	Indoor anechoic chamber test method procedure for ACLR

Agenda item:	6.27.3.1.4

Document for:	Approval

- Introduction

This contribution presents ACLR test procedure for an indoor anechoic chamber test method for conformance testing.

- ACLR conformance requirement

The ACLR requirement is the power ratio of the desired signal channel and the neighbouring channel bandwidth total powers that defined as the sum of the EIRP at a number of discrete directions around the sphere and the ACLR is defined as follows:

, where EIRPd is the filtered mean power within the desired signal channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

, where EIRPe is the filtered mean emission power in the neighbouring channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

To capture properly EIRP, EIRPd, p1 and EIRPe, p2 is associated to two orthogonal polarizations.

The number of discrete points required for conformance of the ACLR OTA being defined as the TRP requirement is subject to the TRP sampling grid discussion.

- Indoor anechoic chamber Test Method for ACLR

In this section, we discuss and propose the ACLR test method for an indoor anechoic chamber based on TR.37.842.

The method measures the ACLR in an anechoic chamber with the separation between the manufacturer declared coordinate system reference point of the AAS BS and the phase centre of the receiving antenna of no less than 2D2/λ, where D is the largest dimension of the antenna of AAS BS and λ is the wavelength. The measurement system setup is as depicted in figure 10.3.2.1-1.

The maximum size of the DUT is a chamber restriction that would affect the quality of the quiet zone. For larger DUT sizes larger size chambers should be considered such that the uncertainty of the quiet zone is taken into account.

The test consists of two stages: the calibration and the measurement. The test procedure is as follows.

# Figure 10.3.2.1-1: Indoor Anechoic Chamber measurement system setup for ACLR

# Figure 10.3.2.3-1: Indoor Anechoic Chamber calibration system setup for ACLR

# Stage 1 - Calibration: [Note: This stage may be omitted provided calibration stage has been performed already during output power measurement]

1)	Connect the reference antenna and the receiving antenna to the measurement RF out port and RF in port of the network analyzer, respectively, as shown in figure 10.3.2.3-1.

2)	Install the reference antenna with its beam peak direction and the height of its phase centre aligned with the receiving antenna.

3)	Set the centre frequency of the network analyzer to the carrier centre frequency of the tested signal for ACLR measurement of AAS BS and measure LFEIRP, E→D , which is equivalent to 20log|S21| (dB) obtained by the network analyzer:

-	LFEIRP, E→D: Pathloss between E and D in figure 10.3.2.3-1.

4)	Measure the cable loss, LFEIRP, E→F between the reference antenna connector and the network analyzer connector:

-	LFEIRP, E→F: Cable loss between E and F in figure 10.3.2.3-1.

5)	Calculate the calibration value between A and D with the following formula:

-	LEIRP_cal, A→D = LFEIRP, E→D  + GREF_ANT_EIRP, A→F -LFEIRP, E→F.

-	LEIRP_cal, A→D:  Calibration value between  A and D in figure 10.3.2.3-1.

-	GREF_ANT_EIRP, A→F: Antenna gain of the reference antenna.

# Stage 2 - Measurement:

6)	Uninstall the reference antenna and install the AAS BS with the manufacturer declared coordinate system reference point in the same place as the phase centre of the reference antenna. The manufacturer declared coordinate system orientation of the AAS BS is set to be aligned with the testing system.

7)	Set the AAS BS to generate the tested beam with the beam peak direction intended to be the same as the testing direction.

8)	Rotate the AAS BS to make the testing direction aligned with the direction of the receiving antenna.

9)	Set the AAS BS to transmit the test signal at the maximum power according to E-TM1.1.

10)	Measure the mean power within the desired signal channel bandwidth and the mean emission power in the neighbouring channel bandwidth for each carrier arriving at the measurement equipment connector, denoted by PR_AAS_desired, D, PR_AAS_emissiont, D

-	PR_AAS_desired, D: Measured mean power within the desired signal channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.3.2.1-1.

-	PR_AAS_emission, D: Measured mean emission power in the neighbouring channel bandwidth for each carrier at the measurement equipment connector at D in figure 10.3.2.1-1.

11)	Calculate the EIRP with the following formula:

# EIRPd = PR_AAS_desired, D+ LEIRP_cal, A→D

# EIRPe = PR_AAS_emission, D+ LEIRP_cal, A→D

12)	Repeat the above steps 10)~11) at a number of discrete directions around the sphere.

13)	Calculate TRPs from EIRPd and EIRPe following equations

, where EIRPd is the filtered mean power within the desired signal channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

emission power in the neighbouring channel bandwidth, and p1 and p2 denotes two orthogonal polarizations.

14) Calculate ACLR.

- Conclusion

This contribution presents:

- An option to combine ACLR and output power measurement

- Further investigation of the dynamic range of ACLR measurement equipment may be necessary.