3GPPRAN4#86	R4-1803020

Athens, Greece, Feb 26th – Mar 2nd

Title: 	Timing advance offset

Source: 	Qualcomm Incorporated

Agenda item:	7.9.8.1 - UE transmit timing, UE timer accuracy and timing advanced [NR_newRAT]

Document for:	Discussion

# Introduction

In this paper we provide analysis showing that the currently agreed upon values for  would potentially cause a serious degradation in throughput, especially for 120KHz SCS in FR2 and 60KHz SCS in FR1. We provide updated values for which should alleviate the throughput issue.

# Background

To illustrate the problem with the current , we need to understand the timeline from both UE and gNodeB perspective (see Figure 1). Here Tprop is propagation delay, TRXTX is UE switching time from Rx to Tx, TTXRX is UE switching time from Tx to Rx, and TAoffset is timing offset.

0

1

2

3

4

11

12

13

BS Timeline

0

1

TAoffset

Slot

0

1

2

3

4

11

12

13

UE Timeline

0

1

Tprop

Tprop

Tprop+TA offset

2

2

TRXTX

TTXRX

Figure 1: Timeline Illustrating timing offset and switching times

To account for round trip propagation delay and align uplink transmissions received at gNodeB, the network will send a timing advance to UE with TA= 2*Tprop. This TA value along with TAoffset is

Let n be number of symbols that are to be used for Rx to Tx switching and Tsym the symbol duration.

Observation 1: The maximum Rx to Tx switching time for UE is

TRXTX = n* Tsym – TA – TAoffset

Observation 2: The maximum Tx to Rx switching time for UE is

TTXRX = TA + TAoffset

Observation 3: The maximum time available for gNodeB for Rx to Tx swtiching is

TRXTX_gNodeB = TAoffset

In RAN4#85, the following values for were agreed,

| Duplex Mode of cell used for uplink transmission |  | TAoffset (in us) |
| --- | --- | --- |
| FDD in FR1 or FR2 | 0 (Note) | 0 |
| TDD in FR1 | 25560 | 13 |
| TDD in FR2 | 13763 | 7 |
| Note: Regardless of whether LTE-NR coexistence is configured or not in FR1 |  |  |

Take the example of 120KHz SCS, with a UE that is 300mts away from gNodeB that TA is 2 us. In this case, since TA+TAoffset = 7+2 = 9 > 8.92 = Tsym. Thus, the UE would need at least 2 symbols to switch from Rx to Tx. A similar problem would exist with 60KHz SCS in FR1. This would cause a ~14% throughput hit in the case of self-contained TTI over the scenario where this switching can be kept within one symbol. One way to avoid this would be to reduce the TAoffset.  Note that we need to ensure that we don’t reduce TAoffset too much as that wouldn’t provide gNodeB enough time to switch from Rx to Tx at its end.

## Proposed TAoffset

We continue with the 120KHz SCS example above with TA = 2 us. Under the assumptions that, 1) switching at UE should be done in 1 symbol and 2) UE and gNodeB should have equal times to switch, we get TAoffset = 3.46 us .

Another, albeit minor point, is that we would want the TAoffset values such that these are integer multiples of natural sampling rate of all possible BW’s at that SCS. This would ensure that we don’t have do any fractional timing adjustment.

Proposal 1: The values should be as defined in the following table

| Duplex Mode of cell used for uplink transmission |  |
| --- | --- |
| FDD | 0 (Note) |
| TDD 15KHz | 25600 |
| TDD 30KHz | 25600 |
| TDD 60KHz | 13600 |
| TDD 120KHz | 6800 |
| Note: Regardless of whether LTE-NR coexistence is configured or not in FR1 |  |

# Conclusions

Proposal 1: The values should be as defined in the following table

| Duplex Mode of cell used for uplink transmission |  |
| --- | --- |
| FDD | 0 (Note) |
| TDD 15KHz | 25600 |
| TDD 30KHz | 25600 |
| TDD 60KHz | 13600 |
| TDD 120KHz | 6800 |
| Note: Regardless of whether LTE-NR coexistence is configured or not in FR1 |  |