3GPP TSG-RAN WG4 Meeting #86	R4-1803103

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	6.21.7

Source:	Ericsson

Title:	Way forward on sTTI UE demodulation and CSI requirements

Document for:	Approval

# 1	General

## 1.1	Reference receiver

LMMSE receiver is baseline receiver for UE demodulation and CSI requirements for both subslot-based and slot-based frame structure. Note the simulation assumption does not model the interfering cells.

# 2	UE demodulation requirements

RAN4 will introduce SPDCCH and Slot-PDSCH/Subslot-PDSCH demodulation requirements.

Interested companies are encouraged to provide the ideal simulation results and propose the options according to the simulation assumption in RAN4#86bis.

## 2.1	Simulation assumption for SPDCCH

### 2.1.1	Test parameters

| Test number | Test 1 | Test 2 |
| --- | --- | --- |
| Description | CRS-based, Localized | DMRS-based, Distributed |
| Bandwidth | 10MHz | 10MHz |
| Number of PDCCH symbols (CFI) | 1 | 2 |
| Cyclic prefix | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN |
| Propagation condition | Option 1: EVA30 Option 2: EPA5 | EPA5 |
| Antenna configuration | Subslot TTI: Option 1: 4x2 Low Option 2: 2x2 Low Slot TTI: Option 1: 4x2 Low Option 2: 2x2 Low | 2x2 Low |
| Antenna ports | 0-3 for 4x2 0 and 1 for 2x2 | 107 |
| Precoder update granularity | N/A | 2 SREGs every 1ms |
| Beamforming precoder | N/A | Random precoder (TS36.101 B.4.4) (Note 4) |
| Short TTI length (Note 1) | Slot-based / subslot-based | Slot-based / Subslot-based |
| Configured SPDCCH-PRB-sets | {Set 1} | {Set 2} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 | Set 2 |
| Aggregation level | 2 SCCE | 8 SCCE |
| DCI format | 7-1C | 7-1F |
| Number of bits for DCI (without CRC) | [29] (FDD, 2 ports) [32] (FDD, 4 ports) [31] (TDD, 2 ports) [34] (TDD, 4 ports) | [28] (FDD) [30] (TDD) |
| PDSCH TM | TM4 | TM9 |
| Duplex modes (Note 2) | FDD / TDD | FDD / TDD |
| Evaluation criteria | 1% Pm-dsg | 1% Pm-dsg |
| Note 1: Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based SPDCCH demodulation only. Note 3: For slot-based transmission, DCI 7-1x is transmitted on PDCCH in slot 0 and on SPDCCH in slot 1. For subslot-based CRS-based transmission, DCI 7-1x is transmitted on PDCCH in subslot 0 and on SPDCCH in the rest of subslots. For subslot-based DMRS-based transmission, DCI 7-1x is transmitted on SPDCCH in all the subslots. Note 4: No CSI feedback is necessary for simulation. |  |  |

### 2.1.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 |
| --- | --- | --- |
| Reference symbol | CRS | DMRS |
| Transmission type | Localized | Distributed |
| Number of OFDM symbols | 1 | 2 |
| Number of PRBs | 16 | 16 |
| Allocated PRBs | {0, 1, … 15} | {1, 2, 7, 8, 13, 14, 19, 20, 29, 30, 35, 36, 41, 42, 47, 48} |
| Total number of SCCEs | 4 (16 SREGs) | 8 (32 SREGs) |

## 2.2	Simulation assumption for Slot-PDSCH/Subslot-PDSCH

### 2.2.1	Minimum processing time

- Subslot-based transmission:

  - UE can choose set1 with n+4 or set 2 with n+6 according to UE capability.

- Slot-based transmission:

  - Minimum processing time for: n+4

### 2.3.1	Test parameters

|  | Test 1 | Test 2 |
| --- | --- | --- |
| Bandwidth | 10MHz | 10MHz |
| Number of PDCCH symbols (CFI) | 2 | 2 |
| Cyclic prefix | Normal | Normal |
| Subframe configuration | Non-MBSFN | MBSFN (Note 3) |
| DMRS sharing (applicable only for subslot TTI) | N/A | Enabled (Note 4) |
| Propagation channel | Option 1: EVA30 Option 2: EPA5 | EPA5 |
| Antenna configuration | Subslot TTI: Option 1: 4x2 Low Option 2: 2x2 Low slot TTI: Option 1: 4x2 Low Option 2: 2x2 Low | 2x2 Low |
| Transmission modes | TM3 dual layer | TM9 Option 1: Single layer Option 2: dual layer |
| Antenna ports | 0-3 for 4x2 0 and 1 for 2x2 | 7 for single layer 7 and 8 for dual layer |
| Precoder update granularity | N/A | 1 PRG every 1ms |
| Beamforming precoder | N/A | Random precoder (TS36.101 B.4.1 for single layer, B.4.2 for dual layer) (Note 5) |
| TTI length (Note 1) | Slot-based / Subslot-based | Slot-based / Subslot-based |
| MCS | 16QAM 1/2 | QPSK 1/3 |
| Duplex mode (Note 2) | FDD / TDD | FDD / TDD |
| Evaluation criteria | 70% of Max Tput | 70% of Max Tput |
| Associated SPDCCH transmission | CRS-based, Localized | CRS-based, Localized |
| Aggregation level | [2 SCCE] | [4 SCCE] |
| Used/Unused SPDCCH resource indication (Note 6) | ’10’ | ’10’ |
| DCI format | 7-1B | 7-1F |
| Configured SPDCCH-PRB-sets | {Set 1},{ Set 2} | {Set 3, Set 4} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 for slot-based Set 2 for subslot-based | Set 3 for slot-based Set 4 for subslot-based |
| Note 1: sTTI PDSCH demodulation requirements should be specified for both slot-based and subslot-based. Applicability is based on the UE capability. Note 2: TDD is applicable for slot-based PDSCH demodulation only. Note 3: For FDD mode, [2 subframes (#1 and #6)] are allocated as MBSFN subframes. Note 4: DMRS is transmitted in subslot #1, #3, and #5. DMRS sharing is enabled in subslots #2 and #4. Note 5: No CSI feedback is necessary for simulation. Note 6: Set when SPDCCH L1 Reuse Indication is configured. Otherwise it is not signaled. |  |  |

### 2.3.2	SPDCCH-PRB-set configuration

|  | Set 1 | Set 2 | Set 3 | Set 4 |
| --- | --- | --- | --- | --- |
| Reference symbol | CRS | CRS | CRS | CRS |
| Transmission type | Localized | Localized | Localized | Localized |
| Number of OFDM symbols | 1 | 1 | 2 | 2 |
| Number of PRBs | 16 | 16 | 8 | 8 |
| Allocated PRBs | {0, 1, …, 15} | {34, 35, …, 49} | {0, 1, …, 7} | {8, 9, …, 15} |
| Rate matching mode | Mode 1 | Not configured | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured | {2, 0} | {1, 1} | Not configured |
| Total number of SCCEs | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) |

### 2.4.3	FRC tables (For information)

#### 2.4.3.1	CRS-based PDSCH

##### 2.4.3.1.1	Slot-based transmission

|  | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | TBD |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |
| Binary channel bits per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |

##### 2.4.3.1.2	Subslot-based transmission

|  | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 8 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |
| Binary channel bits per subslot |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | Bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |

#### 2.4.3.2	DMRS-based PDSCH

##### 2.4.3.2.1	Slot-based transmission

|  | Unit | Test 2 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Number of HARQ processes |  | [8] |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |
| Binary channel bits per slot |  |  |
| Slot index 0 | Bits | TBD |
| Slot index 1 | Bits | TBD |

Number of bits changes whether CSI-RS is transmitted within the same slot.

##### 2.4.3.2.2	Subslot-based transmission

|  | Unit | Test 2 |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | FFS |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |
| Binary channel bits per subslot |  |  |
| Subslot index 0 | Bits | N/A |
| Subslot index 1 | Bits | TBD |
| Subslot index 2 | Bits | TBD |
| Subslot index 3 | Bits | TBD |
| Subslot index 4 | Bits | TBD |

# 3	CSI reporting test

RAN4 will introduce the CQI feedback test with aperiodic CSI feedback with PUSCH 3-1 for both slot TTI and subslot TTI operation.

Interested companies are encouraged to provide the investigation results of metric options according to the simulation assumption in RAN4#86bis.

## 3.1	Minimum processing time

- Subslot-based transmission:

  - UE can choose set1 with n+4 or set 2 with n+6 according to UE capability.

- Slot-based transmission:

  - Minimum processing time for: n+4

## 3.2	CQI reporting under fading conditions with PUSCH

### 3.2.1	Aperiodic reporting mode with PUSCH 3-1 (Cell-specific reference signals)

#### Option 1: test with subband PDSCH scheduling

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [0.02].

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | TS36.101 Clause B.2.4 with s, a = 1, Hz |
| Antenna configuration | 2x2 |
| Transmission modes | TM4 |
| CodeBookSubsetRestrction bitmap | 00001 |
| Reporting interval | 5ms |
| CQI delay | [4ms] for slot-based FFS for subslot-based |
| Reporting mode | PUSCH 3-1 |
| Sub-band size | 12 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

#### Option 2: test with wideband PDSCH scheduling

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02]

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2x2) |
| Transmission modes | TM4 |
| CodeBookSubsetRestrction bitmap | 00001 |
| Reporting interval | 5ms |
| CQI delay | [4ms] for slot-based FFS for subslot-based |
| Reporting mode | PUSCH 3-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

### 3.2.2	Aperiodic reporting mode with PUSCH 3-1 (CSI reference signals)

#### Option 1: test with subband PDSCH scheduling

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	the ratio of the throughput obtained when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;

c)	when transmitting on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level the corresponding TBS, the average BLER for the indicated transport formats shall be greater or equal to [0.02].

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | TS36.101 Clause B.2.4 with s, a = 1, Hz |
| Antenna configuration | 1x2 Low |
| Transmission modes | TM9 |
| Beamforming model | TS 36.101 B.4.3 |
| CRS reference signals | Antenna port 0 |
| CSI reference signals | Antenna ports 15 and 16 |
| CSI-RS periodicity and subframe offset | 5ms / 1 |
| CSI-RS reference signal configuration | [4] |
| CodeBookSubsetRestrction bitmap | 00001 |
| Reporting interval | 5ms |
| CQI delay | [4ms] for slot-based FFS for subslot-based |
| Reporting mode | PUSCH 3-1 |
| Sub-band size | 12 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

#### Option 2: test with wideband PDSCH scheduling

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least  % of the time;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.02]

|  | Test 1 |
| --- | --- |
| Bandwidth | 10MHz |
| TTI length | Slot-based / Subslot-based |
| Number of PDCCH symbols | 2 |
| Cyclic prefix | Normal |
| Subframe configuration | Non-MBSFN |
| Propagation channel | EPA5 |
| Antenna configuration | High (2 x 2) |
| Transmission modes | TM9 |
| Beamforming model | TS 36.101 B.4.3 |
| CRS reference signals | Antenna port 0 |
| CSI reference signals | Antenna ports 15 and 16 |
| CSI-RS periodicity and subframe offset | 5ms / 1 |
| CSI-RS reference signal configuration | [4] |
| CodeBookSubsetRestrction bitmap | 00001 |
| Reporting interval | 5ms |
| CQI delay | [4ms] for slot-based FFS for subslot-based |
| Reporting mode | PUSCH 3-1 |
| PDSCH transmission bandwidth | 50 RB |
| Number of HARQ transmissions | 1 |
| Duplex mode | FDD / TDD |

# Information

## sTTI slot-index and subslot-index

Figure 1 Slot index for slot-based PDSCH and Subslot index for the subslot-based OFDM symbols.

## DCI Payload size (without CRC)

| Parameter | Format 7-1C | Format 7-1F | Notes |
| --- | --- | --- | --- |
| UL/DL differentiation flag | 1 | 1 |  |
| Resource block assignment | 8 | 8 | PDSCH allocation type 0 with 10MHz (RBG=6) |
| Modulation and coding scheme | 5 | 5 |  |
| HARQ process number | 4 | 4 |  |
| New data indicator | 1 | 1 |  |
| Redundancy version | 2 | 2 |  |
| TPC command for PUCCH | 2 | 2 |  |
| DMRS position indicator | 0 | 0 | DMRS sharing |
| Downlink assignment index | 0 (FDD) 2 (TDD) | 0 (FDD) 2 (TDD) |  |
| SRS request | 0 | 0 | SRS triggering not configured |
| Used/Unused SPDCCH resource indication | 0 | 0 | L1-based reuse indication not configured |
| SPUCCH resource indication | 2 | 2 |  |
| Precoding information | 4 (2 CRS ports) 7 (4 CRS ports) | N/A |  |
| Antenna port(s), scrambling identity and number of layers | N/A | 3 |  |
| Total (bits) | 29 (FDD, 2 ports) 32 (FDD, 4 ports) 31 (TDD, 2 ports) 34 (TDD, 4 ports) | 28 (FDD) 30 (TDD) |  |

## RRC-based rate-matching mode

Figure 2	RRC-based SPDCCH rate matching mode.

## L1-based rate-matching mode

Figure 3	L1-based SPDCCH rate matching mode.