3GPP TSG-WG RAN4 Meeting #86	R4-1803207

# Athens, Greece 26th Feb – 2nd March, 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 36.715-04-01: CA_4DL_1A-8A-20A-28A_1UL_BCS0

Agenda Item: 			6.4.2

Document for:	Approval

# Introduction

This TP introduces 4DL/1UL CA configuration CA_1A-8A-20A-28A in to the TR 36.715-04-01.

**************************** Start of changes ********************************************

## 5.X	CA_1A-8A-20A-28A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_1A-8A-20A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  | 8 |  |  | Yes | Yes |  |  |  |  |
|  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	Co-existence studies

Table 5.X.2-1 summarizes frequency ranges where harmonics occur due to Band 1, 8, 20 and 28 with 1 UL.

Table 5.X.2-1: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 1 | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 |
| 8 | 880 | 915 | 1760 | 1830 | 2640 | 2745 |
| 20 | 832 | 862 | 1664 | 1724 | 2496 | 2586 |
| 28 | 703 | 748 | 1406 | 1496 | 2109 | 2244 |

It can be seen from Table 5.X.2-1 that the third harmonic of band 28 UL falls into band 1 DL.

### 5.X.3	∆TIB,c and ∆RIB,c values

In the reference architecture of CA_8A-20A-28A, two main antennas are assumed to support three bands, with one duplexer connected for one antenna to support one band and one quadplexer or triplexer connected for another antenna to support other two bands. A similar architecture can be considered for CA_1A-8A-20A-28A with the main tranceiver of band 1 diplexed to one of the main low band antenna. If we avoid band 1 and 28 in the same antenna, the harmonic issue is possibly avoided.

Two diversity antennas are also required if B8/20/28 diversity path cannot be supported with one diversity antenna. In case UE support 4Rx for high band(s), 2 antennas can be used for low band Tx/Rx and 2 others for low band diversity antenna as illustrated in Figure 5.X.3-1.

Figure 5.X.3-1: A possible architecture for CA_1A-8A-20A-28A

To allow UE architectures including the harmonic trap filter in low bands in case band 1 and 28 are supported in the same antenna, we propose the maximum relaxation value among CA_1A-8A-20A, CA_1A-8A-28A, CA_1A-8A-20A, and CA_8A-20A-28A for this band combination. The proposed ΔTIB,c and ΔRIB,c are included in Table 5.X.3-1 and Table 5.X.3-2, respectively.

Table 5.X.3-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-28A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |

Table 5.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-8A-20A-28A | 1 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |

### 5.X.4	REFSENS requirement

The REFSENS exceptions due to the third harmonic issue are specified in Table 5.X.4-1 for the uplink configuration in Table 5.X.4-2.

Table 5.X.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |
|  | 20 |  |  |  | -93.8 | -91 | -89.8 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 5.X.4-2: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-8A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |

**************************** End of changes ********************************************