3GPP TSG-RAN4 #86	R4-1803299

# Athens, Greece, 26th February -2nd March 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Description of interferer waveform for receiver requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-02-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the receiver ACS, blocking, IM and ICS requirements, the interferer waveform is not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interferer waveform is added as DFT-S-OFDM. ACS frequency offset changed by 0.54MHz for 20MHz interferer in FR1 and 0.72MHz for 50MHz interferer in FR2 to account for SU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of receiver requirements incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4, 7.5, 7.7, 7.8, 10.5, 10.6, 10.8, 10.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 7.4	In-band selectivity and blocking

### 7.4.1	Adjacent Channel Selectivity (ACS)

#### 7.4.1.1	General

Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 7.4.1.2	Minimum requirement for BS type 1-C and BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For BS, the wanted and the interfering signal coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector are specified in table 7.4.1.2-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.2-2 for ACS. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each BS channel bandwidth and further specified in annex A.

The ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For a BS operating in non-contiguous spectrum within any operating band, the ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 7.4.1.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a BS capable of multi-band operation, the ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 7.4.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

# Table 7.4.1.2-1: Base station ACS requirement

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6dB | Wide Area: -52 Medium Range: -47 Local Area: -44 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2, 7.2.2-3 |  |  |

# Table 7.4.1.2-2: Base Station ACS interferer frequency offset values

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [2.5025] | 5MHz DFT-S-OFDM NR signal CS: 15kHz SCS: 15kHz |
| 10 | [2.5075] |  |
| 15 | [2.5125] |  |
| 20 | [2.5025] |  |
| 25 | [9.535] | 20MHz DFT-S-OFDM NR signal CS: 15kHz SCS: 15kHz |
| 30 | [9.585] |  |
| 40 | [9.535] |  |
| 50 | [9.485] |  |
| 60 | [9.585] |  |
| 70 | [9.535] |  |
| 80 | [9.485] |  |
| 90 | [9.585] |  |
| 100 | [9.535] |  |

#### 7.4.1.3	Void

#### 7.4.1.4	Void

### 7.4.2	In-band blocking

#### 7.4.2.1	General

The in-band blocking characteristics is a measure of the receiver’s ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 7.4.2.2	Minimum requirement for BS type 1-C and BS type 1-H

he throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel

For , with a wanted and an interfering signal coupled to BS type 1-C antenna connector or BS type 1-H TAB connector using the parameters in tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3 for general blocking and narrowband blocking requirements. The reference measurement channel for the wanted signal is identified in subclause 7.2.2 for each BS channel bandwidth and further specified in annex A.

The in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

BS type 1-C and BS type 1-H, the in-band blocking requirement applies from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluing the downlink frequency range of the operating band. The ΔfOOB for BS type 1-C and BS type 1-H is defined in table 7.4.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H.

Table 7.4.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB [MHz] |
| --- | --- | --- |
| BS type 1-C | 200 MHz ≥ FUL_high – FUL_low | 20 |
|  | 200 MHz < FUL_high – FUL_low ≤ 900 MHz | 60 |
| BS type 1-H | 100 MHz > FUL_high – FUL_low | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

[For a BS operating in non-contiguous spectrum within any operating band, the in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a BS capable of multi-band operation, the blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 7.4.2.2-1.]

[For a BS operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the BS channel bandwidth of the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For a BS capable of multi-band operation, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in Table 7.4.2.2-3. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

# Table 7.4.2.2-1: Base station general blocking requirement

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + 6 dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | ±7.5 | 5 MHz DFT-S-OFDM NR signal SCS: 15 kHz |
| [25, 30, 40, 50, 60, 70, 80,90, 100 | PREFSENS + 6 dB | Wide Area: -43 Medium Range: -38 Local Area: -35 | 30 | 20 MHz DFT-S-OFDM NR signal SCS: 15 kHz |
| NOTE: PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |  |  |  |

# Table 7.4.2.2-2: Base Station narrowband blocking requirement

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80,90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area: -49 Medium Range: -44 Local Area: -41 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that BS channel bandwidth NOTE 2: PREFSENS depends on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |

# Table 7.4.2.2-3: Base Station narrowband blocking interferer frequency offsets

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset to the lower/upper Base Station RF Bandwdith edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-S-OFDM NR signal, 1 RB CS: 15 kHz SCS: 15kHz |
| 10 | [347.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | [352.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | [342.5]+m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | [557.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 | 20 MHz DFT-S-OFDM NR signal, 1 RB CS: 15 kHz SCS: 15kHz |
| 30 | [562.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 40 | [557.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 50 | [552.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 60 | [562.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 70 | [557.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 80 | [552.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 90 | [562.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| 100 | [557.5]+m*180, m=0, 1, 2, 3, 4, 29, 54, 79, 104 |  |
| Note: Interfering signal consisting of one resource block is positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |

{Unchanged sections omitted}

## 7.7	Receiver intermodulation

### 7.7.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

### 7.7.2	Minimum requirement for BS type 1-C and BS type 1-H

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector or BS type 1-H TAB connector, with the conditions specified in tables 7.7.2-1 and 7.7.2-2 for intermodulation performance and in tables 7.7.2-3, and 7.7.2-4 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in table 7.2.2-1 and table 7.2.2-3 for each BS channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall in general be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60 kHz and BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30 kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in table 7.7.2-2 or 7.7.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

[For a BS capable of multi-band operation or multi-band TAB connectors, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

[For a BS capable of multi-band operation or multi-band TAB connectors, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the NR interfering signal in tables 7.7.2-2 and 7.7.2-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

# Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 | PREFSENS +6 dB | See Table 7.7.2-2 |
| Medium Range BS | -47 | PREFSENS +6 dB |  |
| Local Area BS | -44 | PREFSENS +6 dB |  |
| NOTE 1: PREFSENS depends on the BS class and on the BS channel bandwidth, see subclause 7.2. |  |  |  |

# Table 7.7.2-2: Interfering signals for intermodulation requirement

| N cBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±7.5] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 10 | [±7.45] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 15 | [±7.43] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 20 | [±7.38] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 30 | [±7.43] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 25 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 40 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 50 | [±7.35] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 60 | [±7.49] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 70 | [±7.42] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 80 | [±7.44] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 90 | [±25] | CW |
|  | [±7.43] | 5MHz DFT-S-OFDM NR signal |
| 100 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |

# Table 7.7.2-3: Narrowband intermodulation performance requirement in FR1

| BS type | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS + 6dB (Note 1) | -52 | See Table 7.7.2-4 |
| Medium Range BS | PREFSENS + 6dB (Note 2) | -47 |  |
| Local Area BS | PREFSENS + 6dB (Note 3) | -44 |  |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in table 7.2.2-1. NOTE 2: PREFSENS depends on the BS channel bandwidth as specified in table 7.2.2-2. NOTE 3: PREFSENS depends on the BS channel bandwidth as specified in table 7.2.2-3. |  |  |  |

# Table 7.7.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| channel bandwidthBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±360] | CW |
|  | [±1420] | 5MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 10 | [±325] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 15 (Note 2) | [±380] | CW |
|  | [±1600] | 5MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 20 (Note 2) | [±345] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 25 (Note 2) | [±325] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 30 (Note 2) | [±320] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 40 (Note 2) | [±310] | CW |
|  | [±2710] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 50 (Note 2) | [±330] | CW |
|  | [±3250] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 60 (Note 2) | [±350] | CW |
|  | [±3790] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 70 (Note 2) | [±400] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 80 (Note 2) | [±390] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 90 (Note 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| 100 (Note 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (Note 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

## 7.8	In-channel selectivity

### 7.8.1	General

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex A and shall be time aligned with the wanted signal.

### 7.8.2	Minimum requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A with parameters specified in Table 7.8.2-1 for Wide Area BS, in Table 7.8.2-2 for Medium Range BS and in Table7.8.2-3 for Local Area BS.

# Table 7.8.2-1: Wide Area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 7.8.2-2: Medium Range BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 7.8.2-3: Local area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

{Unchanged sections omitted}

## 10.5	OTA in-band selectivity and blocking

### 10.5.1	OTA adjacent channel selectivity

#### 10.5.1.1	General

OTA Adjacent channel selectivity (ACS) is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel frequency in the presence of an OTA adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

#### 10.5.1.2	Minimum requirement for BS type 1-O

he requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and

-	when the wanted signal is based on  the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 10.5.1.2-1 and table 10.5.1.2-2 for ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.2-1. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For multi-band RIBs, the OTA ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 10.5.1.2-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

# Table 10.5.1.2-1: OTA ACS requirement for BS type 1-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] (Note 2) | Interfering signal mean power [dBm] |
| --- | --- | --- |
| 5, 10, 15, 20 (Note 1) | EISminSENS + 6dB | Wide Area: -52 – ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| [25, 30, 40, 50, 60, 70, 80,90, 100] (Note 1) | EISminSENS + 6dB | Wide Area: -52– ΔminSENS Medium Range: -47– ΔminSENS Local Area: -44– ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth NOTE 2: EISminSENS depends on the BS channel bandwidth |  |  |

# Table 10.5.1.2-2: OTA ACS interferer frequency offset for BS type 1-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [2.5025] | 5MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 10 | [2.5075] | 5MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 15 | [2.5125] | 5MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 20 | [2.5025] | 5MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 25 | [9.535] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 30 | [9.585] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 40 | [9.535] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 50 | [9.485] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 60 | [9.585] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 70 | [9.535] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 80 | [9.485] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 90 | [9.585] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |
| 100 | [9.535] | 20MHz DFT-S-OFDM NR signal SCS: 15kHz |

#### 10.5.1.3	Minimum requirement for BS type 2-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2, the OTA wanted and the interfering signal are specified in table 10.5.1.3-1 and table 10.5.1.3-2 for ACS. The reference measurement channel for the OTA wanted signal is further specified in annex A.

The OTA ACS requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

[For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the NR interfering signal in table 10.5.1.3-2. The OTA interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For multi-band RIBs, the OTA ACS requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the NR interfering signal in table 10.5.1.3-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

# Table 10.5.1.3-1: OTA ACS requirement for BS type 2-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | OWanted signal mean power [dBm] | OInterfering signal mean power [dBm] |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6dB | EISREFSENS +[28.7 -SINR-IM] (Note 1) EISREFSENS +[28.7 -SINR-IM] (Note 2) |
| Note 1: Applicable to bands defined within the frequency spectrum range of 24.24 – 33.4 GHz Note 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |

# Table 10.5.1.3-2: OTA ACS interferer frequency offset for BS type 2-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 | [24.29] | 50MHz DFT-S-OFDM NR signal 60 kHz SCS |
| 100 | [24.31] | 50MHz DFT-S-OFDM NR signal 60 kHz SCS |
| 200 | [24.29] | 50MHz DFT-S-OFDM NR signal 60 kHz SCS |
| 400 | [24.31] | 50MHz DFT-S-OFDM NR signal 60 kHz SCS |

### 10.5.2	OTA in-band blocking

#### 10.5.2.1	General

The OTA in-band blocking characteristics is a measure of the receiver’s ability to receive a OTA wanted signal at its assigned channel in the presence of an unwanted OTA interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

#### 10.5.2.2	Minimum requirement for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

he throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with

For FR1, the OTA wanted and OTA interfering signal specified in tables 10.5.2.2-1, table 10.5.2.2-2 and table 10.5.2.2-3 for general OTA and narrowband OTA blocking requirements. The reference measurement channel for the OTA wanted signal is identified in subclase 10.3.2 further specified in annex A.

The OTA in-band blocking requirements apply outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

BS type 1-O the OTA in-band blocking requirement apply from FUL_low - ΔfOOB to FUL_high + ΔfOOB, excluding the downlink frequency range of the operating band. The ΔfOOB for BS type 1-O is defined in table 10.5.2.2-0.

Table 10.5.2.2-0: ΔfOOB offset for NR operating bands

| BS type | Operating band characteristics | ΔfOOB [MHz] |
| --- | --- | --- |
| BS type 1-O | 100 MHz > FUL_high – FUL_low | 20 |
|  | 100 MHz ≤ FUL_high – FUL_low ≤ 900 MHz | 60 |

[For RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA in-band blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in tables 10.5.2.2-1 and 10.5.2.2-3.]

# Table 10.5.2.2-1: General OTA blocking requirement for BS type 1-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] | OTA Interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| , 10, 15, 20 5, 10, 15, 20 | EISREFSENS + 6 dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 7.5 | 5 MHz DFT-S-OFDM NR signal SCS: 15 kHz |
|  | EISminSENS + 6 dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 7.5 | 5 MHz DFT-S-OFDM NR signal SCS: 15 kHz |
| 5,30, 40, 50, 60, 70, 80, 90, 100 25, | EISREFSENS + 6 dB | Wide Area: -43 - ΔOTAREFSENS Medium Range: -38 - ΔOTAREFSENS Local Area: -35 - ΔOTAREFSENS | 30 | 20 MHz DFT-S-OFDM NR signal SCS: 15 kHz |
|  | EISminSENS + 6 dB | Wide Area: -43 – ΔminSENS Medium Range: -38 – ΔminSENS Local Area: -35 – ΔminSENS | 30 | 20 MHz DFT-S-OFDM NR signal SCS: 15 kHz |

# Table 10.5.2.2-2: OTA narrowband blocking requirement for BS type 1-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | OTA Wanted signal mean power [dBm] | OTA Interfering signal mean power [dBm] |
| --- | --- | --- |
| , 10, 15, 20 5, 10, 15, 20 | EISREFSENS + 6 dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| 5, 30, 40, 50, 60, 70, 80, 90, 100 25, | EISREFSENS + 6 dB | Wide Area: -49 - ΔOTAREFSENS Medium Range: -44 - ΔOTAREFSENS Local Area: -41 - ΔOTAREFSENS |
|  | EISminSENS + 6 dB | Wide Area: -49 – ΔminSENS Medium Range: -44 – ΔminSENS Local Area: -41 – ΔminSENS |
| NOTE: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth |  |  |

# Table 10.5.2.2-3: OTA narrowband blocking interferer frequency offsets for BS type 1-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset to the band edge of the wanted carrier [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [342.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 5 MHz DFT-S-OFDM NR signal, 1 RB CS: 15 kHz SCS: 15kHz |
| 10 | [347.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 15 | [352.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 20 | [342.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 25 | [557.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 | 20 MHz DFT-S-OFDM NR signal, 1 RB CS: 15 kHz SCS: 15kHz |
| 30 | [562.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 40 | [557.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 50 | [552.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 60 | [562.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 70 | [557.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 80 | [552.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 90 | [562.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |
| 100 | [557.5] + m*180, m=0, 1, 2, 3, 4, 9, 14, 19, 24 |  |

#### 10.5.2.3	OTA in-band blocking for BS type 2-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR2 BS, OTA wanted and OTA interfering signal provided at RIB using the parameters in tables 10.5.2.3-1 for general OTA blocking requirements. The reference measurement channel for the wanted signal is further specified in annex A.

The OTA blocking requirements are applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges. The OTA blocking requirements apply in the in-band blocking frequency range, which is from [XX] MHz below the lowest frequency of the uplink operating band up to [XX] MHz above the highest frequency of the uplink operating band for BS type 2-O in an operating band less than [XX] MHz wide, but excludes the downlink frequency range of the operating band.

[For a RIBs supporting operation in non-contiguous spectrum within any operating band, the OTA blocking requirements apply in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as twice the interfering signal minimum offset in table 10.5.2.3-1. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.]

[For a multi-band RIBs, the OTA blocking requirements apply in the in-band blocking frequency ranges for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as twice the interfering signal minimum offset in table 10.5.2.3-1.]

# Table 10.5.2.3-1: General OTA blocking requirement for BS type 2-O

| NBS channel bandwidth of the lowest/highest carrier received [MHz] | OTA wanted signal mean power [dBm] | OTA interfering signal mean power [dBm] | OTA interfering signal centre frequency offset to the band edge of the wanted carrier [MHz] | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6dB | EISREFSENS + 33dB | 75 | 50 MHz DFT-S-OFDM NR signal 60 kHz SCS |

{Unchanged sections omitted}

## 10.8	OTA receiver intermodulation

### 10.8.1	General

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver unit to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. The requirement is defined as a directional requirement at the RIB.

### 10.8.2	Minimum requirement for BS type 1-O

The requirement applies at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

For NR, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal at the assigned channel frequency and two interfering signals at the RIB. with the conditions specified in tables 10.8.2-1 and 10.8.2-2 for intermodulation performance and in tables 10.8.2-3 and 10.8.2-4 for narrowband intermodulation performance.

The reference measurement channel for the wanted signal is identified in table 10.3.2-1, table 10.3.2-2 and table 10.3.2-3 for each BS channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal, except for the case of wanted signal subcarrier spacing 60kHz and BS BS channel bandwidth <=20MHz, for which the subcarrier spacing of the interfering signal should be 30kHz.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

or a RIBs supporting operation in non-contiguous spectrum within any operating band, the narrowband intermodulation requirement applies in addition inside any sub-block gap in case the sub-block gap is at least as wide as the BS channel bandwidth of the NR interfering signal in tables 10.8.2-2 and 10.8.2-4. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

[For multi-band RIBs, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the E-UTRA interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

[For multi-band RIBs, the narrowband intermodulation requirement applies in addition inside any Inter RF Bandwidth gap in case the gap size is at least as wide as the E-UTRA interfering signal in tables 10.8.2-2 and 10.8.2-4. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.]

# Table 10.8.2-1: General intermodulation requirement

| BS class | Mean power of interfering signals [dBm] | Wanted Signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | -52 + ΔOTAREFSENS | EISREFSENS + 6 dB | See Table 10.8.2-2 |
|  | -52 + ΔminSENS | EISminSENS + 6 dB |  |
| Medium Range BS | 47 + ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | 47 + ΔminSENS | EISminSENS + 6 dB |  |
| Local Area BS | 448 + ΔOTAREFSENS | EISREFSENS + 6 dB |  |
|  | 44 + ΔminSENS | EISminSENS + 6 dB |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the BS class and on the BS channel bandwidth, see subclause 10.3 and 10.2. |  |  |  |

# Table 10.8.2-2: Interfering signals for intermodulation requirement

| channel bandwidthBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the lower/upper base station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±7.5] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 10 | [±7.45] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 15 | [±7.43] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 20 | [±7.38] | CW |
|  | [±17.5] | 5MHz DFT-S-OFDM NR signal |
| 30 | [±7.43] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 25 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 40 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 50 | [±7.35] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 60 | [±7.49] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 70 | [±7.42] | CW |
|  | [±25] | 5MHz DFT-S-OFDM NR signal |
| 80 | [±7.44] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |
| 90 | [±25] | CW |
|  | [±7.43] | 5MHz DFT-S-OFDM NR signal |
| 100 | [±7.45] | CW |
|  | [±25] | 20MHz DFT-S-OFDM NR signal |

# Table 10.8.2-3: Narrowband intermodulation performance requirement in FR1

| BS class | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- |
| Wide Area BS | EISREFSENS + 6 dB (Note 1) | -52 - ΔOTAREFSENS | See Table 10.8.2-4 |
|  | EISminSENS + 6 dB (Note 1) | -52 - ΔminSENS |  |
| Medium Range BS | EISREFSENS + 6 dB (Note 1) | -47 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -47 - ΔminSENS |  |
| Local Area BS | EISREFSENS + 6 dB (Note 1) | -44 - ΔOTAREFSENS |  |
|  | EISminSENS + 6 dB (Note 1) | -44 - ΔminSENS |  |
| OTE 1: EISREFSENS / EISminSENS depends on the BS channel bandwidth, see subclause 10.3 and 10.2. NOTE 3: |  |  |  |

# Table 10.8.2-4: Interfering signals for narrowband intermodulation requirement in FR1

| channel bandwidthBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| 5 | [±360] | CW |
|  | [±1420] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 10 | [±325] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 15 (NOTE 2) | [±380] | CW |
|  | [±1600] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 20 (NOTE 2) | [±345] | CW |
|  | [±1780] | 5MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 25 (NOTE 2) | [±325] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 30 (NOTE 2) | [±320] | CW |
|  | [±1990] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 40 (NOTE 2) | [±310] | CW |
|  | [±2710] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 50 (NOTE 2) | [±330] | CW |
|  | [±3250] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 60 (NOTE 2) | [±350] | CW |
|  | [±3790] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 70 (NOTE 2) | [±400] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 80 (NOTE 2) | [±390] | CW |
|  | [±4870] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 90 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| 100 (NOTE 2) | [±340] | CW |
|  | [±5770] | 20MHz DFT-S-OFDM NR signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the BS channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for a G-FRC mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

### 10.8.3	Minimum requirement for BS type 2-O

Throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with OTA wanted signal at the assigned channel frequency and two OTA interfering signals provided at the RIB using the parameters in tables 10.8.3-1 and 10.8.3-2. All of the OTA test signals arrive from the same direction, and the requirement is valid if the signals arrive from any direction within the OTA REFSENS RoAoA. The reference measurement channel for the wanted signal is identified in [table 10.3.2-1] for each BS channel bandwidth and further specified in annex A.

The subcarrier spacing for the modulated interfering signal shall be the same as the subcarrier spacing for the wanted signal.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

[For a multi-band RIBs, the intermodulation requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least twice as wide as the NR interfering signal centre frequency offset from the Base Station RF Bandwidth edge.]

# Table 10.8.3-1: General intermodulation requirement

| Mean power of interfering signals [dBm] | Wanted signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- |
| EISREFSENS + 25dB | EISREFSENS + 6dB | See Table 10.8.3-2 |

# Table 10.8.3-2: Interfering signals for intermodulation requirement

| BBS channel bandwidth of the lowest/highest carrier received [MHz] | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| 50 MHz | [±7.5] | CW |
|  | [±40] | 50MHz DFT-S-OFDM NR signal |
| 100 MHz | [±6.88] | CW |
|  | [±40] | 50MHz DFT-S-OFDM NR signal |
| 200 MHz | [±5.64] | CW |
|  | [±40] | 50MHz DFT-S-OFDM NR signal |
| 400 MHz | [±6.02] | CW |
|  | [±45] | 50MHz DFT-S-OFDM NR signal |

## 10.9	OTA in-channel selectivity

### 10.9.1	General

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. In this condition a throughput requirement shall be met for a specified reference measurement channel. The interfering signal shall be an NR signal as specified in annex A and shall be time aligned with the wanted signal.

### 10.9.2	Minimum requirement for BS type 1-O

For BS type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A with parameters specified in table 10.9.2-1 for Wide Area BS, in table 10.9.2-2 for Medium Range BS and in table 10.9.2-3 for Local Area BS.

# Table 10.9.2-1: Wide Area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 10.9.2-2:  Medium Range BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 10.9.2-3: Local area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 10 PRB |
| 10,15,20,25,30 | 15 | G-FR1-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 25 PRB |
| 40,50 | 15 | G-FR1-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 15 kHz, 106 PRB |
| 5 | 30 | G-FR1-A1-8 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 5 PRB |
| 10,15,20,25,30 | 30 | G-FR1-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 11 PRB |
| 40,50,60,70,80,90,100 | 30 | G-FR1-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 30 kHz, 51 PRB |
| 10,15,20,25,30 | 60 | G-FR1-A1-9 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 5 PRB |
| 40,50,60,70,80,90,100 | 60 | G-FR1-A1-6 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 24 PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

### 10.9.3	Minimum requirement for BS type 2-O

For BS type 2-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A with parameters specified in table 10.9.3-1 for Wide Area BS, in table 10.9.3-2 for Medium Range BS and in table 10.9.3-3 for Local Area BS.

# Table 10.9.3-1: Wide Area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 33 PRB |
| 100,200 | 60 | G-FR2-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 66PRB |
| 50 | 120 | G-FR2-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 120 kHz, 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 120 kHz, 32PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 10.9.3-2: Medium Range BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 33 PRB |
| 100,200 | 60 | G-FR2-A1-1 | TBD | TBD | DFT-S-OFDM NR signal, SCS 60 kHz, 66PRB |
| 50 | 120 | G-FR2-A1-5 | TBD | TBD | DFT-S-OFDM NR signal, SCS 120 kHz, 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | TBD | TBD | DFT-S-OFDM NR signal, SCS 120 kHz, 32PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |

# Table 10.9.3-3: Local area BS in-channel selectivity

| NBS channel bandwidth [MHz] | Subcarrier spacing[KHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 50 | 60 | G-FR2-A1-4 | TBD | TBD | NR signal, SCS 60 kHz, 33 PRB |
| 100,200 | 60 | G-FR2-A1-1 | TBD | TBD | NR signal, SCS 60 kHz, 66PRB |
| 50 | 120 | G-FR2-A1-5 | TBD | TBD | NR signal, SCS 120 kHz, 16PRB |
| 100,200,400 | 120 | G-FR2-A1-2 | TBD | TBD | NR signal, SCS 120 kHz, 32PRB |
| NOTE: Wanted and interfering signal are placed adjacently around Fc |  |  |  |  |  |