3GPP TSG-RAN WG4 Meeting #88	R4-1809692

Gothenburg, Sweden, 20 – 24 August 2018

Agenda Item:	6.1.3.4.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for eAAS OTA receiver spurious emissions requirement

Document for:	Approval

# 1.	Introduction

The MU for eAAS OTA receiver spurious emissions requirement was discussed without conclusion [1].

This contribution provides our proposal on the MU for eAAS OTA receiver spurious emissions requirement, according to the comments received during discussion on [1].

# 2.	Discussion

For the MU for the eAAS OTA receiver spurious emissions requirement, we proposed in [1] to adopt the same MU as the OTA transmitter spurious emissions (mandatory) requirement, following the practice that has been adopted for conducted spurious emissions requirements in E-UTRA [2], MSR [3], and AAS [4] BS.

During the discussion on [1], it was commented that for receiver spurious emissions, power levels are much lower than transmitter spurious emission levels, and thus we may need larger MU for receiver. While we agree that the receiver spurious emission levels are much lower than the transmitter spurious emission (mandatory) levels, we consider the situation is no different than the conducted spurious emissions requirements for E-UTRA, MSR, and AAS BS, where the same MU as the transmitter spurious emissions (mandatory) requirement are adopted for the receiver spurious emissions requirement even though the receiver spurious emission levels are much lower than transmitter spurious emission (mandatory) levels. Consequently, we continue to propose to adopt the same MU as the eAAS OTA transmitter spurious emissions (mandatory) requirement for the eAAS OTA receiver spurious emissions requirement.

3.	Conclusion

This contribution has provided our proposal on the MU for the eAAS OTA receiver spurious emissions requirement, according to the comments received during discussion on [1].

Proposal:

Adopt the same MU as the eAAS OTA transmitter spurious emissions (mandatory) requirement for the eAAS OTA receiver spurious emissions requirement.

# References

[1]	R4-1808667, “Proposals on MU for OTA receiver spurious emissions requirement”, Nokia, Nokia Shanghai Bell.

[2]	3GPP TS 36.141 v15.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[3]	3GPP TS 37.141 v15.3.0, “E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing”.

[4]	3GPP TS 37.145-1 v15.0.0, “Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing”.