3GPP TSG-RAN WG4#88 Meeting	R4-1809804

Gothenburg, Sweden, 20 – 24 August 2018

Agenda item:	6.12.2.1

Title: 	PDSCH simulation results and discussion for 8Rx UEs

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

  - Define SDR test for 8Rx in WI.

  - Define CSI test for 8Rx.

  - No PDCCH/PCFICH demodulation requirement is expected for 8Rx.

- Considering the test coverage of 8Rx, test applicability rule is needed to define

  - Define applicability rule of existing performance requirements for 8Rx capable UEs.

In RAN4#86bis Meeting, a WF [2] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - For test cases of ranks lower than or equal to 4, introduce:

TM2: Rank 1, 16QAM 1/2, EVA5, 2x8 medium correlation B

TM3: Rank 2, 16QAM 1/2, EVA70, 2x8 low

  - FFS: Whether to introduce rank 5~8 demodulation performance tests

In RAN4#87 Meeting, a WF [3] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - Introduce 8Rx demodulation test(s)

TM9

Rank higher than four, including at least one rank from {5, 6, 7, 8}

16QAM

EPA5

In this contribution, we first provide simulation results and requirement proposals for TM2 and TM3 test cases, and then provide simulation results and our views on performance tests for ranks lager than 4.

# Discussion

## Simulation results and requirement proposals for PDSCH test cases with ranks lower than or equal to 4

Table 1 lists the simulation parameters for TM2 and TM3 test cases.

Table 1 Simulation parameters for TM2 and TM3 test cases

| Test number | Transmission mode | Bandwidth and MCS | Reference channel | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | TM2 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA5 | 2x8 Low Medium correlation B, ULA |
| 2 | TM3 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA70 | 2x8 Low |

Figure 1 and 2 illustrate the simulation results for the above two test cases without considering any impairment margins.

Figure 1 Simulation results for TM2

Observation 1: For TM2 test case, the maximum configured throughput is reached at 2dB, and 70% of maximum configured throughput is reached at -2.6dB.

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.6dB without considering impairment margins.

Figure 2 Simulation results for TM3

Observation 2: For TM3 test case, the maximum configured throughput is reached at 5dB, and 70% of maximum configured throughput is reached at 1.6dB.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.6dB without considering impairment margins.

## Simulation results and views on PDSCH demodulation performance requirements for ranks higher than 4

Table 2 lists the simulation parameters for TM9 test cases with 256QAM.

Table 2 Simulation parameters for TM9 test cases

| TM9 | 10 MHz 256QAM, MCS20 4/6/8 layers | R.xx FDD | EPA5 | 8x8 Low |
| --- | --- | --- | --- | --- |

Figure 3 illustrates the simulation results for the above TM9 test cases with 8×8 antenna configuration and 256QAM modulation (MCS20), and without considering any impairment margins.

Figure 3 PDSCH TM9 simulation results with 8×8 antenna configuration and 256QAM

Observation 2: Combination of 256QAM and 4-layer can fly, in terms of achieving the maximum configured throughput around 30dB; while maximum configured throughput for combination of 256QAM with 6-layer can fly given a higher than 35dB SNR; 8-layer cannot be achieved within reasonable SNR range.

Proposal 3: Define TM9 with 256QAM and 6-layer test case for the 8×8 8Rx UE’s PDSCH demodulation requirement.

# Conclusion

In this contribution, we first provide simulation results and requirement proposals for TM2 and TM3 test cases, and then provide our views on performance tests for ranks lager than 4.

Observation 1: For TM2 test case, the maximum configured throughput is reached at 2dB, and 70% of maximum configured throughput is reached at -2.6dB.

Observation 2: For TM3 test case, the maximum configured throughput is reached at 5dB, and 70% of maximum configured throughput is reached at 1.6dB.

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.6dB without considering impairment margins.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.6dB without considering impairment margins.

Proposal 3: Define TM9 with 256QAM and 6-layer test case for the 8×8 8Rx UE’s PDSCH demodulation requirement.

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1805552, “Way forward on 8Rx demodulation requirements”, Huawei, April 2018

[3] R4-1807964, “Way forward on 8Rx demodulation requirements”, Huawei, May 2018