3GPP TSG-WG4 RAN#88 	R4-1809847

# Gothenburg, Sweden, 20th – 24th August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0000 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2: MPR for single CC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1). MPR table for single CC is not finished. The values are still TBD 2). The reference waveform is not captured. It is already approved in WF R4-1714445 with BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0 as reference waveform with 0dB MPR and is used for power class definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MPR values are specified. And reference waveform is specified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPR values are not specified. Reference waveform is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 38.521-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

#### 6.2.2.3	UE maximum output power reduction for power class 3

Power class 3 UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-1.

Table 6.2.2.3-1 Maximum power reduction (MPR) for UE

|  |  | Channel Bandwidth / MPR |  |
| --- | --- | --- | --- |
|  |  | 50 / 100 / 200 MHz | 400 MHz |
| DFT-s-OFDM |  | Pi/2 BPSK |  | 2.0 |  | 3.5 |  |
|  |  | QPSK |  | 2.0 |  | 3.5 |  |
|  |  | 16QAM |  | 3.5 |  | 5.0 |  |
|  |  | 64QAM |  | 6.0 |  | 7.5 |  |
| CP-OFDM |  | QPSK |  | 4.0 |  | 5.5 |  |
|  |  | 16QAM |  | 5.5 |  | 7.0 |  |
|  |  | 64QAM |  | 8.5 |  | 10.0 |  |

The waveform defined by BW = 100MHz, SCS=60KHz, DFT-S-OFDM QPSK, 128RB0 is the reference waveform with 0dB MPR and is used for the power class definition.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

## <End of Changes>