3GPP TSG-RAN WG4 Meeting #88	    R4-1809934

Gothenburg, SE, 20th – 24th Aug, 2018

Agenda Item:	7.11.4.4.1

Source: 	Samsung

Title: 	Simulation results for SSB based L1-RSRP measurement

Document for:	Discussion

- Introduction

In RAN4 #87 meeting, the WF on evaluation period for L1-RSRP measurement for beam management and beam failure recovery procedures was agreed in [1]. And in RAN4 #AH-1807 meeting, several companies had submit their simulation results and shown their views on this topic [2~5].

Agreements on Candidate Beam detection - The requirements on both CSI-RS based candidate beam detection and SSB based candidate beam detection will be introduced, and the following aspects will be specified: - The L1-RSRP measurement period for each configured RS resource in set q1, which will be specified in core requirements. - FFS how many samples are assumed. - FFS whether to reflect Rx beam sweeping in FR2 Agreements on Beam Reporting - Both CSI-RS based and SSB based L1-RSRP measurement period for beam reporting will be specified. - FFS how to capture the requirements

Regarding to the FFS issue “how many samples are assumed”, in this contribution we provide our simulation results of SSB-based L1-RSRP measurement accuracy for beam management and beam failure recovery procedure, and give our views on the summarized results.

- Simulation Assumptions

The detailed simulation assumptions are listed in Table A.1, and A.2 in Annex A, with some downselections highlighted by red. For TDL-A/B/C channel, the worse cases are evaluated, i.e. delay spread = 300 ns and UE speed = 30 km/h.

Since the beam detection and reporting works mainly for serving cell, the channel quality is not very bad, we evaluate the RSRP accuracy as well as on better SINR side condition. So, three SINR conditions are considered in this paper, i.e. -5.97 dB, -3.00 dB and 0.00 dB.

In our simulation, only SSB based L1-RSRP measurement accuracy is evaluated, and the RSRP is calculated only on SS signal, i.e. 127 RE in one OFDM symbol in one SS burst period. The PBCH DMRS REs is not used for RSRP calculation, just like data REs.

The “Case A” option is adopted for UE receive antennas configuration, in which the receive antenna with maximum gain is used between the configured 2 Rx.

The metric is Delta RSRP = (estimated RSRP– ideal RSRP) [dB], where estimated RSRP is calculated from real received signal, and ideal RSRP is calculated from ideal channel estimation.

- Simulation Results

In this section, we will provide our simulation results of 5th, 50th, and 95th %-iles of the delta SS-RSRP CDF curves for different SCS configurations.

SCS 15 kHz

Table 3.1 shows the 5%, 50%, 95%-ile of the CDF curves of Delta SS-RSRP under different channel conditions for 15 kHz SCS.

Table 3.1: Delta SS-RSRP, SCS 15 kHz

| CDF Channel | Samples | SINR = -6dB |  |  |  | SINR = -3dB |  |  |  | SINR = 0dB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. |
| AWGN | 1 | -0.78 | 0.38 | 1.36 | 1.36 | -0.57 | 0.32 | 1.12 | 1.12 | -0.44 | 0.29 | 0.99 | 0.99 |
|  | 2 | -0.42 | 0.36 | 1.07 | 1.07 | -0.29 | 0.31 | 0.88 | 0.88 | -0.22 | 0.29 | 0.78 | 0.78 |
|  | 3 | -0.31 | 0.36 | 0.95 | 0.95 | -0.19 | 0.31 | 0.78 | 0.78 | -0.13 | 0.28 | 0.70 | 0.70 |
|  | 4 | -0.21 | 0.36 | 0.84 | 0.84 | -0.11 | 0.31 | 0.71 | 0.71 | -0.08 | 0.28 | 0.64 | 0.64 |
|  | 5 | -0.15 | 0.35 | 0.81 | 0.81 | -0.07 | 0.31 | 0.68 | 0.68 | -0.04 | 0.29 | 0.60 | 0.60 |
| EPA5 | 1 | -1.47 | 0.07 | 2.32 | 2.32 | -1.24 | 0.04 | 1.50 | 1.50 | -1.01 | 0.02 | 1.13 | 1.13 |
|  | 2 | -1.12 | 0.05 | 1.99 | 1.99 | -0.88 | 0.03 | 1.13 | 1.13 | -0.72 | 0.01 | 0.83 | 0.83 |
|  | 3 | -0.91 | 0.05 | 1.88 | 1.88 | -0.73 | 0.02 | 1.03 | 1.03 | -0.60 | 0.01 | 0.71 | 0.71 |
|  | 4 | -0.81 | 0.05 | 1.88 | 1.88 | -0.64 | 0.02 | 1.00 | 1.00 | -0.50 | 0.01 | 0.63 | 0.63 |
|  | 5 | -0.73 | 0.05 | 1.72 | 1.72 | -0.59 | 0.02 | 0.89 | 0.89 | -0.45 | 0.01 | 0.57 | 0.57 |
| ETU30 | 1 | -2.15 | 0.08 | 2.13 | 2.15 | -1.43 | 0.01 | 1.35 | 1.43 | -0.96 | -0.00 | 0.91 | 0.96 |
|  | 2 | -1.77 | 0.07 | 1.82 | 1.82 | -1.12 | 0.00 | 1.06 | 1.12 | -0.72 | -0.01 | 0.67 | 0.72 |
|  | 3 | -1.55 | 0.08 | 1.54 | 1.55 | -0.95 | -0.00 | 0.90 | 0.95 | -0.59 | -0.01 | 0.56 | 0.59 |
|  | 4 | -1.31 | 0.04 | 1.42 | 1.42 | -0.84 | -0.01 | 0.80 | 0.84 | -0.50 | -0.01 | 0.47 | 0.50 |
|  | 5 | -1.23 | 0.05 | 1.32 | 1.32 | -0.81 | -0.00 | 0.75 | 0.81 | -0.48 | -0.02 | 0.45 | 0.48 |
| ETU70 | 1 | -2.09 | 0.07 | 2.11 | 2.11 | -1.40 | 0.01 | 1.32 | 1.40 | -0.97 | -0.00 | 0.90 | 0.97 |
|  | 2 | -1.51 | 0.04 | 1.56 | 1.56 | -1.00 | -0.01 | 0.95 | 1.00 | -0.69 | -0.01 | 0.64 | 0.69 |
|  | 3 | -1.22 | 0.05 | 1.28 | 1.28 | -0.83 | -0.01 | 0.79 | 0.83 | -0.58 | -0.01 | 0.51 | 0.58 |
|  | 4 | -1.06 | 0.04 | 1.14 | 1.14 | -0.70 | -0.01 | 0.68 | 0.70 | -0.49 | -0.01 | 0.45 | 0.49 |
|  | 5 | -0.99 | 0.05 | 1.02 | 1.02 | -0.66 | -0.00 | 0.62 | 0.66 | -0.45 | -0.01 | 0.40 | 0.45 |
| TDL-A | 1 | -2.01 | 0.11 | 2.19 | 2.19 | -1.41 | 0.06 | 1.40 | 1.41 | -0.98 | 0.03 | 0.93 | 0.98 |
|  | 2 | -1.47 | 0.11 | 1.63 | 1.63 | -1.02 | 0.03 | 1.01 | 1.02 | -0.68 | 0.02 | 0.68 | 0.68 |
|  | 3 | -1.16 | 0.09 | 1.37 | 1.37 | -0.83 | 0.04 | 0.88 | 0.88 | -0.57 | 0.02 | 0.57 | 0.57 |
|  | 4 | -1.02 | 0.09 | 1.20 | 1.20 | -0.73 | 0.03 | 0.77 | 0.77 | -0.49 | 0.02 | 0.48 | 0.49 |
|  | 5 | -0.93 | 0.08 | 1.12 | 1.12 | -0.66 | 0.03 | 0.69 | 0.69 | -0.45 | 0.01 | 0.45 | 0.45 |
| TDL-B | 1 | -2.00 | 0.10 | 2.09 | 2.09 | -1.42 | 0.04 | 1.35 | 1.42 | -0.99 | 0.02 | 0.93 | 0.99 |
|  | 2 | -1.48 | 0.08 | 1.55 | 1.55 | -1.03 | 0.02 | 0.99 | 1.03 | -0.71 | 0.01 | 0.68 | 0.71 |
|  | 3 | -1.16 | 0.08 | 1.34 | 1.34 | -0.84 | 0.01 | 0.83 | 0.84 | -0.57 | 0.00 | 0.55 | 0.57 |
|  | 4 | -0.99 | 0.08 | 1.16 | 1.16 | -0.73 | 0.02 | 0.73 | 0.73 | -0.52 | 0.00 | 0.46 | 0.52 |
|  | 5 | -0.92 | 0.07 | 1.09 | 1.09 | -0.66 | 0.01 | 0.66 | 0.66 | -0.46 | 0.00 | 0.44 | 0.46 |
| TDL-C | 1 | -1.94 | 0.12 | 2.24 | 2.24 | -1.36 | 0.04 | 1.43 | 1.43 | -0.98 | 0.03 | 0.99 | 0.99 |
|  | 2 | -1.39 | 0.11 | 1.66 | 1.66 | -1.00 | 0.05 | 1.05 | 1.05 | -0.71 | 0.02 | 0.71 | 0.71 |
|  | 3 | -1.15 | 0.12 | 1.41 | 1.41 | -0.83 | 0.05 | 0.89 | 0.89 | -0.59 | 0.02 | 0.59 | 0.59 |
|  | 4 | -0.99 | 0.13 | 1.26 | 1.26 | -0.73 | 0.05 | 0.77 | 0.77 | -0.49 | 0.02 | 0.51 | 0.51 |
|  | 5 | -0.91 | 0.12 | 1.14 | 1.14 | -0.66 | 0.05 | 0.72 | 0.72 | -0.45 | 0.02 | 0.47 | 0.47 |

SCS 30 kHz

Table 3.2 shows the 5%, 50%, 95%-ile of the CDF curves of Delta SS-RSRP under different channel conditions for 30 kHz SCS.

Table 3.2: Delta SS-RSRP, SCS 30 kHz

| CDF Channel | Samples | SINR = -6dB |  |  |  | SINR = -3dB |  |  |  | SINR = 0dB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. |
| AWGN | 1 | -0.78 | 0.37 | 1.37 | 1.37 | -0.57 | 0.30 | 1.13 | 1.13 | -0.46 | 0.28 | 0.98 | 0.98 |
|  | 2 | -0.45 | 0.36 | 1.10 | 1.10 | -0.32 | 0.30 | 0.90 | 0.90 | -0.24 | 0.27 | 0.78 | 0.78 |
|  | 3 | -0.29 | 0.34 | 0.97 | 0.97 | -0.20 | 0.29 | 0.80 | 0.80 | -0.16 | 0.27 | 0.69 | 0.69 |
|  | 4 | -0.20 | 0.35 | 0.89 | 0.89 | -0.13 | 0.30 | 0.73 | 0.73 | -0.10 | 0.27 | 0.62 | 0.62 |
|  | 5 | -0.15 | 0.34 | 0.82 | 0.82 | -0.08 | 0.29 | 0.68 | 0.68 | -0.05 | 0.27 | 0.59 | 0.59 |
| EPA5 | 1 | -1.75 | 0.12 | 2.15 | 2.15 | -1.30 | 0.06 | 1.36 | 1.36 | -0.96 | 0.04 | 0.97 | 0.97 |
|  | 2 | -1.46 | 0.10 | 1.90 | 1.90 | -1.05 | 0.04 | 1.16 | 1.16 | -0.70 | 0.03 | 0.74 | 0.74 |
|  | 3 | -1.39 | 0.08 | 1.84 | 1.84 | -0.97 | 0.03 | 1.09 | 1.09 | -0.60 | 0.03 | 0.64 | 0.64 |
|  | 4 | -1.32 | 0.10 | 1.73 | 1.73 | -0.89 | 0.04 | 1.02 | 1.02 | -0.53 | 0.02 | 0.57 | 0.57 |
|  | 5 | -1.24 | 0.09 | 1.70 | 1.70 | -0.85 | 0.02 | 1.01 | 1.01 | -0.48 | 0.02 | 0.53 | 0.53 |
| TDL-A | 1 | -1.96 | 0.14 | 2.23 | 2.23 | -1.34 | 0.06 | 1.40 | 1.40 | -0.94 | 0.03 | 0.96 | 0.96 |
|  | 2 | -1.35 | 0.13 | 1.68 | 1.68 | -0.96 | 0.05 | 1.01 | 1.01 | -0.66 | 0.02 | 0.69 | 0.69 |
|  | 3 | -1.14 | 0.13 | 1.43 | 1.43 | -0.78 | 0.05 | 0.86 | 0.86 | -0.55 | 0.02 | 0.56 | 0.56 |
|  | 4 | -0.92 | 0.15 | 1.22 | 1.22 | -0.68 | 0.06 | 0.76 | 0.77 | -0.47 | 0.02 | 0.50 | 0.50 |
|  | 5 | -0.83 | 0.13 | 1.13 | 1.13 | -0.59 | 0.05 | 0.69 | 0.69 | -0.44 | 0.02 | 0.45 | 0.45 |
| TDL-B | 1 | -2.14 | 0.14 | 2.22 | 2.22 | -1.44 | 0.06 | 1.39 | 1.44 | -0.95 | 0.03 | 0.93 | 0.95 |
|  | 2 | -1.53 | 0.12 | 1.63 | 1.63 | -1.02 | 0.05 | 1.01 | 1.02 | -0.68 | 0.02 | 0.67 | 0.68 |
|  | 3 | -1.25 | 0.11 | 1.37 | 1.37 | -0.86 | 0.04 | 0.85 | 0.86 | -0.54 | 0.02 | 0.55 | 0.55 |
|  | 4 | -1.09 | 0.10 | 1.22 | 1.22 | -0.75 | 0.03 | 0.75 | 0.76 | -0.48 | 0.02 | 0.48 | 0.48 |
|  | 5 | -0.92 | 0.09 | 1.12 | 1.12 | -0.65 | 0.02 | 0.66 | 0.66 | -0.43 | 0.02 | 0.43 | 0.43 |
| TDL-C | 1 | -2.02 | 0.11 | 2.19 | 2.19 | -1.38 | 0.05 | 1.36 | 1.38 | -0.95 | 0.02 | 0.93 | 0.95 |
|  | 2 | -1.46 | 0.12 | 1.58 | 1.58 | -0.98 | 0.04 | 0.99 | 0.99 | -0.68 | 0.02 | 0.67 | 0.68 |
|  | 3 | -1.19 | 0.11 | 1.34 | 1.34 | -0.81 | 0.02 | 0.83 | 0.83 | -0.54 | 0.01 | 0.54 | 0.54 |
|  | 4 | -1.02 | 0.09 | 1.21 | 1.21 | -0.71 | 0.03 | 0.73 | 0.73 | -0.49 | 0.01 | 0.49 | 0.49 |
|  | 5 | -0.88 | 0.10 | 1.10 | 1.10 | -0.63 | 0.02 | 0.65 | 0.65 | -0.44 | 0.00 | 0.42 | 0.44 |

SCS 120 kHz

Table 3.3 shows the 5%, 50%, 95%-ile of the CDF curves of Delta SS-RSRP under different channel conditions for 120 kHz SCS.

Table 3.3: Delta SS-RSRP, SCS 120 kHz

| CDF Channel | Samples | SINR = -6dB |  |  |  | SINR = -3dB |  |  |  | SINR = 0dB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. |
| TDL-A | 1 | -2.33 | -0.03 | 1.85 | 2.33 | -1.58 | -0.10 | 1.14 | 1.58 | -1.14 | -0.10 | 0.78 | 1.14 |
|  | 2 | -1.66 | -0.08 | 1.28 | 1.66 | -1.16 | -0.13 | 0.81 | 1.16 | -0.83 | -0.11 | 0.52 | 0.83 |
|  | 3 | -1.39 | -0.10 | 1.02 | 1.39 | -0.95 | -0.13 | 0.61 | 0.95 | -0.70 | -0.12 | 0.40 | 0.70 |
|  | 4 | -1.24 | -0.10 | 0.89 | 1.24 | -0.85 | -0.13 | 0.52 | 0.85 | -0.60 | -0.12 | 0.34 | 0.60 |
|  | 5 | -1.12 | -0.10 | 0.81 | 1.12 | -0.80 | -0.12 | 0.46 | 0.80 | -0.56 | -0.12 | 0.29 | 0.56 |
| TDL-B | 1 | -2.32 | 0.02 | 1.97 | 2.32 | -1.53 | -0.07 | 1.22 | 1.53 | -1.05 | -0.10 | 0.77 | 1.05 |
|  | 2 | -1.61 | 0.00 | 1.42 | 1.61 | -1.11 | -0.09 | 0.83 | 1.11 | -0.76 | -0.11 | 0.51 | 0.76 |
|  | 3 | -1.35 | -0.01 | 1.17 | 1.35 | -0.92 | -0.09 | 0.68 | 0.92 | -0.65 | -0.11 | 0.40 | 0.65 |
|  | 4 | -1.16 | -0.02 | 1.01 | 1.16 | -0.83 | -0.09 | 0.57 | 0.83 | -0.57 | -0.10 | 0.33 | 0.57 |
|  | 5 | -1.05 | -0.03 | 0.91 | 1.05 | -0.73 | -0.10 | 0.52 | 0.73 | -0.51 | -0.12 | 0.29 | 0.51 |
| TDL-C | 1 | -2.43 | 0.05 | 2.07 | 2.43 | -1.63 | -0.04 | 1.29 | 1.68 | -1.08 | -0.06 | 0.83 | 1.08 |
|  | 2 | -1.72 | 0.00 | 1.48 | 1.72 | -1.16 | -0.06 | 0.89 | 1.16 | -0.79 | -0.07 | 0.56 | 0.79 |
|  | 3 | -1.38 | -0.02 | 1.22 | 1.38 | -0.93 | -0.07 | 0.71 | 0.93 | -0.66 | -0.08 | 0.44 | 0.66 |
|  | 4 | -1.21 | -0.02 | 1.03 | 1.21 | -0.81 | -0.08 | 0.59 | 0.81 | -0.57 | -0.09 | 0.37 | 0.57 |
|  | 5 | -1.09 | -0.01 | 0.91 | 1.09 | -0.77 | -0.07 | 0.52 | 0.77 | -0.53 | -0.08 | 0.33 | 0.53 |

SCS 240 kHz

Table 3.4 shows the 5%, 50%, 95%-ile of the CDF curves of Delta SS-RSRP under different channel conditions for 240 kHz SCS.

Table 3.4: Delta SS-RSRP, SCS 240 kHz

| CDF Channel | Samples | SINR = -6dB |  |  |  | SINR = -3dB |  |  |  | SINR = 0dB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. | 5% | 50% | 95% | abso. |
| TDL-A | 1 | -2.86 | -0.25 | 1.80 | 2.86 | -2.15 | -0.36 | 0.99 | 2.15 | -1.62 | -0.40 | 0.54 | 1.62 |
|  | 2 | -2.15 | -0.27 | 1.21 | 2.15 | -1.64 | -0.38 | 0.58 | 1.64 | -1.25 | -0.42 | 0.28 | 1.25 |
|  | 3 | -1.79 | -0.29 | 0.95 | 1.79 | -1.45 | -0.41 | 0.42 | 1.45 | -1.12 | -0.42 | 0.13 | 1.12 |
|  | 4 | -1.60 | -0.32 | 0.80 | 1.60 | -1.29 | -0.40 | 0.35 | 1.29 | -1.01 | -0.44 | 0.06 | 1.01 |
|  | 5 | -1.42 | -0.31 | 0.64 | 1.42 | -1.21 | -0.41 | 0.24 | 1.21 | -0.97 | -0.43 | 0.01 | 0.97 |
| TDL-B | 1 | -2.87 | -0.22 | 1.74 | 2.87 | -2.05 | -0.38 | 0.95 | 2.05 | -1.53 | -0.44 | 0.49 | 1.53 |
|  | 2 | -2.16 | -0.28 | 1.17 | 2.16 | -1.60 | -0.41 | 0.55 | 1.60 | -1.21 | -0.45 | 0.22 | 1.21 |
|  | 3 | -1.82 | -0.29 | 0.91 | 1.82 | -1.39 | -0.42 | 0.40 | 1.39 | -1.06 | -0.46 | 0.11 | 1.06 |
|  | 4 | -1.66 | -0.31 | 0.77 | 1.66 | -1.26 | -0.43 | 0.28 | 1.26 | -1.00 | -0.46 | 0.03 | 1.00 |
|  | 5 | -1.43 | -0.33 | 0.65 | 1.43 | -1.16 | -0.43 | 0.22 | 1.16 | -0.94 | -0.46 | -0.01 | 0.94 |
| TDL-C | 1 | -2.47 | -0.17 | 1.66 | 2.47 | -1.71 | -0.22 | 0.99 | 1.71 | -1.26 | -0.23 | 0.63 | 1.26 |
|  | 2 | -1.76 | -0.20 | 1.14 | 1.76 | -1.27 | -0.24 | 0.65 | 1.27 | -0.95 | -0.25 | 0.39 | 0.95 |
|  | 3 | -1.54 | -0.22 | 0.89 | 1.54 | -1.11 | -0.26 | 0.47 | 1.11 | -0.83 | -0.24 | 0.27 | 0.83 |
|  | 4 | -1.29 | -0.23 | 0.74 | 1.29 | -0.98 | -0.26 | 0.37 | 0.98 | -0.75 | -0.25 | 0.20 | 0.75 |
|  | 5 | -1.24 | -0.25 | 0.68 | 1.24 | -0.91 | -0.26 | 0.32 | 0.91 | -0.69 | -0.25 | 0.16 | 0.69 |

Summary and analysis

Based on above simulation results, the maximum absolute RSRP among different channel condtions and different SCS configurations is summarized in Table 3.5.

Table 3.5: Summarized absolute Delta SS-RSRP resutls

| Measument Samples | SINR = -6dB | SINR = -3dB | SINR = 0dB |
| --- | --- | --- | --- |
| 1 | 2.87 | 2.15 | 1.62 |
| 2 | 2.16 | 1.64 | 1.25 |
| 3 | 1.84 | 1.45 | 1.12 |
| 4 | 1.70 | 1.29 | 1.01 |
| 5 | 1.70 | 1.21 | 0.97 |

From the above summarized results, it can be seen that, the larger the number of measurement samples is, the better L1-RSRP measurement accuracy achieve. However, then the number of measurement samples increases from 3 to 5, the accuracy performance only has a slight gain.

Since large number of measurement samples will cause large beam detection delay or reporting delay, we suggest no more than 3 measurement samples should be configured for beam detection and reporting.

Observation 1: L1-RSRP measuremet accuracy improves slowly when the number of measurement samples increases from 3 to 5.

Here we take the RSRP measurement requirement in legacy LTE as a consideration, i.e. ±2 dB without RF margin. From the above summarized results, we can see that, to achieve 2 dB RSRP measurement accuracy, as least 3 samples are needed for -6 dB SINR condition, 2 samples are needed for -3 dB SINR condition, and 1 sample is enough for 0 dB SINR condition.

Observation 2: To achieve 2 dB RSRP measurement accuracy, at least 3/2/1 sample(s) are necessary for -6/-3/0 dB SINR conditions respectively.

Conclusion

In this paper, we provided the SSB based L1-RSRP measurement accuracy simulation results. The results covered different SCSs, channel conditions, and numbers of measurement samples. From the simulation results, we obtain the following observations:

Observation 1: L1-RSRP measuremet accuracy improves slowly when the number of measurement samples increases from 3 to 5.

Observation 2: To achieve 2 dB RSRP measurement accuracy, at least 3/2/1 sample(s) are necessary for -6/-3/0 dB SINR conditions respectively.

Reference

- R4-1808444, Huawei, HiSilicon, “Way forward on Beam management requirements in NR,” May 2018.

- R4-1808722, Intel Corporation, “Discussion about L1-RSRP measurement accuracy for beam management”, July 2018.

- R4-1808764, NTT DOCOMO, “Evaluation period for L1-RSRP measurement”, July 2018.

- R4-1808799, Ericsson, “New beam detection”, July 2018.

- R4-1809082, Nokia, Nokia Shanghai Bell, “CSI-RS and SSB L1 RSRP for beam management”, July 2018.

- Annex A: Simulation Assumptions

Table A.1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) | Case A: 2 rx (uncorrelated with equal gain, no rx beamforming); Case B: 2 rx (with rx beamforming Note1) |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5 | 5 |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - Actual SS block transmissions | always transmitted | always transmitted |
| NOTE1: For Case B, companies are encouraged to state their assumptions on beamforming and rx antenna configuration together with the provided simulation results |  |  |

Table A.2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | (,)=(112, 1) | (,)=(224, 1) |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR (beamforming gain is included for Case B) | dB | 6 dB 1.8 dB -5.87 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | -5.97 dB -3.00 dB 0.00 dB |
| Propagation conditions | - | * For 4 GHz: AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS), TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) * For 30 GHz: CDL-C, optional TDL-A/B/C Note (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |