3GPP TSG-RAN WG4 Meeting #88                	R4-1809948

Gothenburg, Sweden, 20 Aug – 24 Aug 2018

Agenda item:	6.5.7.1

Source:	Samsung

Title:	Discussion on BS demodulation requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

During RAN4#87 Busan meeting, WF about the new BS performance requirements for eFeMTC was agreed as

  - RAN4 will introduce new PUSCH demodulation requirements with subPRB transmission

    - RAN4 will specify the demodulation requirements with 2 of 3 sub-carriers pi/2 BPSK

      - FFS: CE Mode A and/or CE Mode B

    - FFS: 3/6 subcarriers with QPSK demodulation requirements

    - RAN4 will wait for RAN1 conclustion on the detailed specfication

  - RAN4 will introduce new PUSCH demodulation requirements for CE Mode A with the following condition. Interested companies are encouraged provide the simulation results for alignment.

    - 8 PUSCH repetitions

    - FRC A3-2 (QPAK1/3, TBS =600bit, 6PRB)

    - Bandwidth 3, 5, 10, 15, and 20MHz

    - ETU200

    - Antenna configuration: 1x2

In this contribution, we share our view about the remaining issues and give our proposals about the general test parameters, based on the latest RAN1 conclusion.

# 2	Discussion

# 2.1	DMRS

As per RAN1 agreement, each DMRS symbol is length-2 BPSK with DFT-s-OFDM. The two DMRS pi/2 BSPK, noted as, R1(n) and R2(n), can be calculated as:

R1(n) = B(n)

B2(n)= B(n)(-1)(n+Physical_Cell_ID)

where B(n) is the same as the NB-IOT single tone DMRS sequence. As for how to determine which 2 of 3 subcarriers will be used,  RAN1 agreement is to decide by Physical_cell_ID mod 2 as follows:

- 0 indicates that the two subcarriers having the lowest indices among the three allocated are utilized

- 1 indicates that the two subcarriers having the highest indices among the three allocated are utilized

As for the how to mapping the DMRS to the selected two subcarriers, for example, in case of subcarrier 0, 1 are selected, whether B1(n) is the DMRS mapping for subcarrier 0 or subcarrier 1, there is no mentioned yet. In our view, in our view, there is not impact on the receiver algorithm and the channel estimation performance for different mapping rule. Thus, we recommend fixing one mapping rule. For simplicity, B1(n) is the DMRS mapping for subcarrier 0, B2(n) is the DMRS mapping for subcarrier 1.

Observation 1: For DMRS sequence of length -2 with DFT-s-OFDM, there should be no impact on the performance with different mapping rule.

# 2.2	RU size and Repetition number

As per RAN1 agreements, the RU size of 2 of 3 sub-carrier pi/2 BPSK is 8 for CE Mode A and CE Mode B for FDD and TDD.  For the allocated RU number, 1, 2 and 4 RU can be allocated with CE Mode A, and only 2, 4 RUs can be allocated for CE Mode B. Considering that the total number of valid subframes for transmission is agreed as 32 subframes and 2048 subframes for CE Mode A and CE Mode B respectively, we can obtain the maximum repetition number with scaled by RU number and RU size as for table1for CE A and CE B respectively.

Table.1 Maximum repetition number for sub-PRB allocation for CE Mode A and CE Mode B with 2 of 3 sub-carrier pi/2 BPSK

|  | RU size | Maximum repetition number for 1 RU | Maximum repetition number for 2 RUs | Maximum repetition number for 4 RUs |
| --- | --- | --- | --- | --- |
| 2 of 3 subcarriers (CE Mode A) | 8ms | 4 | 2 | 1 |
| 2 of 3 subcarriers (CE Mode B) | 8ms | NA | 128 | 64 |

In our view, 32 for CE Mode A and 2048 for CE Mode B is same as max repetition number of the existing eMTC without sub-PRB allocation. In order to reduce the test time and test tolerance, maximum repetition cases are not preferred. In existed LTE 36.104 specification, repetition number 8 and repetition number 256 are selected as the performance requirement for CE Mode A and CE Mode B. With following the legacy way of repetition number, we propose 1 or 2 repetition for 1RU case, 1 repetition for 2RU for CE Mode A, with limited the test cases. As for 4RU allocation, considering it will be 32ms, we prefer it is not introduced to performance test. For CE Mode B, we propose 16 repetitions is for 2 RU allocation, 8 repetitions is for 4RU allocation

Proposal 1: In demodulation tests for PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK for FDD, the number of repetition with different RU allocation number is

    - CE Mode A , 1 or [2] repetition number for 1RU allocation, 1 repetition for 2 RU allocation

    - CE Mode B, 16 repetition for 2 RU allocation,  8 repetition for 4RU allocation

As per RAN1 agreement, the frequency hopping of PRB between 2 narrowband of sub-PBB allocation is the same as legacy. Sub-carrier locations in the PRB are the same with both narrowband

Proposal 2: Reuse the legacy frequency hopping pattern in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

As for the number of antennas, we propose to use the legacy number existing eMTC without sub-PRB., i.e, 1Tx antenna, and  2 Rx antennas will be tested..

Proposal 3: Reuse the legacy number of antennas in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

As for system bandwidths, for legacy eMTC, 3MHz, 5MHz, 10MHz, 15MHz, 20MHz, are tested. In order to define the requirement with frequency hopping, we propose to use the same bandwidth for PUSCH with sub-PRB allocation

Proposal 4: Reuse the legacy system bandwidth 3MHz,5MHz,10MHz,15MHz,20MHz in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

As for the Performance metric for CE Mode A with sub-PRB allocation, values for fraction of maximum throughput used in legacy requirements is 70 %. We propose to use the same values for SPUSCH.

As the propagation model, we propose to use the legacy way with EPA5 for CE Mode A, ETU for CE Mode B.

As the reference receiver we propose to use MRC. We do not see a need to test more complicated reference receivers. Normal cyclic prefix shall be used in the tests.

As for RV, 4 RVs will be used for sub-PRB transmission. Considering the maximum repetition is 4, and cyclic repetition is not supported for sub-PRB allocation for CE Mode A. Then the RV sequence in RV sequence in the PUSCH with sub-PRB allocation is {0, 2, 3, 1}.

Proposal 5: Reuse the legacy values for maximum number of HARQ transmissions (4), and RV sequence (0, 2, 3, 1) in PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK scheme in CE Mode A for FDD

As per RAN1 agreement, for sub-PRB allocation in CE Mode A, there is no RRC configuration need to indicate the

- PRB location in the system bandwidth

- Sub-PRB location within PRB

- # of sub-carriers

- # of RUs

- MCS Index

For 2 of 3 sub-carrier pi/2 BPSK allocation, 4 non-overlapping allocations per PRB, i.e., {0, 1, 2}, {3, 4, 5}, {6, 7, 8} and {9, 10, 11}, will be defined. In our view, there is no impact on the performance requirement. We can select one of allocation scheme for performance test such as {0, 1, 2}.

Summarized, we propose the SPUCCH simulation assumptions as shown in Table 3.

Proposal 6: In demodulation tests for PUSCH with sub-PRB allocation, we propose to use the parameters as shown

Table3: eFeMTC with sub-PRB transmission simulation assumptions for FDD

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Propagation condition | EPA5 for CE Mode A, ETU for CE Mode B |
| Cyclic prefix | Normal |
| Fraction of maximum throughput | 70 % |
| RV sequences | 0,2,3,1,0,2,3,1 |
| Maximum number of HARQ transmissions | 4 |
| Reference receiver | MRC |
| Channel bandwidth | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |
| RU size | CE Mode A: 1RU, 2RU, [4RU] CE Mode B: 2RU, 4RU |
| Repetition Number | CE Mode A: 1,[2] for 1RU, 1 for 2RU,[1 for 4RU] CE Mode B: 16 for 2RU, [8 for 4RU] |
| DMRS | Fixed one DMRS mapping rule |
| Frequency hopping | On |
| * Reduce the number of test cases by testing all BWs, , but not testing all combinations of these parameters. |  |

# 2.3   FRC

As per RAN1 agreements, the TBS table for CE Mode A and CE Mode B can be referred as table 3

Table3: eFeMTC with sub-PRB transmission simulation assumptions for FDD

|  | NPRB |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |  |  |  |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 |  |  |  |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 |  |  |  |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 |  |  |  |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 |  |  |  |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 |  |  |  |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 |  |  |  |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 |  |  | 1 RU |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 |  |  | 2 RUs |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  | 4 RUs |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 |  |  |  |

RAN4 has already defined the demodulation requirement for 1 subcarrier with SC-FDMA BPSK for NB-IOT NPUSCH format1 and the related FRC. For pi/2 BPSK, RAN4 also defined the FRC table for NB-IOT reference senility and repetitions sensitivity. In our view, we can follow the similar way to define the FRC table for sub-PRB 2 of 3 subcarrrier with pi/2 BPSK method. As for details of value, such as coding rate, MCS index, number of repetition for each number of RU, we need to further discussion.

# 2.4   3/6 subcarriers with QPSK

Sub-PRB resource allocation is discussed in RAN1 and current agreement is to support the following transmission [2]:

- 6 subcarriers with SC-FDMA QPSK modulation,

- 3 subcarriers with SC-FDMA QPSK modulation, and

- 2 out of 3 subcarriers with SC-FDMA Pi/2 BPSK modulation.

In our view, RAN4 has already defined performance requirement for 3/6 subcarriers with SC-FDMA QPSK modulation for NB-IOT NPUSCH format 1. Besides, in terms of symbol location, 2 of 3 subcarriers is same with 3 subcarriers with QPSK. In terms of DMRS sequence, 3/6 subcarriers with SC-FDMA has the same structure with non-sub PRB method. Thus, we recommend that there is no performance requirement for 3/6 subcarriers with SC-OFDMA QPSK method.

Proposal 7: there is no performance requirement for 3/6 subcarriers with SC-FDMA QPSK modulation.

# 3	Conclusion

In this contribution, we provide our view about the remaining issues and give our proposals about the general test parameters.

Observation 1: For DMRS sequence of length -2 with DFT-s-OFDM, there should be no impact on the performance with different mapping rule.

Proposal 1: In demodulation tests for PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK for FDD, the number of repetition with different RU allocation number is

    - CE Mode A , 1 or 2 repetition number for 1RU allocation, 1 repetition for 2 RU allocation

    - CE Mode B, 16 repetition for 2 RU allocation,  8 repetition for 4RU allocation

Proposal 2: Reuse the legacy frequency hopping pattern in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

Proposal 3: Reuse the legacy number of antennas in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

Proposal 4: Reuse the legacy system bandwidth 3MHz,5MHz,10MHz,15MHz,20MHz in PUSCH with PRB allocation for the demodulation test of PUSCH with sub-PRB allocation for 2/3 Pi/2 BPSK

Proposal 5: Reuse the legacy values for maximum number of HARQ transmissions (4), and RV sequence (0, 2, 3, 1) in PUSCH with sub-PRB allocation 2/3 Pi/2 BPSK scheme in CE Mode A for FDD

Proposal 6: In demodulation tests for PUSCH with sub-PRB allocation, we propose to use the parameters as shown in Table 2.

Table2: eFeMTC with sub-PRB transmission simulation assumptions for FDD

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Fraction of maximum throughput | 70 % |
| RV sequences | 0,2,3,1,0,2,3,1 |
| Maximum number of HARQ transmissions | 4 |
| Reference receiver | MRC |
| Channel bandwidth | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |
| RU size | CE Mode A: 1RU, 2RU, [4RU] CE Mode B: 2RU, 4RU |
| Repetition Number | CE Mode A: 1,2 for 1RU, 1 for 2RU,[1 for 4RU] CE Mode B: 16 for 2RU, 8 for 4RU |
| DMRS | Fixed one |
| Frequency hopping | On |
| * Reduce the number of test cases by testing all BWs, , but not testing all combinations of these parameters. |  |

Proposal 7: there is no performance requirement for 3/6 subcarriers with SC-FDMA QPSK modulation.

# References

R1-1807971, Agreement summary for Rel-15 Even further enhanced MTC for LTE,

36.211

R4-1807976, WF on BS demodulation requirements for eFeMTC, Ericsson