3GPP TSG-RAN WG4 #88 Meeting                                     R4-1809987

Gothenburg, Sweden, August 20-24, 2018

Agenda item:	7.13.2.2

Source: 	ZTE

Title: 	On NR BS PUSCH demodulation performance requirements

Document for:	Discussion

Introduction

During RAN4#AH-1807 meeting in July two WFs [1][2] relevant for NR BS PUSCH demodulation performance requirements and simulation assumptions for alignment purpose have been approved. This contribution presents simulation results.

Simulation assumptions and simulation results

- Simulation assumptions

The simulation assumptions considering [2] are summarized in the table below.

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Transmission scheme | n.a. | n.a. |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 front-loaded |
| Boosting | On | n.a |
| symbols length | 14 | 2 |
| Time domain resource allocation type | type A | type B |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns | AWGN |
| SCS and BW | 30kHz，40MHz | 120kHz: 100 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured |
| Phase noise | Not modelled | Not modelled |
| Waveform | CP-OFDM | CP-OFDM |
| Decoding iteration number | 8 | 8 |
| Number of HARQ transmission | 4 | 4 |
| Limited buffer rate matching | Disabled | Disabled |

- Initial simulation results for FR1 conducted

The initial simulation results for FR1 conducted are summarized in the table below. The simulation results are the same for 1 DMRS and 2 DMRSs.

Initial simulation results for FR1 conducted:

1Tx, 2Rx, SCS 30 kHz @ CBW 40 MHz, 1 DMRS and 2 DMRS, Decoding iteration number = 8

| Number of TX | Number of RX (Number of diversity branches in baseband) | Cyclic prefix | Propagation conditions | Modulation, MCS | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDL-A 30ns | QPSK, MCS 2 | 70% | -4.7 |
|  |  |  |  | 16QAM, MCS 16 | 70% | 7.64 |
|  |  |  |  | 64QAM, MCS 20 | 70% | 9.7 |

- Initial simulation results for FR2

The initial simulation results for FR2 are summarized in the table below.

Initial simulation results for FR2:

1Tx, 2Rx, SCS 120 kHz @ CBW 100 MHz, 1 DMRS, Decoding iteration number = 8

| Number of TX | Number of RX (Number of diversity branches in baseband) | Cyclic prefix | Propagation conditions | Modulation, MCS | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | QPSK, MCS 2 | 70% | 0.6 |
|  |  |  |  | 16QAM, MCS 16 | 70% | 12.4 |
|  |  |  |  | 64QAM, MCS 20 | 70% | 15.5 |

Conclusions

The simulation results are summarized in the tables.

# References

- R4-1809370 Way forward on General part of NR BS demodulation performance, July 2018.

- R4-1809336 WF on NR PUSCH demodulation requirements, July 2018.