# 3GPP TSG-RAN WG4 Meeting #88                                                                  R4-1810171

# Gothenburg, Sweden, 20th - 24th Aug, 2018

Source: 	ZTE Corporation

Title: 	TP to TS 38.141-1: Applicability of test configurations(Section 4.8.3)

Agenda Item:	7.9.3.2

Document for:	Approval

1	Introduction

In the last meeting, the contribution[1] on applicability of test configuration was agreed and captured into TS 38.141-2 [2]. However, for the applicability of test configuration for multi-band operation using single-band connector is FFS.

In this contribution, we provides a TP to Section 4.8.3 of TS 38.141-1. The main modifications are:

- Remove the [] in table 4.8.3-1.

- Adding a new table 4.8.3-2 to capture the test configuration for multi-band operation using single-band connector.

2	Reference

[1] R4-1809470, TP to TS 38.141-1：Applicability of test configurations, Huawei

[2] 38.141-1 v0.4.0

--------------Start of TP  -------------

4.8.3	Applicability of test configurations for single-band connector

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to subclause 4.6. The generation and power allocation for each test configuration is defined in subclause 4.7. This subclause contains the test configurations for single-band connector.

For a NR BS declared to be capable of single carrier operation only, a single carrier (SC) shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA operation in contiguous spectrum operation, the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum and where the parameters in the manufacture's declaration according to subclause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 4.8.3-1 shall be used for testing.

For a single-band connector declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum and where the parameters in the manufacture's declaration according to subclause 4.6 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 4.8.3-1 shall be used for testing.

Table 4.8.3-1: Test configurations for a single-band connector

| BS test case | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |
| --- | --- | --- | --- |
| Base station output power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Total power dynamic range | SC | SC | SC |
| Transmit ON/OFF power (only applied for NR TDD BS) | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Time alignment error | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| Occupied bandwidth | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Cumulative ACLR requirement in non-contiguous spectrum | - | NRTC3 | NRTC3 |
| Operating band unwanted emissions | NRTC1, [SC (Note 2)] | NRTC1, NRTC3, [SC (Note 2)] | NRTC1, NRTC3, [SC (Note 2)] |
| Transmitter spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Transmitter intermodulation | NRTC1 | NRTC1, NRTC3 | NRTC1, NRTC3 |
| Reference sensitivity level | SC | SC | SC |
| Dynamic range | SC | SC | SC |
| Adjacent Channel Selectivity (ACS) | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| In-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Out-of-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Receiver spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| Receiver intermodulation | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| In-channel selectivity | SC | SC | SC |
| Note 1: NRTC2 is only applicable when contiguous CA is supported. [Note 2: OBUE SC shall be tested using the widest supported Channel Bandwidth and the highest supported sub-carrier spacing.] |  |  |  |

For multi-band operation using single-band connector, the test configuration in Table 4.8.3-2 shall be used for testing.

Table 4.8.3-2: Test configurations for a single-band connector in multi-band operation

| BS test case | Test configuration |
| --- | --- |
| Base station output power | NRTC1/3 (Note 1), NRTC4 |
| RE Power control dynamic range | Tested with Error Vector Magnitude |
| Total power dynamic range | SC |
| Transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 |
| Frequency error | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NRTC1/3 (Note 1), NRTC4 |
| Time alignment error | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| Occupied bandwidth | SC, NRTC2 (Note 3) |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1/3 (Note 1, 5), NRTC5 (Note 4, 5) |
| Cumulative ACLR requirement in non-contiguous spectrum | NRTC3 (Note 1, 5) |
| Operating band unwanted emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5), [SC(Note 7)] |
| Transmitter spurious emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5) |
| Transmitter intermodulation | NRTC1/3 (Note 1, 5) |
| Reference sensitivity level | SC |
| Dynamic range | SC |
| Adjacent Channel Selectivity(ACS) | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| In-band blocking | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| Out-of-band blocking | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| Receiver spurious emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5) |
| Receiver intermodulation | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| In-channel selectivity | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. Note 5: Single-band requirement apply to each antenna connector for both multi-band operation test and single-band operation test. For single-band operation test, other antenna connector(s) is (are) terminated. Note 6: NRTC5 is only applicable for multi-band receiver. [Note 7: OBUE SC shall be tested using the widest supported Channel Bandwidth and the highest supported sub-carrier spacing.] |  |

4.8.4	Applicability of test configurations for multi-band connector

For a multi-band connector, the test configuration in Table 4.8.4-1 shall be used for testing.

Table 4.8.4-1: Test configuration for a multi-band connector

| BS test case | Test configuration |
| --- | --- |
| Base station output power | NRTC1/3 (Note 1), NRTC4 |
| RE Power control dynamic range | Tested with Error Vector Magnitude |
| Total power dynamic range | SC |
| Transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 |
| Frequency error | Tested with Error Vector Magnitude |
| Error Vector Magnitude | NRTC1/3 (Note 1), NRTC4 |
| Time alignment error | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| Occupied bandwidth | SC, NRTC2 (Note 3) |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1/3 (Note 1), NRTC5 (Note 4) |
| Cumulative ACLR requirement in non-contiguous spectrum | NRTC3 (Note 1), NRTC5 (Note 4) |
| Operating band unwanted emissions | NRTC1/3 (Note 1), NRTC5, [SC (Note 5)] |
| Transmitter spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| Transmitter intermodulation | NRTC1/3 (Note 1) |
| Reference sensitivity level | SC |
| Dynamic range | SC |
| Adjacent Channel Selectivity(ACS) | NRTC5 |
| In-band blocking | NRTC5 |
| Out-of-band blocking | NRTC5 |
| Receiver spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| Receiver intermodulation | NRTC5 |
| In-channel selectivity | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. [Note 5: OBUE SC shall be tested using the widest supported Channel Bandwidth and the highest supported sub-carrier spacing.] |  |

--------------End of TP ------------